3GPP TSG-RAN5 Meeting #90-e	R5-210070

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1676 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to generic procedure for UE-requested PDU session modification after S1 to N1 change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From 25.401: To allow the network to release the N1 NAS signalling connection, the UE: a) shall start the timer T3540 if the UE receives any of the 5GMM cause values #7, #11, #12, #13, #15, #27, #31, #62, #72, #73, #74, #75, #76; b) shall start the timer T3540 for a UE in 3GPP access if: 1) the UE receives a REGISTRATION ACCEPT message which does not include a Pending NSSAI IE or UE radio capability ID deletion indication IE; 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; Upon expiry of T3540, - in cases a), b), f) and g), the UE shall locally release the established N1 NAS signalling connection; - in cases c) and d) the UE shall locally release the established N1 NAS signalling connection and the UE shall initiate the registration procedure as described in subclause 5.5.1.3.5 or 5.6.1.5; or - in case e), the UE shall locally release the established N1 NAS signalling connection and perform a new registration procedure as specified in subclause 5.5.1.3.2. This means if the UE has sent REGISTRATION REQUEST with “no follow-on request pending” and no further downlink signaling is received for 10s (=T3540) the UE will locally release the NAS signaling connection. In this procedure, the SS will wait always 10s for a PDU SESSION MODFICATION REQUEST (regardless of follow-on request setting). A UE which is not reauesting PDU session modification will locally release the NAS connection at a time which can clash with the expiry of this timer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reduce wait time to 8s, so the SS can send further signalling before the UE has locally released its NAS signalling connection due to T3540 expiry. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail test cases using this procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5A.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s210077 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.5A.2C	Procedure for UE-requested PDU session modification after the first S1 to N1 mode change / Single-registration mode with N26

4.5A.2C.1	Scope

The purpose of this procedure is to handle UE-requested PDU session modification after the first S1 to N1 mode change in the scenario of Single-registration mode, 'Interworking without N26 interface not supported'.

Table 4.5A.2C.1-1: Conditions and other parameters

| Condition | Explanation |
| --- | --- |
| ExpectedNumberOfPDUSessionModifications | Usage: Parameter determining procedure sequence. A value for this parameter shall be provided when the procedure is called. Unless the test requires specific number of PDU modifications to take place the value should be set to pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection ([19]). Meaning: The number of PDU sessions which are expected to happen. Depends on the UE configuration and/or the context in which the procedure is used. |
| K | Usage: Parameter determining procedure sequence. Meaning: The number of PDN SESSION MODIFICATION REQUEST messages already processed including the one that is currently being processed. |
| L | Usage: Parameter determining procedure sequence. Meaning: The number of PDN SESSION MODIFICATION REQUEST being received so far. |
| N | Usage: Parameter determining procedure sequence. The parameter is initialised with the value of ExpectedNumberOfPDUSessionModifications Meaning: Loop (step sequence repetition) control. |

4.5A.2C.2	Procedure description

4.5A.2C.2.1	Initial conditions

The UE is in RRC_CONNECTED state.

4.5A.2C.2.2	Procedure sequence

Table 4.5A.2C.2.2-1: Procedure for UE-requested PDU session modification after the first S1 to N1 mode change

| St | Procedure | Message Sequence |  | Verdict |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 0 | Set K = 0, L = 0, N = (ExpectedNumberOfPDUSessionModifications). | - | - | - |
| 1 | The procedure specified in Table 4.5A.2C.2.2-2 takes place. | - | - | - |
| 2 | Set K = K +1. | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 3-5a1 below the events specified in Table 4.5A.2C.2.2-2 may take place. | - | - | - |
| 3 | The SS transmits a PDU SESSION MODIFICATION COMMAND. | <-- | NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - |
| 4 | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - |
| - | EXCEPTION: Steps 5a1 to 5b1 describe behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - |
| 5a1 | IF L > K (NOTE 1) THEN repeat from step 2. | - | - | - |
| 5b1 | ELSE IF K < N (NOTE 2) THEN repeat from step 1. | - | - | - |
| NOTE 1: One (or more) further PDU SESSION MODIFICATION REQUEST message has been received in parallel. NOTE 2: Less PDU SESSION MODIFICATION REQUEST messages than expected have been received and processed so far -> Further request are expected from the UE. |  |  |  |  |

Table 4.5A.2C.2.2-2: Reception of PDU SESSION MODIFICATION REQUEST message

| St | Procedure | Message Sequence |  | Verdict |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 1 | Start Wait_Timer = 8 sec. NOTE: 8 sec is an arbitrary value less than T3540. | - | - | - |
| - | EXCEPTION: Steps 2a1 to 2b1 describe behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - |
| 2a1 | The UE transmits an ULInformationTransfer message and a PDU SESSION MODIFICATION REQUEST with PDU session ID which has been associated with a default EPS bearer set up during the UE operation in S1. | --> | NR RRC: ULInformationTransfer 5GSM: PDU SESSION MODIFICATION REQUEST | - |
| 2a2 | Stop Wait_Timer. | - | - | - |
| 2a3 | Set L = L +1. | - | - | - |
| 2a4 | Check: Does the L>N? (NOTE 1) | - | - | F |
| 2b1 | Check: Does Wait_Timer expire? (NOTE 2) | - | - | F |
| NOTE 1: The SS shall raise a fail verdict when there are more PDU SESSION MODIFICATION REQUEST messages received than expected and terminate the test. The reason for such a behaviour can be e.g. wrongly set PICS with which the ExpectedNumberOfNewPDNConnections was initiated. NOTE 2: Unless this is specified explicitly otherwise e.g. in a test procedure which calls the procedure specified in the present table, the SS shall raise a fail verdict when there are less PDU SESSION MODIFICATION REQUEST messages than expected received at this point and terminate the test. The reason for such a behaviour can be e.g. wrongly set PICS with which the ExpectedNumberOfNewPDUSessions was initiated, |  |  |  |  |

4.5A.2C.2.3	Specific message contents

All specific message contents shall be according clause 4.6 and 4.7 with the below exceptions:

Table 4.5A.2C.2.3-1: UL NAS TRANSPORT (Step 1, Table 4.5A.2C.2.2-2)

| Derivation Path: Table 4.7.1-10. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | '101'B | "modification request" |  |

Table 4.5A.2C.2.3-2: PDU SESSION MODIFICATION REQUEST (Step 1, Table 4.5A.2C.2.2-2)

| Derivation Path: Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | The PDU session ID associated with the default EPS bearer of the PDN connection which is being transferred into PDU session and for which the UE needs to indicate its capabilities via the modification procedure. (NOTE 1) |  |  |
| 5GSM capability |  | Present. Contents not checked. |  |  |
| Integrity protection maximum data rate |  | Present. Contents not checked. |  |  |
| NOTE 1: The PDU session ID was provided in the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT when the UE attached in S1. If the UE provides here an unknown PDU session ID then the SS shall set an Inconclusive verdict. |  |  |  |  |