3GPP TSG-RAN5 Meeting #90-e	R5-210793

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1142 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the test configuration for carrier leakage for SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TP analysis in 38.905, in SUL test cases the test configuration for SUL carrier adopts the same selection of that in corresponding single carrier test cases. Compared with Table 6.4.2.2.4-1 in clause 6.4.2.2.4, the test configuration in Table 6.4C.2.2.4-1 was not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the test configuration for SUL carrier in Table 6.4C.2.2.4-1 to keep alignment with Table 6.4.2.2.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The carrier leakage for SUL would not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4C.2.2	Carrier leakage for SUL

6.4C.2.2.1	Test purpose

Same test purpose as in clause 6.4.2.2.1.

6.4C.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.2.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.2.2.

6.4C.2.2.4	Test description

Same test description as specified in clause 6.4.2.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.2.4-1  use Table 6.4C.2.2.4-1

Table 6.4C.2.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid for both SUL carrier and Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for both SUL carrier and Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: When the signalled DC carrier position is at Inner_1RB_Left, use Inner_1RB_Right for UL RB allocation. |  |  |  |  |  |

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.3-14 with condition SUL_SUL, Table 4.6.3-15 with condition SUL_NUL, and Table 4.6.3-167 with condition PUSCH_ON_SUL.

Table 6.4C.2.2.4-1: Void

6.4C.2.2.5	Test requirement

Same test requirement for carrier leakage measured on the SUL carrier as specified in 6.4.2.2.5.

<End of Changes>