3GPP TSG-RAN5 Meeting #90-e	R5-211015

# Electronic Meeting, 22nd February – 5th March 2021

Title:	Discussion on test coverage for reference sensitivity for EN-DC configs with exceptions

Source:	Huawei, Hisilicon

Agenda item:	5.3.2.17

Document for:	Endorsement

Introduction

During RAN5#89-e the following proposals for test coverage of refsens was agreed in [1]:

Proposal 2: For EN-DC configurations with exceptions, add test coverage for standalone FR1 requirement when the exception is avoided whenever possible for NSA only UE.

Proposal 3: Also for SA+NSA UE add test coverage for standalone FR1 requirement when the exception is avoided whenever possible.

The above proposals have confirmed the necessity of exception avoiding testing for EN-DC refsens testing case if possible. But it was left to this meeting which exception types are needed in both SA and EN-DC mode and which UL configuration shall be configured, as the following proposal indicated in [1]:

Proposal 4b: Until next RAN5 meeting, analyze which of the exception types that necessitate testing for NR carrier in both standalone and EN-DC mode for a certain EN-DC configuration and which UL configuration that shall be configured (LTE UL, NR UL or LTE+NR UL)

The purpose of this document is to analyze how to select test points for inter-band 2CC EN-DC so that both exception avoiding and NR single carrier requirements could be covered. Moreover, exception testing in >2CC is also be discussed.

Inter-band 2CC EN-DC with exceptions

Exception avoiding test

For EN-DC configs with exception, exception avoiding testing is needed when the exception can be avoided under certain test configuration. When testing exception avoiding, single carrier requirements are applicable since the exception does little affect the refsens requirements of DL carrier although it exists. The implication of “avoided” is “exist but little affect”.

For inter-band EN-DC configs there are 4 types of exceptions as follows:

- UL harmonic interference (HD)

HD exception applies when the transmitted harmonics of a UL carrier on the aggressor band falls inside the DL carrier on the victim band. For inter-band EN-DC configs, the aggressor is the UL carrier on the lower frequency band while the victim is the DL carrier on the higher frequency band.

- Receiver Harmonic Mixing (HM)

HM exception applies when the receiving harmonic mixing of a UL carrier on the aggressor band falls inside the DL carrier on the victim band. For inter-band EN-DC configs, the aggressor is the UL carrier on the higher frequency band while the victim is the DL carrier on the lower frequency band.

- Cross band isolation (CBI)

CBI exception applies when the frequency range of the aggressor UL band is very close to that of the victim DL band so that DL carrier is impacted by the UL carrier on the aggressor band. For inter-band EN-DC configs, CBI could not be avoided as long as the carriers on both aggressor band and victim band are activated.

- Intermodulation due to dual uplink (IMD)

IMD exception applies when the intermodulation products of two uplink carriers fall inside the DL carrier. Given that RAN4 has specified the MSD test point for IMD exception due to dual uplink operation for EN-DC configs, RAN5 is not able to define other TPs to make IMD exception avoided.

By analysing the cause of the four types of exception, it can be deduced which types of exception can be avoided and which can’t. HD/HM exception can be avoided by designing the frequency of aggressor UL carrier and victim DL carrier does not meet the harmonic relationship. CBI exception could not be avoided since it is caused by the whole band. IMD exception also could not be avoided since RAN4 does not define non-exception test points with dual uplink operation.

Proposal 1: Exception avoiding testing is only defined in HD/HM exception since CBI and IMD are exceptions which could not be avoided.

As introduced previously, the UL aggressor of HD exception is in the lower frequency band while UL aggressor of HM exception is in the higher frequency band. Take DC_3_n78 as an example, NR band n78 works as the DL victim band for HD exception while it works as the UL aggressor band for HM exception. For EN-DC configs with both HD and HM exceptions, regarding TP selection of exception avoiding there are two options RAN5 may consider to adopt:

- Option 1: Only one TP is designed to cover both HD and HM exceptions.

- Option 2: Two TPs are designed to cover HD exception and HM exception, respectively.

For Option 1, the test point needs to configure 2 UL active since the UL aggressor carrier are in the different band. However, DC_3_n78 is additionally affected by IMD exception, the IMD exception caused by dual uplink might result in NR single carrier requirements not met. Therefore, Option 2 is more preferred than Option 1 to cover exception avoiding test for EN-DC configs with both HD and HM exceptions

Proposal 2: For EN-DC configs with both HD and HM exceptions, it’s suggested that two TPs are designed to cover HD exception and HM exception, respectively.

By designing that the frequency of aggressor UL carrier and victim DL carrier does not meet the harmonic relationship, HD exception avoiding and HM exception avoiding can be tested using Test ID 1 and Test ID 2 respectively as show in Table 2.1-1.

Table 2.1-1: Exception avoiding TPs for DC_3_n78

| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_3A_n78 Configuration |  |  |  |  |  |  |  |  |
| 1 | 3 | Mid 1747.5MHz |  |  | n78 | 3595 MHz |  |  |
|  | QPSK | QPSK | 20 | All RBs / REFSENS_ENDC_1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 100 | All RBs/0 |
| 2 | 3 | Low 1815 MHz |  |  | n78 | High 3750 MHz |  |  |
|  | QPSK | QPSK | 20 | All RBs/0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | 20 | All RBs / REFSENS_ENDC_2 |

According to Table 7.3B.2.3.1-1 in 38.101-3, the 2nd order harmonics may hit into the DL victim band or fall into the adjacent frequency of the DL victim band when DL bandwidth is no larger than 20 MHz. For HD exception affected by the 2nd order harmonics, only the “hit” exception is considered in exception avoiding testing since “adjacent” interference allows very low MSD.

NR single carrier requirements

For EN-DC affected by exceptions, test coverage includes not only exceptional requirements but also NR single carrier requirements. For those exceptions that NR band is not the DL affected band, in exception testing single carrier requirements specified in 38.101-1 are still applied to the NR carrier of EN-DC configs since it’s not affected by interferences. Hence, for EN-DC configs with exceptions that NR band is not the affected DL band, it can be considered that NR single carrier requirements have been covered in exception testing.

Proposal 3: For EN-DC configs with exceptions that NR band is not the affected DL band, it can be considered that NR single carrier requirements have been covered in exception testing.

As for EN-DC configs that NR band is always the affected DL band for all exceptions, additional testing may need to be designed to cover NR single carrier requirements. As previously mentioned, since exception avoiding TPs also apply to single carrier requirements, exception avoiding testing can be viewed as an approach to cover NR single carrier requirements. Therefore, there are three ways that can be used to cover NR single carrier requirements for EN-DC configs:

- Exception avoiding TP

- Applicable to EN-DC configs with HD/HM exception.

- More critical than other ways

- NR SA testing in 38.521-1

- Applicable to SA+NSA capable UEs

- LTE anchor agnostic

- Applicable to NSA only capable UEs

Considering that exception avoiding TP has more critical test condition than standalone tests, it has higher testing priority when comes to cover NR single carrier requirements compared to another two ways. Therefore, if exception avoiding has been tested for one EN-DC config, it can be considered that NR single carrier requirements have been covered.

Proposal 4: If exception avoiding has been tested, it can be considered that NR single carrier requirements have been covered.

For EN-DC configs without HD/HM exception, there is no such an exception avoiding TP that could cover NR single carrier requirements. In this case, to cover NR single carrier requirements, SA testing in 38.521-1 needs to be performed for NSA+SA capable UEs while LTE anchor agnostic is used for NSA  only capable UEs.

Proposal 5: For NSA+SA capable UEs, SA testing in 38.521-1 needs to be performed to cover non-exceptional requirements only if EN-DC configs without HD/HM exception or exceptions that NR band is not the affected DL band.

Proposal 6: For NSA only capable UEs, LTE anchor agnostic is used to cover non-exceptional requirements only if EN-DC configs without HD/HM exception or exceptions that NR band is not the affected DL band.

2UL intermodulation exception

According to 38.101-3, UE is mandated to support dual uplink operation for inter-band EN-DC configs indicating “No” in column single uplink allowed in Table 5.5B.4.1-1 since the IMD products caused by dual uplink do not interfere with its own primary downlink transmission. For other inter-band EN-DC configs indicating “DC_X_nY” in column single uplink allowed Table 5.5B.4.1-1, due to 2UL IMD interference UE is optional to support dual uplink operation, even may indicate that simultaneous UL transmissions are not allowed by the support of singleUL-Transmission in the UE-MRDC-Capability IE as in 38.306 [2].

| singleUL-Transmission Indicates that the UE does not support simultaneous UL transmissions as defined in TS 38.101-3 [4]. The UE may only include this field for certain band combinations defined in TS 38.101-3 [4]. If included for a particular band combination, the field applies to all fallback band combinations of this band combination that are defined in TS 38.101-3 [4] as being allowed to include this field and does not apply to any other fallback band combinations defined in TS 38.101-3 [4]. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

For inter-band EN-DC configs with the support of UE capability singleUL-Transmission, if the base station still insists on scheduling 2UL that is not consistent with UE capability indication, there would be two possibilities about UE’s behavior:

- UE follows 2UL schedule, but the receiving performance does not have to meet the RAN 4 requirements as the RAN plenary conclusion in [4].

- UE only schedules 1 UL, then it’s pointless to test 2UL IMD exception because actually no IMD products occurs.

No matter the UE behaves like 1 or 2, the EN-DC refsens due to the IMD exception can’t be tested correctly. Therefore, it is proposed that exception requirements due to IMD exception only are tested for those inter-band EN-DC configs without the support of UE capability singleUL-Transmission.

Proposal 7: For EN-DC combination with 2UL IMD exception, exception requirements due to IMD only are tested for UEs not supporting UE capability singleUL-Transmission.

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A | No |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |
| DC_1A_n8A | DC_1A_n8A | No |  |
| DC_1A_n20A | DC_1A_n20A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |
| DC_1A_n40A | DC_1A_n40A | No |  |
| DC_1A_n41A | DC_1A_n41A | No |  |
| DC_1A_n50A | DC_1A_n50A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n77(2A) | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No | No |
| … | … | … | … |

Overall TP selection

Based on proposals above, the overall TP selection for inter-band 2CC EN-DC with exceptions can be summarized in the following Table 2.4-1.

Table 2.4-1: TP selection for 	inter-band 2CC EN-DC affected by exceptions

| Exception type | NSA+SA capable UEs | NSA only capable UEs |
| --- | --- | --- |
| HD exception applied | 1 TP w harmonic relation | 1 TP w harmonic relation |
| HD exception avoiding | 1 TP w/o harmonic relation | 1 TP w/o harmonic relation |
| HM exception applied | 1 TP w harmonic relation | 1 TP w harmonic relation |
| HM exception avoiding | 1 TP w/o harmonic relation | 1 TP w/o harmonic relation |
| CBI exception | 1 TP | 1 TP |
| IMD exception2 | 1 TP with 2 UL active | 1 TP with 2 UL active |
| NR single carrier require.3 | SA testing in 38.521-1 | LTE anchor agnostic |
| Note 1: Unless specified otherwise, TPs are configured with 1 UL active. Note 2: Applicable to EN-DC configs without the support of UE capability singleUL-Transmission. Note 3: EN-DC configs without HD/HM exceptions or exceptions that NR band is not the affected band. |  |  |

EN-DC with >2CC

Based on the proposals agreed in [3]:

- Let the 2CC test cases cover all refsens exceptions (as done already). .

- The 2CC test cases always need to be tested even for UE supporting more CCs since some of the exceptions is not covered in >2CC test cases.

- Let the 3CC-4CC test cases cover mainly non-exceptions, for EN-DC configurations where no exception apply (anchor agnostic testing). If 2UL intermodulation exception always apply, include testing of this exception in all test cases.

In the testing structure of EN-DC refsens, 2CC test cases mainly covers exception requirement and >2CC test cases mainly cover non-exception requirements. For most >2CC EN-DC affected with refsens exceptions, 2CC fallback is sufficient to test exception requirements. But for >2CC EN-DC configs with one of the following situations, exception requirements need to be tested in >2CC test cases:

- 2UL IMD exception for three bands

- No 2CC fallback EN-DC that could cover HD/HM/CBI exception.

With respect to the 2nd situation, take DC_20A_n28A-n75A as an example, band n75 is affected by the UL harmonic interference from band n28 but there is no any 2CC EN-DC config that could cover the HD exception. Similar to 2CC EN-DC, HD exception avoiding also shall be tested in 3CC cases for DC_20A_n28A-n75A.

Proposal 8: For >2CC EN-DC without the fallback that could cover 2CC exception, exception requirements need to be tested in >2CC test cases, and the exception avoiding testing also shall be considered.

Proposal

For EN-DC Rx test cases, it is proposed RAN5 consider below proposals.

Proposal 1: Exception avoiding testing is only defined in HD/HM exception since CBI and IMD are exceptions which could not be avoided.

Proposal 2: For EN-DC configs with both HD and HM exceptions, it’s suggested that two TPs are designed to cover HD exception and HM exception, respectively.

Proposal 3: For EN-DC configs with exceptions that NR band is not the affected DL band, it can be considered that NR single carrier requirements have been covered in exception testing.

Proposal 4: If exception avoiding has been tested, it can be considered that NR single carrier requirements has been covered.

Proposal 5: For NSA+SA capable UEs, SA testing in 38.521-1 needs to be performed to cover non-exceptional requirements only if EN-DC configs without HD/HM exception or exceptions that NR band is not the affected DL band.

Proposal 6: For NSA only capable UEs, LTE anchor agnostic is used to cover non-exceptional requirements only if EN-DC configs without HD/HM exception or exceptions that NR band is not the affected DL band.

Proposal 7: For EN-DC combination with 2UL IMD exception, exceptional requirements due to IMD only can be tested for UEs not supporting UE capability singleUL-Transmission.

Proposal 8: For >2CC EN-DC without the fallback that could cover 2CC exception, exception requirements need to be tested in >2CC test cases, and the exception avoiding testing also shall be considered.

References

[1] R5-206841: “Discussion on test points in Receiver test cases for EN-DC configurations with exception requirements”, RAN5#89-e.

[2] 3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

[3] R5-204953: “Discussion on EN-DC reference sensitivity test case structure”, RAN5#88-e.

[4] RP-172085, Signalling for Single/Dual UL Tx, RAN2 Chairman