3GPP TSG-RAN5 Meeting #90-e	R5-211033

# Electronic Meeting, 22 Feb – 5 Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1760 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction test frequencies for CA_n261(2A) for protocol testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Referenses in test frequencies for protocol testing and NR Intra-Band non-contiguous CA configuration CA_n261(2A) need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR Intra-Band non-contiguous CA configuration CA_n261(2A) corrected in Table 6.2.3.4-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Protocol testing of NR Intra-Band non-contiguous CA configuration CA_n261(2A) not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.3.4	Test frequencies for NR CA configurations for signalling testing

The default channel bandwidths for NR CA signalling test are specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For NR CA Inter-band case (2 bands) the NR CA configurations are specified in clause 4.3.1.1.2 and NR test frequencies are specified in clause 6.2.3.1 for the NR band used as PCell (NRf1, NRf2, NRf3, NRf4) and for the NR band used as Scell (NRf5, NRf6, NRf7).

For NR CA Intra-band Contiguous case (2 CCs) the NR CA configurations and the test frequencies are specified in Table 6.2.3.4-1 for FR1 and in Table 6.2.3.4-2 for FR2. For NR CA Intra-band Non-Contiguous (2 CCs) case the NR CA configurations and test frequencies are specified in Table 6.2.3.4-3 for FR1 and in Table 6.2.3.4-4 for FR2.

For NR CA Intra-Band Contiguous case (2 CCs) and NR CA Intra-Band Non-Contiguous case (2 CCs) the mapping of frequency ranges to NR test frequencies NRf1, NRf2, NRf3, and NRf4 and PCell (CC1) amd SCell (CC2) are as follows:

-	for Intra-band configurations with only one test frequency: Low Range (NRf1=CC1 and NRf2=CC2); and

-	for Intra-band configurations with up to two frequencies: Low Range (NRf1=CC1 and NRf2=CC2), High Range (NRf3=CC1 and NRf4=CC2).

Table 6.2.3.4-1: Test frequencies for NR CA Intra-band Contiguous configurations with FR1

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41C | CC1, CC2 | 60+60 | 162+162 | Downlink & Uplink | Low | Same values as for Low range in Table 4.3.1.1.3.41.1-1 for CBW combination 60+60 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B | CC1 | 10+ | 52+ | Downlink | Low | Same values as for Low range in Table 4.3.1.1.3.66.1-1 for CBW combination 10+15 and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 79 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78C | CC1, | 100+100 | 273+ | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.3.78.1-1 for CBW combination 100+100 and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  | 273 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-2: Test frequencies for NR CA Intra-band Contiguous configurations with FR2

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n257G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.1.6-3 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.4.6-2 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n261G | CC1, | 100+100 | 66+66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.3.5.6-2 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-3: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR1

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A) | CC1, | 10+10 | 52+52 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.1.5.66-1 for CBW combination 10+10. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  |  | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-4: Test frequencies for NR CA Intra-Band Non-Contiguous configurations with FR2

| NR CA configuration | CC | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n261(2A) | CC1 | 100 | 66 | Downlink | Low | Same values as for Low and High ranges in Table 4.3.1.2.4.5.1-1 for CBW combination 100+100 and SCS=120 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 66 | & Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |