3GPP TSG-RAN WG5 Meeting #90-e	R5-211045

# Online, , 22nd Feb 2021 – 5th Mar 2021

# Source:	Qualcomm Incorporated

# Title:	Discussion paper on impact of UE’s highest channel bandwidth capability on Transport Block Selection test cases

# Document for:	Agreement

Agenda Item:	6.3.2.20

## 1.	Introduction

The purpose of this paper is to discuss the impact of UE’s support of highest channel bandwidth on Transport Block Selection test cases in [1].

## 2.	Discussion

UE can indicate the supported channel bandwidths for each subcarrier spacing in UE Capability Information message as defined in [2].

In Transport Block Selection test cases of [1], to enable testing of nPRB up to maximum value, NR cell bandwidth is set to highest value for Band under test as specified in Table 5.3.5-1 of [3]. These test cases fail, when executed on a UE which does not support highest channel bandwidth.

## [[2], cl 4.2.7.2]

channelBWs-DL

Indicates for each subcarrier spacing the UE supported channel bandwidths.
Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry.

## …

channelBWs-UL

Indicates for each subcarrier spacing the UE supported channel bandwidths.

Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry.

## 3.   Summary

Based on the discussion in section 2, impacted testcases of [1] needs to be updated to ensure that UE’s supported highest channel bandwidth is configured instead of maximum value defined in [3].

## 4.   Proposal

Proposal #1 – For all impacted test cases of [1], configure NR Cell bandwidth to highest channel bandwidth supported by UE by defining a new PIXIT parameter with highest channel bandwidth for band under test.

Proposal #2 – Configure highest channel bandwidth for band under test based on received UE Capability Information message from UE.

## 5.	References

## [1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 38.306: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio access capabilities

## [3]	TS 38.101-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception