3GPP TSG-RAN5 Meeting #90-e	R5-211160

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0297 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.3.2.1.3 2Rx FDD FR1 Multiple PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the work plan, a new test case 6.3.2.1.3 should be introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new test case 6.3.2.1.3 4Rx FDD FR1 Multiple PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA. Add MU and TT for the new test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This WI will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.3 (New) F.1.1.3 F.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR 38.522 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

6.3.2.1.3	2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA

6.3.2.1.3.1	Test purpose

To test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

6.3.2.1.3.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.3.3-2.

Table 6.3.2.1.3.3-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLC300-5 |
| Antenna configuration |  |  | High XP 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5, -) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

Table 6.3.2.1.3.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 2.5 |

6.3.2.1.3.4	Test description

6.3.2.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 6.1.2-1 and Table 6.3.2.1.3.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On for SA or (EN-DC, DC bearer MCG and SCG, Connected without Release On) for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.3.2.1.3.4.3.

6.3.2.1.3.4.2	Test procedure

1.	Set the parameters of bandwidth, the propagation condition, antenna configuration and measurement channel according to Table 6.3.2.1.3.3-1 as appropriate.

2.	The SS shall transmit PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC with precoding matrix according to PMI report from the UE. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format [0_1] with aperiodic CSI request triggered. Establish and  according to Annex G.3.2.

3.	Set SNR to. The SS shall transmit PDSCH with randomly selected precoding matrix from codebook (Table 5.2.2.2.1-5 in TS 38.214 [12]) every slot regardless of PMI reports from the UE. Note that each precoding matrix shall be selected in equal probabilities. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format [0_1] with aperiodic CSI request triggered. Measure according to Annex G.3.3.

4.	Calculate. If the ratio  which is specified in table 6.3.2.1.3.5-1, then the test is pass. Otherwise, the test is fail.

6.3.2.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

6.3.2.1.3.4.3.1	Message exceptions for SA

Table 6.3.2.1.3.4.3.1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | aperiodic |  |  |
| } |  |  |  |

Table 6.3.2.1.3.4.3.1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table 5.4.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 011110 |  |  |
| } |  |  |  |
| nrofPorts | P16 |  |  |
| firstOFDMSymbolInTimeDomain | 5 |  |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| } |  |  |  |

Table 6.3.2.1.3.4.3.1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table 5.4.2-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.2.1.3.4.3.1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |

Table 6.3.2.1.3.4.3.1-5: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| four-two-TypeI-SinglePanel-Restriction | FFFF FFFF FFFF FFFF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000010 |  |  |

Table 6.3.2.1.3.4.3.1-6: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 5 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| pmi-FormatIndicator | subbandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | 1111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.3.2.1.3.4.3.2	Message exceptions for NSA

Same as in clause 6.3.2.1.3.4.3.1.

6.3.2.1.3.5	Test requirement

Table 6.3.2.1.3.5-1: Test requirement

| Parameter | Test 1 |
| --- | --- |
|  | 2.49 |

<End of change 1>

<Start of change 2>

F.1.1.3	Measurement of Channel State Information reporting

<Unchanged part skipped>

Table F.1.1.3-2: Maximum test system uncertainty for FR1 channel state information reporting test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | +/- 0.3 dB | Overall system uncertainty for AWGN conditions comprises: Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 6.2.2.1.2.1 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | +/- 0.8 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty ±0.3 dB 2. Fading profile power uncertainty for 2Tx ±0.7 dB Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |
| 6.2.2.1.2.2 2Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.2.2.1.1 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 | Same as 6.2.2.1.1.1 |
| 6.2.2.2.2.1 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.2.2.2.2 2Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.1.2.1 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.1.2.2 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.2.2.1 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.2.2.2 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.2.1.1 2Rx FDD FR1 Single PMI with 4Tx Type I- SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.2.1.2 2Rx FDD FR1 Single PMI with 8Tx Type I – SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.2.1.3 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.1.1.1 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 | Same as 6.2.2.1.1.1 |
| 6.2.3.1.2.1 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.1.2.2 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.2.1.1 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Same as 6.2.2.1.1.1 | Same as 6.2.2.1.1.1 |
| 6.2.3.2.2.1 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.2.3.2.2.2 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.3.1.1 Single PMI with 4TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.3.1.2 Single PMI with 8TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.3.2.1 4Rx TDD FR1 Single PMI with 4Tx Type1 - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.2.2.1 2Rx TDD FR1 Single PMI with 4Tx TypeI - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.2.2.2 2Rx TDD FR1 Single PMI with 8Tx TypeI - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.3.3.2.2 4Rx TDD FR1 Single PMI with 8Tx Type1 - SinglePanel codebook for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.4.2.1_1 2Rx FDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.4.2.2_1 2Rx TDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.4.3.1_1 4Rx FDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| 6.4.3.2_1 4Rx TDD FR1 RI reporting for both SA and NSA | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |

<End of change 2>

<Start of change 3>

F.1.3.3	Measurement of Channel State Information reporting

The derivation of the test requirements for the test cases in section 6 is defined in Table F.1.3.3-1.

Table F.1.3.3-1: Derivation of Test Requirements (FR1 channel state information reporting tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.2.1 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.1.2.2 2Rx FDD FR1 periodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.2.2.1.1 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.2.2.1 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.2.2.2 2Rx TDD FR1 periodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.3.1.1.1 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.1.2.1 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  5% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.3.1.2.2 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.2.3.2.1.1 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.3.2.2.1 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  5% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.3.2.2.2 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | SNRs as specified  2% 55% 1.05 BLER 0.02 | SNR 0 dB  0% 0% 0.01 BLER 0 | SNR unchanged  limit unchanged limit unchanged  1.04 BLER limit unchanged |
| 6.3.2.1.1 2Rx FDD FR1 Single PMI with 4Tx Type I- SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.2.1.2 2Rx FDD FR1 Single PMI with 8Tx Type I – SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.2.1.3 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | SNRs as specified  2.50 | SNR 0 dB 0.01 | SNR unchanged  2.49 |
| 6.3.2.2.1 2Rx TDD FR1 Single PMI with 4Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.2.2.2 2Rx TDD FR1 Single PMI with 8Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.3.1.1 Single PMI with 4TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.3.1.2 Single PMI with 8TX TypeI-SinglePanel Codebook– SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.3.3.2.1 4Rx TDD FR1 Single PMI with 4Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 1.30 | SNR 0 dB 0.01 | SNR unchanged  1.29 |
| 6.3.3.2.2 4Rx TDD FR1 Single PMI with 8Tx Type1 - SinglePanel codebook for both SA and NSA | SNRs as specified 1.50 | SNR 0 dB 0.01 | SNR unchanged  1.49 |
| 6.4.2.1_1 2Rx FDD FR1 RI reporting for both SA and NSA | SNRs as specified 1.00 for Test 1 1.05 for Test 2 0.90 for Test 3 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 | SNR unchanged  0.99 for Test 1  1.04 for Test 2  0.89 for Test 3 |
| 6.4.2.2_1 2Rx TDD FR1 RI reporting for both SA and NSA | SNRs as specified 1.00 for Test 1 1.05 for Test 2 0.90 for Test 3 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 | SNR unchanged  0.99 for Test 1  1.04 for Test 2  0.89 for Test 3 |
| 6.4.3.1_1 4Rx FDD FR1 RI reporting for both SA and NSA | SNRs as specified 0.90 for Test 1 1.05 for Test 2 0.90 for Test 3 0.90 for Test 4 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 0.01 for Test 4 | SNR unchanged  0.89 for Test 1  1.04 for Test 2  0.89 for Test 3  0.89 for Test 4 |
| 6.4.3.2_1 4Rx TDD FR1 RI reporting for both SA and NSA | SNRs as specified 0.90 for Test 1 1.05 for Test 2 0.90 for Test 3 0.90 for Test 4 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 0.01 for Test 4 | SNR unchanged  0.89 for Test 1 as per Table G.3.4  1.04 for Test 2 as per Table G.3.4  0.89 for Test 3 as per Table G.3.4  0.89 for Test 4 as per Table G.3.4 |

<End of change 3>