3GPP TSG-RAN5 Meeting #90-e	R5-211233

# Electronic Meeting, 22 Feb – 5 Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1792 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for NR band n48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The channel bandwidth >=50 MHz for SCS 15, 30 and 60 kHz is in TS 38.101 restricted to downlink operation as SCell in CA configurations. Current test frequencies specifies UL and parameters for CORESET#0 for carrier to operate as PCell. The test frequency tables for n48 need to be updated to only specify DL operation as SCell for CBW>=50Mhz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test frequency tables for n48 removing UL and CORESET#0 parameters. Adding notes for the CBW only used for downlink SCell operation in CA configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies not aligned to core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.48 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.48	Reference test frequencies for NR operating band n48

Table 4.3.1.1.1.48-1: Test frequencies for NR operating band n48 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 3552.51 | 636834 | 3550.26 | 636684 | 0 | (Note 3) | - | - | - | - | - | - |
|  |  | & | Mid | 3625.005 | 641667 | 3604.395 | 640293 | 102 |  | - | - | - | - | - | - |
|  |  | Uplink | High | 3697.5 | 646500 | 3604.53 | 640302 | 504 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3601.95 | 640130 | 102 |  | 7933 | 641664 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3599.52 | 639968 | 102 |  | 7931 | 641472 | 4 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 20 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3597.09 | 639806 | 102 |  | 7930 | 641376 | 10 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 40 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3587.19 | 639146 | 102 |  | 7923 | 640704 | 10 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 50 | 270 | Downlink | Low | 3575.01 | 638334 | 3550.71 | 636714 | 0 | 30 | - | 637056 | - | - | - | - |
|  |  | (Note 4) | Mid | 3624.99 | 641666 | 3582.33 | 638822 | 102 |  | - | 640320 | - | - | - | - |
|  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell. Note 4: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-2: Test frequencies for NR operating band n48 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3583.95 | 638930 | 102 |  | 7933 | 641664 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3581.43 | 638762 | 102 |  | 7931 | 641472 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3579.09 | 638606 | 102 |  | 7930 | 641376 | 10 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3569.19 | 637946 | 102 |  | 7923 | 640704 | 22 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 3575.01 | 638334 | 3551.07 | 636738 | 0 | 30 | - | 637056 | - | - | - | - |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3564.33 | 637622 | 102 |  | - | 640320 | - | - | - | - |
|  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | - | 643680 | - | - | - | - |
| 60 | 162 | Downlink | Low | 3580.02 | 638668 | 3550.86 | 636724 | 0 | 30 | - | 637056 | - | - | - | - |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3559.11 | 637274 | 102 |  | - | 640032 | - | - | - | - |
|  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | - | 643008 | - | - | - | - |
| 80 | 217 | Downlink | Low | 3590.01 | 639334 | 3550.95 | 636730 | 0 | 30 | - | 637056 | - | - | - | - |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3549.21 | 636614 | 102 |  | - | 639360 | - | - | - | - |
|  |  |  | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  | - | 641664 | - | - | - | - |
| 90 | 245 | Downlink | Low | 3595.02 | 639668 | 3550.92 | 636728 | 0 | 30 | - | 637056 | - | - | - | - |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3544.17 | 636278 | 102 |  | - | 638976 | - | - | - | - |
|  |  |  | High | 3654.99 | 643666 | 3429.45 | 628630 | 504 |  | - | 640992 | - | - | - | - |
| 100 | 273 | Downlink | Low | 3600 | 640000 | 3550.86 | 636724 | 0 | 30 | - | 637056 | - | - | - | - |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3539.13 | 635942 | 102 |  | - | 638688 | - | - | - | - |
|  |  |  | High | 3649.98 | 643332 | 3419.4 | 627960 | 504 |  | - | 640320 | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-3: Test frequencies for NR operating band n48 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3555 | 637000 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3547.59 | 636506 | 102 |  | - | 641642 |
|  |  | Uplink | High | 3694.98 | 646332 | 3328.14 | 621876 | 504 |  | - | 646308 |
| 15 | 18 | Downlink | Low | 3557.52 | 637168 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3545.07 | 636338 | 102 |  | - | 641474 |
|  |  | Uplink | High | 3692.49 | 646166 | 3323.13 | 621542 | 504 |  | - | 645974 |
| 20 | 24 | Downlink | Low | 3560.01 | 637334 | 3551.37 | 636758 | 0 | 30 | - | 636998 |
|  |  | & | Mid | 3624.99 | 641666 | 3542.91 | 636194 | 102 |  | - | 641330 |
|  |  | Uplink | High | 3690 | 646000 | 3318.48 | 621232 | 504 |  | - | 645664 |
| 40 | 51 | Downlink | Low | 3570 | 638000 | 3551.64 | 636776 | 0 | 30 | - | 637016 |
|  |  | & | Mid | 3624.99 | 641666 | 3533.19 | 635546 | 102 |  | - | 640682 |
|  |  | Uplink | High | 3679.98 | 645332 | 3298.74 | 619916 | 504 |  | - | 644348 |
| 50 | 65 | Downlink | Low | 3575.01 | 638334 | 3551.61 | 636774 | 0 | 30 | - | 637014 |
|  |  | (Note 2) | Mid | 3624.99 | 641666 | 3528.15 | 635210 | 102 |  | - | 640346 |
|  |  |  | High | 3675 | 645000 | 3288.72 | 619248 | 504 |  | - | 643680 |
| 60 | 79 | Downlink | Low | 3580.02 | 638668 | 3551.58 | 636772 | 0 | 30 | - | 637012 |
|  |  | (Note 2) | Mid | 3624.99 | 641666 | 3523.11 | 634874 | 102 |  | - | 640010 |
|  |  |  | High | 3669.99 | 644666 | 3278.67 | 618578 | 504 |  | - | 643010 |
| 80 | 107 | Downlink | Low | 3590.01 | 639334 | 3551.49 | 636766 | 0 | 30 | - | 637006 |
|  |  | (Note 2) | Mid | 3624.99 | 641666 | 3513.03 | 634202 | 102 |  | - | 639338 |
|  |  |  | High | 3660 | 644000 | 3258.6 | 617240 | 504 |  | - | 641672 |
| 90 | 121 | Downlink | Low | 3595.02 | 639668 | 3551.46 | 636764 | 0 | 30 | - | 637004 |
|  |  | (Note 2) | Mid | 3624.99 | 641666 | 3507.99 | 633866 | 102 |  | - | 639002 |
|  |  |  | High | 3654.99 | 643666 | 3248.55 | 616570 | 504 |  | - | 641002 |
| 100 | 135 | Downlink | Low | 3600 | 640000 | 3551.4 | 636760 | 0 | 30 | - | 637000 |
|  |  | (Note 2) | Mid | 3624.99 | 641666 | 3502.95 | 633530 | 102 |  | - | 638666 |
|  |  |  | High | 3649.98 | 643332 | 3238.5 | 615900 | 504 |  | - | 640332 |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |