3GPP TSG-RAN5 Meeting #90-e	R5-210286r1

# Online, , 22th Feb 2021 - 5th Mar 2021

Title:						Discussion about n259 OBW relaxation

Source:			DOCOMO Communications Lab.

Agenda Item:			5.3.11.10

Document for:			Endorsement

- Introduction

During the last meeting RAN5#88 (Aug-2020), WF of Band n259 (39.5 to 43.5 GHz) was updated[1]. The purpose of this contribution is to provide Test Tolerance (TT) proposals for P1 and P2 test cases.

- Discussion

2.1 Discussions of RAN5#89

The scope of this contribution is to define “Upper limit of MU (for TT calculation)” and “Upper limit of Relaxation (due to testability issue)” for P1 and P2 test cases. Progress about the upper limit of MU and relaxation for Band n259 P1 and P2 test cases after RAN5#89 is as below:

Table 2.1-1 Upper limit of MU and Relaxation

| TC | Upper limit (MU for TT calculation) | Upper limit (relaxation due to testability issue) |
| --- | --- | --- |
| MOP (Max TRP) | 5.5 dB [2] | 0 dB [6] |
| REFSENS | 5.8 dB [2] | 0 dB [6] |
| Frequency Error | 0.01 PPM [2] | 0 dB [6] |
| SEM | 5.8 dB [2] | 0 dB [6] |
| OFF power | Not required because TT = 0 dB [3] | FR2b relaxation + 2.23dB [6] |
| OBW | Not required because TT = 0 dB [4] | TBD |
| ACLR | 7.59 dB [6] | value in table 2.1-2 +0.5dB [6] |
| General spurious | Not required because TT = 0 dB [4] | 0 dB [6] |
| Spurious co-existence | Not required because Fixed value has been specified [5] | Not required because Fixed value has been specified [5] |
| Inband blocking | Not required because TT = 0 dB [4] | value in table 2.1-3 + 0.5dB [6] |
| ACS | Not required because TT = 0 dB [4] | value in table 2.1-3 + 0.5dB [6] |
| Rx spurious | Not required because TT = 0 dB [4] | f <= 66GHz : Same as FR2b value. 66GHz<=f<=87GHz [6] FR2b 66GHz relaxation+ 3.9 dB [6] |

Table 2.1-2 Estimation Relaxation for FR2c ACLR

| Test Point MPR | T(MPR) | 400MHz | 200MHz | 100MHz | 50Mhz |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.28 | 0.00 | 0.00 | 0.00 |
| 0.5 | 1.5 | 2.28 | 0.00 | 0.00 | 0.00 |
| 1 | 1.5 | 2.78 | 0.00 | 0.00 | 0.00 |
| 1.5 | 1.5 | 3.28 | 0.27 | 0.00 | 0.00 |
| 2 | 1.5 | 3.78 | 0.77 | 0.00 | 0.00 |
| 2.5 | 2 | 4.78 | 1.77 | 0.00 | 0.00 |
| 3 | 2 | 5.28 | 2.27 | 0.00 | 0.00 |
| 3.5 | 3 | 6.78 | 3.77 | 0.76 | 0.00 |
| 4 | 3 | 7.28 | 4.27 | 1.26 | 0.00 |
| 4.5 | 4 | 8.78 | 5.77 | 2.76 | 0.00 |
| 5 | 4 | 9.28 | 6.27 | 3.26 | 0.25 |
| 5.5 | 5 | 10.78 | 7.77 | 4.76 | 1.75 |
| 6 | 5 | 11.28 | 8.27 | 5.26 | 2.25 |
| 6.5 | 5 | 11.78 | 8.77 | 5.76 | 2.75 |
| 7 | 5 | 12.28 | 9.27 | 6.26 | 3.25 |
| 7.5 | 5 | 12.78 | 9.77 | 6.76 | 3.75 |
| 8 | 5 | 13.28 | 10.27 | 7.26 | 4.25 |
| 8.5 | 5 | 13.78 | 10.77 | 7.76 | 4.75 |
| 9 | 5 | 14.28 | 11.27 | 8.26 | 5.25 |
| 9.5 | 5 | 14.78 | 11.77 | 8.76 | 5.75 |

Table 2.1-3 Estimated relaxation for ACS and IBB for n259

|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- | --- |
| ACS Case1, IBB | Pinterferer [dBm] | -49.45 | -46.45 | -43.45 | -40.45 |
|  | Relaxation [dB] | 8.28 | 11.28 | 14.28 | 17.28 |
| ACS Case 2 | Pinterferer [dBm] | -25 | -25 | -25 | -25 |
|  | Relaxation [dB] | 32.73 | 32.73 | 32.73 | 32.73 |

2.2 Relaxation proposal

Based on the WF, contributions will be submitted in this RAN5#90-e meeting according to the pre-meeting MU lists. References and proposals can be summarized as below.

Table 2.2-1 Estimation about Relaxation (FR2c)

| Test cases | Ref. [7] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBW | BW [MHz] | Option 1 |  | Option 2 |  |
|  |  | MU [%CBW] | Relaxation for OBW req. [%CBW] | MU%[CBW] | Relaxation for OBW definition (%) |
|  | 50, 100(SC) | 0.8 | 0 | Same as Option 1 | Same as Option 1 |
|  | 200, 400(SC) | 20.0 | 20.0 | 2.3 | 1 (98% of Ptotal) |
|  | BWagg≤100(CA) | 1.5 | 0 | Same as Option 1 | Same as Option 1 |
|  | BWagg ≤400(CA) | 40.0 | 40.0 | 3.7 | 3 (96% of Ptotal) |

Relaxation value due to testability issue is estimated in [7]. In [7], 3 Options are given for how to apply relaxation.

Option 1 : Apply the relaxation for the OBW requirement (e.g. 100MHz for 100MHz BW)

Option 2 : Keep the OBW requirement but change the OBW definition (BW containing 99% of total power) in test procedure

Option 3 : Do not test in conformance test

Which option applies is discussed in [7].  This paper proposes the upper limit of  Relaxation (due to testability issue) for each option.

Proposal 1			: To define FR2c Upper limit of relaxation (due to testability issue) as Table2.2-2 in case relaxation is applied for the OBW requirement.

Table2.2-2 Proposed upper limits of MU and relaxation (applied for the OBW requirement) for FR2c OBW

| TC | Upper limit (Relaxation due to testability issue) |  |  |
| --- | --- | --- | --- |
| OBW | BW [MHz] | MU [%CBW] | Relaxation for OBW req. [%CBW] |
|  | 50, 100(SC) | 0.8 | 0 |
|  | 200, 400(SC) | 20.5 | 20.5 |
|  | BWagg≤100(CA) | 1.6 | 0 |
|  | BWagg ≤400(CA) | 40.0 | 40.0 |

Proposal 2	: To define FR2c Upper limit of relaxation (due to testability issue) as Table2.2-3 in case relaxation is applied for the OBW definition in test procedure.

Table2.2-3 Proposed upper limits of MU and relaxation (applied for the OBW definition) for FR2c OBW

| TC | Upper limit (Relaxation due to testability issue) |  |  |
| --- | --- | --- | --- |
| OBW | BW [MHz] | MU%[CBW] | Relaxation for OBW definition (%) |
|  | 50, 100(SC) | 0.8 | 0 |
|  | 200, 400(SC) | 2.3 | 1 (98% of Ptotal) |
|  | BWagg≤100(CA) | 1.6 | 0 |
|  | BWagg ≤400(CA) | 3.7 | 3 (96% of Ptotal) |

- Conclusion

Proposal 1	: To define FR2c Upper limit of relaxation (due to testability issue) as Table2.2-2 in case relaxation is applied for the OBW requirement.

Proposal 2	: To define FR2c Upper limit of relaxation (due to testability issue) as Table2.2-3 in case relaxation is applied for the OBW definition in test procedure.

Reference

[1] R5-204851	: “Update of WF on Band n259”, NTT DOCOMO, INC., RAN5#88 (Aug-2020)

[2] R5-201035	: “Test Tolerance of Band n259”, NTT DOCOMO, INC., RAN5#86 (Feb-2020)

[3] R5-203526	: “Update Test Tolerance of Band n259”, NTT DOCOMO, INC., RAN5#88 (Aug-2020)

[4] R5-197734	: “Test tolerance of Band n259”, NTT DOCOMO, INC., RAN5#85 (Nov-2020)

[5] TS 38.521-2		: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone (Release 16) (v16.5.0)”

[6] R5-206788	: “Discussion about Test Tolerance of Band n259”, DOCOMO communications Lab., RAN5#89 (Nov-2020)

[7] R5-210843	: “On FR2 OBW MU”, Anritsu., RAN5#90 (Feb-2021)