3GPP TSG-RAN5 Meeting #91-e	R5-212962

# Online, , 17th May 2021 - 28th Mar 2021

Title:						On FR2 OBW MU

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

In [1], following MU was agreed in RAN5#90e for FR2 OBW.

Table 1 Proposed MU for FR2 OBW

| Frequency range | Single Carrier OBW MU for 50MHz and 100MHz CHBW | Single Carrier OBW MU for 200 MHz and 400MHz BW |
| --- | --- | --- |
| FR2a | ±0.4% | ±1.2% |
| FR2b | ±0.4% | ±1.3% |

# Also, for FR2c, following upper limit of MU and relaxations are agreed in [2].

Table2.2-2 Proposed upper limits of MU and relaxation (applied for the OBW requirement) for FR2c OBW

| TC | Upper limit (Relaxation due to testability issue) |  |  |
| --- | --- | --- | --- |
| OBW | BW [MHz] | MU [%CBW] | Relaxation for OBW req. [%CBW] |
|  | 50, 100(SC) | 0.8 | 0 |
|  | 200, 400(SC) | 20.5 | 20.5 |
|  | BWagg≤100(CA) | 1.6 | 0 |
|  | 100<BWagg ≤400(CA) | 40.0 | 40.0 |

# For the relaxation for FR2 OBW, there were 3 options shown in [1], and the relaxation in [2] is based on Option 1.

Option 1 : Apply the relaxation for the OBW requirement (e.g. 100MHz for 100MHz BW)

Option 2 : Keep the OBW requirement but change the OBW definition (BW containing 99% of total power) in test procedure

Option 3 : Do not test in conformance test

# In this paper we propose remaining MU and relaxations for FR2 OBW test.

# 2.	Discussion

Based on the simulation model in Annex A, which was already assumed in the papers from us, and with the measured quantity of the actual system on the SNRs and flatness, we propose the MU value and relaxation for FR2 OBW test case as shown in Table 1. The assumed peak EIRP value including MBR, MPR and T(MPR) is shown in Table B.

Table 1 shows the proposed MU and relaxation. Relaxation is based on Option 1 as it is already assumed in [2]. Green values are already endorsed value in [1] and are shown for information. Black values are proposals this time.

Table 1: Occupied bandwidth MU

| Freq | BW [MHz] | Proposal |  |
| --- | --- | --- | --- |
|  |  | MU [%CBW] | Relaxation for OBW req. [%CBW] |
| FR2a | 50, 100(SC) | 0.4 | 0 |
|  | 200, 400(SC) | 1.2 (0.8 by original Anritsu proposal) | 0 |
|  | BWagg≤100(CA) | 0.4 | 0 |
|  | BWagg ≤400(CA) | 1.3 | 0 |
| FR2b | 50, 100(SC) | 0.4 | 0 |
|  | 200, 400(SC) | 1.3 | 0 |
|  | BWagg≤100(CA) | 0.8 | 0 |
|  | BWagg ≤400(CA) | 20.0 | 20.0 |

Proposal 2 : Adopt MU and relaxation in Table 1 for FR2 OBW test

# 3.	Conclusion

# RAN5 is asked to endorse following proposals.

Proposal 2 : Adopt MU and relaxation in Table 1 for FR2 OBW test

# 4.	References

# [1] R5-211946, “On FR2 OBW MU”, Anritsu, RAN5#90e

# [2] R5-211951, “Discussion about n259 OBW relaxation”, NTT DOCOMO, RAN5#90e

Annex A : OBW evaluation algorithm

- f, Ptheta, Pphi = Raw data from reference spectrum, N:Number of samples

- P = 10log10( 10^( Ptheta /10) + 10^( Pphi /10))

- M = 1.5*CBW/0.05 # Fixed 50kHz interval resampling.

- f’ = linspace(Fcenter -1.5*CBW/2, Fcenter + 1.5*CBW/2,  M)

- P ‘ = lininterp(f’, f, P) # Make a uniformly sampled data within OBW measurement window by linear interpolation

- Pnoise = Average Power per sample of P’ within TxBW centered on Fcenter – SNR.

- For each realization 1…10000

  - flatness_d = 3 independent random variables of normal distribution

  - flatness_f = linspace(Fcenter -1.5* CBW/2, Fcenter + 1.5*CBW/2, 3)

  - flatness = lininterp(f’, flatness_f, flatness_d) # linear interpolate flatness over OBW measurement window

  - S = 10log10(10^(P’/10) + 10^(Pnoise/10)) + flatness

  - OBWmeas = Frequency width(centered on Fcenter) where the power within it (in W) equals to 0.99 * total power(in W) of S

- Obtain 2.5%-tile and 97.5%-tile value from CDF of 10000 OBWmeas

linspace(x0, x1, N) : N evenly spaced samples, calculated over the interval [x0, x1].

lininterp(x’, x, y) : Linearly interpolated data of (x,y) sampled at x’

Annex B :  Min Peak EIRP with MBR, MPR and T(MPR)

Table B : Min Peak EIRP with MBR, MPR and T(MPR)

|  | BW | Min Peak EIRP | MBR | MPR | T(MPR) | Min Peak EIRP (w/ MBR,MPR) |
| --- | --- | --- | --- | --- | --- | --- |
| FR2a | 100 | 22.4 | 0.75 | 2 | 1.5 | 18.15 |
|  | 400 | 22.4 | 0.75 | 3 | 2.0 | 16.65 |
|  | 100(CA) | 22.4 | 0.75 | 5 | 4.0 | 12.65 |
|  | 400(CA) | 22.4 | 0.75 | 5 | 4.0 | 12.65 |
| FR2b | 100 | 20.6 | 0.75 | 2 | 1.5 | 16.35 |
|  | 400 | 20.6 | 0.75 | 3 | 2.0 | 14.85 |
|  | 100(CA) | 20.6 | 0.75 | 5 | 4.0 | 10.85 |
|  | 400(CA) | 20.6 | 0.75 | 5 | 4.0 | 10.85 |
| FR2c | 100 | 18.7 | 0.75 | 2 | 1.5 | 14.45 |
|  | 400 | 18.7 | 0.75 | 3 | 2.0 | 12.95 |
|  | 100(CA) | 18.7 | 0.75 | 5 | 4.0 | 8.95 |
|  | 400(CA) | 18.7 | 0.75 | 5 | 4.0 | 8.95 |
| NOTE1 : MPR values are those corresponding to full RB allocation with DFT-s-OFDM QPSK. |  |  |  |  |  |  |