3GPP TSG-RAN5 Meeting #91-e	R5-213271

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1951 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FR1 and E-UTRA OTA signal level testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Non-suitable "Off" cell power level can be defined even though calibration procedure for FR1 or E-UTRA OTA is not available. [TS38.133, 9.3.2] An inter-frequency cell shall be considered detectable when for each relevant SSB: - SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band, … As per above, irrespective of NR Operating band in FR1, -121dbm power level will be undetectable level for the UE. In case of uncalibrate link, uncertainity can be around 22dB. Therefore, -156dBm is selected to ensure as it is undetectable power level for Non-suitable "Off" cell in FR1 OTA. Similarly for, -156dBm is selected for E-UTRA OTA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 6.2.2.2-4 and Table 6.2.2.2-7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA Test environment for FR1 or E-UTRA does not include Non-suitable "Off" cell power level |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### 6.2.2.2	Signal Levels for OTA testing

Editor’s Note:	It is FFS whether some or all of the restrictions listed in the working assumptions below will be removed.

The power levels defined in this clause are based on the following assumptions:

-	No more than one E-UTRA cell is configured in the test case and this E-UTRA cell uses an uncalibrated OTA link with no power level change.

-	No more than one NR FR1 cell is configured in the test case and this NR FR1 cell uses an uncalibrated OTA link with no power level change.

-	AWGN is not configured in the test case

For NR FR2 cell, the downlink power settings in Table 6.2.2.2-1 are used unless otherwise specified in a test case.

Table 6.2.2.2-1: Default Downlink power levels for FR2 NR cell (50MHz - 400MHz)

|  | SCS(kHz) | Unit | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |
| Channel BW Power | 60 | dBm | FFS | FFS | FFS | FFS |
|  | 120 | dBm | -57 | -57 | -57 | -57 |
| SS/PBCH SSS EPRE | All | dBm/SCS | -82 | -82 | -82 | -82 |
| Note 1: The channel bandwidth powers are informative, based on -82 dBm/SCS SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. A maximumRE allocation of 24 simultaneously transmitted RBs with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |  |  |  |

The default settings of suitable cells and non-suitable cells for NR FR2 are specified in table 6.2.2.2-2.

NR FR2 cells which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2-2.

Table 6.2.2.2-2: Default settings of suitable / non-suitable FR2 NR cells

| Power level type | NR (Note 1-3) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/SCS | -82 |
| Suitable neighbour intra-frequency cell | dBm/SCS | -91 |
| Suitable neighbour inter-frequency cell | dBm/SCS | -91 |
| Non-suitable cell | dBm/SCS | -100 |
| Non-suitable "Off" cell | dBm/SCS | ≤-139 |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. Note 3: DL level is applied for any of the Subcarrier Spacing configuration (µ) with the same power spectrum density in dBm/SCS (SubCarrier Spacing). |  |  |  |

For E-UTRA cell with FR2 NR, since the LTE OTA link is uncalibrated in the signalling test setup, the table 6.2.2.2-3 provides only suggestive value. It is left to the TE vendor to ensure that LTE cell power level fulfils the cell selection criteria.

Table 6.2.2.2-3: Default Downlink power levels for E-UTRA cells with NR FR2

|  | Unit | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Number of RBs |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Channel BW Power | dBm | -77 | -73 | -71 | -68 | -66 | -65 |
| RS EPRE | dBm/15kHz | -96 | -96 | -96 | -96 | -96 | -96 |
| Note 1: The channel bandwidth powers are informative, based on -96 dBm/15kHz RS_EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |  |  |  |  |

The default settings of suitable cells and non-suitable cells for E-UTRA with FR2 NR are specified in table 6.2.2.2-4.

E-UTRA Cells with FR2 NR which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2-4.

Table 6.2.2.2-4: Default settings of suitable / non-suitable E-UTRA cells with NR FR2

| Power level type | E-UTRAN (Note 1-2) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/15KHz | -96 |
| Suitable neighbour intra-frequency cell | dBm/15KHz | TBD |
| Suitable neighbour inter-frequency cell | dBm/15KHz | TBD |
| Non-suitable cell | dBm/15KHz | TBD |
| Non-suitable "Off" cell | dBm/15KHz | ≤-156 |
| Note 1: The power level is specified in terms ofcell-specific RS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |

The Test system default signal level uncertainty is specified in tables 6.2.2.2-5 and 6.2.2.2-6 for any level specified, unless a tighter uncertainty is specified by a test case in TS 38.523-1 [12].

Table 6.2.2.2-5: SS Absolute FR2 NR signal level uncertainty

|  | Absolute signal level uncertainty |
| --- | --- |
| At each frequency | +/-6 dB at centre of the quiet zone |

Table 6.2.2.2-6: SS Relative FR2 NR signal level uncertainty

|  | Relative signal level uncertainty between any two SS EPRE levels at the same frequency |
| --- | --- |
| At each frequency | +/-2.0 dB |

For NR cell in FR1 with FR2 NR, since the NR FR1 OTA link is uncalibrated in the signalling test setup, -88dBm/SCS should be applied as defined in the table 6.2.2.1-1 and table 6.2.2.1-2. -88dBm/SCS is suggested value and it is left to the TE vendor to ensure that NR cell power level fulfils the cell selection criteria.

The default settings of suitable cells and non-suitable cells for NR in FR1 with FR2 NR are specified in table 6.2.2.2-7.

NR Cells in FR1 with FR2 NR which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2-7.

Table 6.2.2.2-7: Default settings of suitable / non-suitable NR cells in FR1 with NR FR2

| Power level type | NR (Note 1-2) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/SCS | -88 |
| Suitable neighbour intra-frequency cell | dBm/SCS | TBD |
| Suitable neighbour inter-frequency cell | dBm/SCS | TBD |
| Non-suitable cell | dBm/SCS | TBD |
| Non-suitable "Off" cell | dBm/SCS | ≤-156 |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |

<End of Change>