3GPP TSG-RAN5 Meeting#91-e	R5-213826

# Electronic, 17th – 28th May, 2021

Title:							Discussion on FR2 TT calculation in ETC

Source:			Huawei, HiSilicon

Agenda Item:

Document for:			Endorsement

# 1.	Background

MU values for MOP EIRP and EIS were agreed in RAN5#90. In order to complete the test cases, TT needs to be discussed and determined.

# 2.	Discussion

The MU and TT for MOP EIRP and EIS could be summarized as below:

| Test case | MU for NTC | TT for NTC | MU for ETC |
| --- | --- | --- | --- |
| MOP EIRP | PC3: 4.89 for FR2a 5.09 for FR2b | PC3: 2.87dB for FR2a (0.6 x (MTSU-0.1) ) 2.87dB for FR2b (0.6 x (MTSU-0.3) ) | PC3: 5.17 for FR2a 5.37 for FR2b |
| EIS | PC3: 5.19 for FR2a and FR2b | PC3: 2.34dB (0.45 x MTSU) | PC3: 5.45 for FR2a and FR2b |

To facilitate the TT discussion, EIS is taken as an example:

- The PC3 test limit for band n258 400MHz is the minimum requirement + TT = -79.3dBm + 2.34dB = -76.96dBm.

- If the actual EIS of certain DUT is -82.15dBm, the maximum measured EIS would be -82.15dBm + MTSU = -82.15dBm + 5.19dB = -76.96dBm. The DUT could pass the EIS test case.

- When we test the same DUT under ETC, the maximum measured EIS would be -82.15dBm + MTSU = -82.15dBm +5.45dB = -76.7dBm > -76.96dBm. The DUT would be failed due to the larger MTSU in ETC condition.

Option 1: Since the DUT is good enough to pass the test under NTC, it’s reasonable to pass the same DUT under ETC. The test limit needs to be adapted taking the MTSU difference between NTC and ETC into consideration. If the TT under ETC condition is specified as TTETC = TTNTC + (MTSUETC – MTSUNTC), the same DUT could be passed with actual EIS performance unchanged.

It should be highlighted that the MTSU and TT under normal condition is not changed, so there is no risk of bad UE being released into the market.

Option 2: Another alternative option might be using the same calculation method as NTC (option 2). In this case the TT could be calculated as in below table, and a purple line in the figure shows the corresponding test limit. We can see from the figure that UE still would be failed with the TT option 2.

| Test case | MU for NTC | TT for NTC | MU for ETC | TT for ETC option 1 | TT for ETC option 2 |
| --- | --- | --- | --- | --- | --- |
| MOP EIRP | PC3: 4.89 for FR2a 5.09 for FR2b | PC3: 2.87dB for FR2a (0.6 x (MTSU-0.1) ) 2.87dB for FR2b (0.6 x (MTSU-0.3) ) | PC3: 5.17 for FR2a 5.37 for FR2b | PC3: 3.15dB for FR2a (TTNTC + (MTSUETC – MTSUNTC) ) 3.15dB for FR2b (TTNTC + (MTSUETC – MTSUNTC) ) | PC3: 3.04dB for FR2a (0.6 x (MTSUETC -0.1) ) 3.04dB for FR2b (0.6 x (MTSUETC -0.1)) ) |
| EIS | PC3: 5.19 for FR2a and FR2b | PC3: 2.34dB (0.45 x MTSU) | PC3: 5.45 for FR2a and FR2b | PC3: 2.60dB (TTNTC + (MTSUETC – MTSUNTC)) | PC3: 2.45dB (0.45 x MTSUETC) |

Test limit for ETC option 2

option 1

Based on above discussion, the proposal is as below:

Proposal 1: The TT for MOP EIRP and EIS under ETC condition is specified as below table.

| Test case | MU for NTC | TT for NTC | MU for ETC | TT for ETC option 1 |
| --- | --- | --- | --- | --- |
| MOP EIRP | PC3: 4.89 for FR2a 5.09 for FR2b | PC3: 2.87dB for FR2a (0.6 x (MTSU-0.1) ) 2.87dB for FR2b (0.6 x (MTSU-0.3) ) | PC3: 5.17 for FR2a 5.37 for FR2b | PC3: 3.15dB for FR2a (TTNTC + (MTSUETC – MTSUNTC) ) 3.15dB for FR2b (TTNTC + (MTSUETC – MTSUNTC) ) |
| EIS | PC3: 5.19 for FR2a and FR2b | PC3: 2.34dB (0.45 x MTSU) | PC3: 5.45 for FR2a and FR2b | PC3: 2.60dB (TTNTC + (MTSUETC – MTSUNTC)) |

# 3.	Conclusion

Please RAN5 discuss and endorse the proposal:

Proposal 1: RAN5 endorses one of following options:

Option 1: For MOP EIRP and EIS, TTETC = TTNTC + (MTSUETC – MTSUNTC),

Option 1a: For MOP EIRP and EIS, TTETC = TTNTC + (MTSUETC – MTSUNTC). Revisit the calculation method at RAN5#93-e meeting.

Option 2: For MOP EIRP, TTETC = 0.6 x (MTSUETC – 0.1) for FR2a, and TTETC = 0.6 x (MTSUETC – 0.3) for FR2b; For EIS, TTETC = 0.45 x MTSUETC for FR2a and FR2b.

Option 3: Continue the discussion with concluding deadline at RAN5#92-e meeting.

# 4.	References

[1] R5-210733, “Discussion on FR2 TT calculation in special scenarios”, Huawei, HiSilicon, Vivo, RAN5 #81