3GPP TSG-RAN5 Meeting #92-e	R5-215047

# Electronic Meeting, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1096 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting references in EN-DC TX test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 5.2B.2 to 5.2B.7 is voided and should not be referenced in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed references to 5.2B to 5.5.B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will have incorrect references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3.2.4.1, 6.2B.4.2.4, 6.2B.4.2.5, 6.4B.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

6.2B.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.5B.3-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in test configuration table 6.2B.3.1.4.1-1 through 6.2B.3.1.4.1-2 with additional IM3 test frequencies for NS_04 in 6.2B.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A2. Configurations of PDSCH and PDCCH before measurement are specified in Annex TS 36.521-1 [10] Annex C and in Annex C2 for LTE link and NR link respectively.

Table 6.2B.3.2.4.1-0: E-UTRA test configuration table

| E-UTRA Test Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency (Note 1) | Downlink | Uplink |  |
|  |  | N/A for A-MPR testing | Modulation | RB allocation |
| 20 MHz | Low range and High range (Note 2) |  | QPSK | 100 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [6] clause 4.3.1 NOTE 2: NR carrier shall be the outermost carrier during test. |  |  |  |  |

Table 6.2B.3.2.4.1-1: Additional IM3 Test Frquencies for NS_04 intra-band non-contiguous EN-DC

| Additional Initial Condition |  |
| --- | --- |
| Additional IM3 Test Frquencies | if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 15 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-2. if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 30 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-3. if maximum WGAP > 88.4 MHz, and SCS 15 kHz, 60 kHz NR raster then IM3 test frequencies as specified in Table 6.2B.3.2.4.1-4. |

Table 6.2B.3.2.4.1-2: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 15 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2528.86 | 505772 | 0 | 15 | 6327 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 216 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2657.26 | 531452 | 0 | 15 | 6648 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2665.000 | 533000 | 2640.7 | 528140 | 0 | 15 | 6606 | 533000 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 270 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519 | 503800 | 0 | 15 | 6303 | 508660 |

Table 6.2B.3.2.4.1-3: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 30 kHz, 30 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 106 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2529.22 | 505844 | 0 | 30 | 6333 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 106 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2657.62 | 531524 | 0 | 30 | 6654 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 133 | Downlink & Uplink | IM3 | 2665.000 | 533000 | 2641.06 | 528212 | 0 | 30 | 6612 | 533000 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 133 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519.36 | 503872 | 0 | 30 | 6309 | 508660 |

Table 6.2B.3.2.4.1-4: EN-DC combination DC_41A_n41A, intra-band non-contiguous, SCS 60 kHz, 15 kHz NR raster, IM3 test frequencies

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA: 20MHz + NR: 40MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2548.300 | 509660 | 2529.94 | 505988 | 0 | 15 | 6330 | 509660 |
| NR: 40MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2558.300 | 40273 | - | - | - | - | - | - |
|  | NR CC1 | 40 | 51 | Downlink & Uplink | IM3 | 2676.700 | 535340 | 2658.34 | 531668 | 0 | 15 | 6651 | 535340 |
| E-UTRA: 20MHz + NR: 50MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2524.900 | 39939 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2665.000 | 535340 | 2653.3 | 530660 | 0 | 15 | 6639 | 535340 |
| NR: 50MHz + E-UTRA: 20MHz | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | IM3 | 2666.700 | 41357 | - | - | - | - | - | - |
|  | NR CC1 | 50 | 65 | Downlink & Uplink | IM3 | 2543.300 | 508660 | 2519.9 | 503980 | 0 | 15 | 6306 | 508660 |

Editor's note:	The following lines belong at the end of clause 6.2B.3.2.4.1. As new tables are added to this clause, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8].

5.	The UL Reference Measurement channels are set according to TS 36.521-1 [10] Annex A.2 and Annex A for LTE link and NR link respectively.

6.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.2.4.3.

8.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

<< Clauses skipped here>>

##### 6.2B.4.2.4	Inter-band EN-DC including FR2

6.2B.4.2.4.1	ΔTIB,c for EN-DC two bands

Unless otherwise stated, ΔTIB,c for E-UTRA and FR2 NR bands of inter-band EN-DC combinations defined in table 5.5B.5.1-1 is set to zero.

Table 6.2B.4.2.4.1-1: Void

6.2B.4.2.4.2	ΔTIB,c for EN-DC three bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.2-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 6.2B.4.2.4.2-1: Void

6.2B.4.2.4.3	ΔTIB,c for EN-DC four bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.3-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 6.2B.4.2.4.3-1: Void

6.2B.4.2.4.4	ΔTIB,c for EN-DC five bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.4-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 6.2B.4.2.4.4-1: Void

6.2B.4.2.4.5	Void

##### 6.2B.4.2.5	Inter-band EN-DC including both FR1 and FR2

6.2B.4.2.5.1	ΔTIB,c for EN-DC three bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.2-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

Table 6.2B.4.2.5.1-1: Void

6.2B.4.2.5.2	ΔTIB,c for EN-DC four bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.3-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

6.2B.4.2.5.3	ΔTIB,c for EN-DC five bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.4-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

6.2B.4.2.5.4	ΔTIB,c for EN-DC six bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.5-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

<< Clauses skipped here>>

6.4B.1.2.4	Test description

Same test description as in clause 6.4.1.4 in TS 38.521-1 [8] for the NR carrier with the following exception:

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.3, all of these configurations shall be tested with applicable test parameters for each intra-band non-contiguous EN-DC configuration specified in clause 5.3B.1.3, and are shown in table 6.4B.1.2.4-1.

Table 6.4B.1.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  | MinWgap , MaxWgap |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 |  |  | Highest NRB_agg (NOTE3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation |  | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |  |  |  |  |

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1 with the exception that E-UTRA test frequency are specified in Table 7.4B.2.4.1-1 based on intra-band non-contiguous EN-DC configuration specified in clause 5.3B.1.3.

For Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10].

Steps 4 and 6 of Initial conditions as in clause 6.4.1.4.1 in TS 38.521-1 [8] is replaced by:

4.	The DL and UL Reference Measurement frequencies are set according to Table 6.4B.1.2.4-1.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

<< End of changes>>