3GPP TSG-RAN5 Meeting #92-e	R5-215082

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0355 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eMIMO demod test case 5.2.2.1.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New eMIMO demod test case 5.2.2.1.13 is introduced by RAN4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 5.2.2.1.13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is missing in test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.13 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.2.2.1.4_1.3.4	Test requirement

Table 5.2.2.1.4.0-3 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.4_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.4_1.3.4-1: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.1 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | 0.0 |
| 1-2 | R.PDSCH.1-7.2 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | 0.0 |

5.2.2.1.11_1.3.4	Test requirement

Table 5.2.2.1.11.0-3 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.11_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.11_1.3.4-1: Test requirement

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
| 1-1 | R.PDSCH.1-3.2 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 70 | [19.4]+TT |
| 1-2 | R.PDSCH.1-3.2 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 70 | [19.1]+TT |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP |  |  |  |  |  |  |  |

5.2.2.1.13	2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance

5.2.2.1.13.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.1.13.0-3, with the addition of test parameters in Table 5.2.2.1.13.0-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.13.0-1.

Table 5.2.2.1.13.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when UE is configured with “FDMSchemeA” in “RepetitionScheme-r16” defined in clause 5.1 of TS 38.214 [12] | 1-1 |

Table 5.2.2.1.13.0-2: Test parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  |  | Not configured |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  | FDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | Wideband |  |
|  | Resource allocation type |  |  |  | Type 0 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000,1001 | 1000,1001 |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | -0.5 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 200 |  |
| Number of HARQ Processes |  |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | 2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs |  |  |  |  |  |  |

Table 5.2.2.1.13.0-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition(Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.1-2.5 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x2, ULA Low | 70 | 17.3 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.1.13.

5.2.2.1.13_1	2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA

5.2.2.1.13_1.1	Test purpose

To verify the PDSCH performance under 2 receive antenna conditions when UE is configured with “FDMSchemeA” in “RepetitionScheme-r16”.

5.2.2.1.13_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE supportFDM-SchemeA-r16.

5.2.2.1.13_1.3	Test description

5.2.2.1.13_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.7 for TE diagram and section A.3.2.3 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2.2.1.13.0-2 and Table 5.2.2.1.13.0-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On, for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.13_1.3.3.

5.2.2.1.13_1.3.2	Test procedure

1.	SS transmits PDSCH in TRxP#1 and TRxP#2 via PDCCH DCI format 1_1 for C_RNTI (Table 5.2.2.1.13_1.3.3_1-2), to transmit the DL RMC according to Table 5.2.2.1.13_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR for TRxP#1 and TRxP#2 according to Table 5.2.2.1.13_1.4-1.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

5.2.2.1.13_1.3.3	Message contents

5.2.2.1.13_1.3.3_1	Message exceptions for SA

As defined in clause 5.4.2 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.1.13_1.3.3_1-1: PDCCH-ControlResourceSet (Preamble)

| Derivation Path: TS 38.508-1 [4],Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| tci-PresentInDCI | enabled |  |  |
| } |  |  |  |

Table 5.2.2.1.13_1.3.3_1-2: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 4.3.6.1.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Antenna port(s) | DMRS port 0 and 1 | “0111” |  |
| Transmission configuration indication | TCI codepoint 0, corresponding to TCI State #1 and #2 | “000” |  |

Table 5.2.2.1.13_1.3.3_1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 2 entries |  |  |
| TCI-State[1] SEQUENCE { |  | TCI-state-0 |  |
| tci-StateId | 0 |  |  |
| qcl-type1 SEQUENCE { |  |  |  |
| cell | Not present |  |  |
| bwp-Id | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index |  |  |
| } |  |  |  |
| qcl-Type | typeC |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[2] |  | TCI-state-1 |  |
| tci-StateId | 1 |  |  |
| qcl-type1 SEQUENCE { |  |  |  |
| cell | Not present |  |  |
| bwp-Id | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 1 |  |  |
| } |  |  |  |
| qcl-Type | typeA |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[3] |  | TCI-state-2 |  |
| tci-StateId | 2 |  |  |
| qcl-type1 SEQUENCE { |  |  |  |
| cell | Not present |  |  |
| bwp-Id | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 5 |  |  |
| } |  |  |  |
| qcl-Type | typeA |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | wideband |  |  |
| } |  |  |  |
| } |  |  |  |
| repetitionSchemeConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| fdm-TDM-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| repetitionScheme-r16 | fdmSchemeA |  |  |
| startingSymbolOffsetK-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.13_1.3.3_1-4: CSI-RS-ResourceMapping for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0000 | For CSI-RS resources 1, 2, 3, 4 |  |
|  | 0001 | For CSI-RS resources 5,6,7,8 |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 6 | For CSI-RS resources 1,3,5,7 |  |
|  | 10 | For CSI-RS resources 2,4,6,8 |  |
| } |  |  |  |

Table 5.2.2.1.13_1.3.3_1-5: CSI-ResourcePeriodicityAndOffset for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots20 | 10 | For CSI-RS resources 1,2,5,6 |  |
| slots20 | 11 | For CSI-RS resources 3,4,7,8 |  |
| } |  |  |  |

5.2.2.1.13_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.1.13_1.3.3_1.

5.2.2.1.13_1.4	Test requirement

Table 5.2.2.1.13.0-3 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.13_1.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.13_1.4-1: Test requirement for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition(Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.1-2.5 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x2, ULA Low | 70 | 18.3 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

5.2.2.2	TDD

5.2.2.2.1	2Rx TDD FR1 PDSCH mapping Type A performance

5.2.2.2.1.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.1.0-3 and Table 5.2.2.2.1.0-4, with the addition of test parameters in Table 5.2.2.2.1.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.2.2.1.0-1.

<End of Changes >