3GPP TSG-RAN5 Meeting #92-e	R5-215204

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1474 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS LTE A.5.3-Notify for event package-distinguishing audio video conference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In annex A.5.3 (NOTIFY for event package), the message body only considered audio conference case, thus it is not applicable for video conference. Revision is needed to expand the message body to include the video conference in media attribute definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the message body of the NOTIFY: - Changed current condition A3 into audio-only conference. - Added a new conditon A5 for video conference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support for video conference in the message body. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.5.3	NOTIFY for conference event package

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | NOTIFY |  |  |
| Request-URI |  | UE’s contact address in SIP URI form, as provided in the Contact header within the SUBSCRIBE creating the dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  | order of the parameters in this header must be like in this table |  | RFC 3261 [15] |
| via-parm1: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by | A1 | IP address and protected server port of SS |  |  |
| sent-by | A2 | IP address and unprotected server port of SS (optional) |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| via-parm2: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by |  | scscf.3gpp.org |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | sip:final@conf-factory. appended with px_IMS_HomeDomainName |  |  |
| tag |  | tag value corresponding to the SIP URI in the From header |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | any IMPU within the set of IMPUs on ISIM |  |  |
| tag |  | tag value corresponding to the SIP URI in the To header |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same as value received in SUBSCRIBE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |
| method |  | NOTIFY |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec |  | sip:final@conf-factory. appended with px_IMS_HomeDomainName |  |  |
| Event |  |  |  | RFC 6665 [140] RFC 4575 [86] |
| event-type |  | conference |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | 69 |  |  |
| Subscription-State |  |  |  | RFC 6665 [140] |
| substate-value | A3 | active |  |  |
| expires | A3 | 7200 |  |  |
| substate-value | A4 | terminated |  | RFC 4575 [86] |
| reason | A4 | noresource |  | RFC 6665 [140] |
| Content-Type | A3 |  |  | RFC 3261 [15] |
| media-type |  | application/conference-info+xml |  | RFC 4575 [86] |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |
| Message-body | A3 A5 | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity=sip:final@conf-factory. appended with px_IMS_HomeDomainName state="full" version="0"> <users> <user entity="any IMPU within the set of IMPUs on ISIM"> <endpoint entity=" Contact URI of the UE"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label>34567</label> <src-id>SSRC of UE's RTP packets</src-id> <status>sendrecv</status> </media> </endpoint> </user> </user> </conference-info> <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity=sip:final@conf-factory. appended with px_IMS_HomeDomainName state="full" version="0"> <users> <user entity="any IMPU within the set of IMPUs on ISIM"> <endpoint entity=" Contact URI of the UE"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label>34567</label> <src-id>SSRC of UE's RTP packets</src-id> <media id="2"> <type>video</type> <label>45678</label> <src-id>SSRC of UE's RTP packets</src-id> <status>sendrecv</status> </media> </endpoint> </user> </user> </conference-info> |  | RFC 4575 [86] |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 TS 34.229-2 [5])) |
| A2 | GIBA (A.6a/1 TS 34.229-2 [5])) |
| A3 | SS sends NOTIFY to indicate that UE is now subscribed to the audio conference event package |
| A4 A5 | SS sends NOTIFY to indicate that UE’s subscription to conference event is terminated now SS sends NOTIFY to indicate that UE is now subscribed to the video conference event package |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

<End of modified section 1>