3GPP TSG-RAN5 Meeting #92-e	R5-215590

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0021 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MCVideo Test Case 6.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Revised test case 6.1.2.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Edited Test Purposes 1-4; added Test Purposes 4-10. - Edited first paragraph of Conformance Requirements section. For TS 24.281, deleted test cases 9.2.2.2.1.1. and 9.2.1.2.1.2; added test cases 9.2.2.2.1.2 and 9.2.2.2.1.6. For TS 24.581, added 6.2.5.3.2 and 6.2.5.3.3. - In Pre-Test Conditions section, changed subhead UE to IUT. - Edited Main Behaviour Table section - In Specific Message Contents section, edited table numbering. - Edited Note formats. - Edited exception messages for end of LTE connection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The last test specification version would be obsolete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

#### 6.1.2.4	On-network / Chat Group Call / Emergency Call / Imminent Peril Call / Client Terminated (CT)

# 6.1.2.4.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVideo Service }

ensure that {

when { the UE (MCVideo Client) receives a SIP INVITE message from the SS (MCVideo Server) with an emergency indication for a MCVideo On-demand Emergency Chat Group Call }

then { the UE (MCVideo Client) displays an indication for the MCVideo Emergency Chat Group Call to the MCVideo User , and responds to the SS (MCVideo Server) with a SIP 200 (OK) message }

}

(2)

with { the UE (MCVideo Client) having an ongoing MCVideo Emergency Chat Group Call, with implicit Reception Control }

ensure that {

when { the UE (MCVideo Client) receives a Media Transmission Notification message from the SS (MCVideo Server) }

then {the UE (MCVideo Client) )  provides media transmission notification to the MCVideo User and, upon receipt of approval from the user, sends a Receive Media Request message to the SS (MCVideo Server) and respects the Reception Control imposed by the SS (MCVideo Server) (Media Transmission Notification, Receive Media Request, Receive Media Response, Transmission Granted, Media Reception End Request, Media Reception End Response) }

(3)

with { the UE (MCVideo Client) having an ongoing Emergency Chat Group Call, with implicit Reception Control and having sent a Receive Media Request message }

ensure that {

when { the UE (MCVideo Client) receives a Receive Media Response message from the SS (MCVideo Server) }

then {the UE (MCVideo Client) provides receive media success notification to the MCVideo User }

}

(4)

with { the UE (MCVideo Client) having an ongoing  Emergency Chat Group Call, with implicit Reception Control }

ensure that {

when {the UE (MCVideo Client) receives a Media Reception End Request message from the SS (MCVideo Server) }

then {the UE (MCVideo Client) notifies the MCVideo User, and upon receipt of user acceptance, sends a Media Reception End Response message to the SS (MCVideo Server }

(5)

with { the UE (MCVideo Client) having an ongoing Emergency Chat Group Call with Automatic Commencement Mode }

ensure that {

when { the MCVideo User requests to end the Emergency Chat Group Call, }

then { the UE (MCVideo Client) sends a SIP BYE message to the SS (MCVideo Server, receives a SIP 200 (OK) message, and leaves the MCVideo session }

}

(6)

with { UE (MCVideo Client) registered and authorised for MCVideo Service }

ensure that {

when { the UE (MCVideo Client) receives a SIP INVITE message from the SS (MCVideo Server) with an imminent peril indication for a MCVideo On-demand Imminent Peril Chat Group Call }

then { the UE (MCVideo Client) displays an indication for the Pre-arranged MCVideo Imminent Peril Chat Group Call to the MCVideo User, and responds to the SS (MCVideo Server) with a SIP 200 (OK) message }

}

(7)

with { the UE (MCVideo Client) having an ongoing Imminent Peril Chat Group Call, with implicit Reception Control }

ensure that {

when { the UE (MCVideo Client) receives a Media Transmission Notification message from the SS (MCVideo Server) }

then {the UE (MCVideo Client) provides media transmission notification to the MCVideo User and, upon receipt of approval from the user, sends a Receive Media Request message to the SS (MCVideo Server) and respects the Reception Control imposed by the SS (MCVideo Server) (Media Transmission Notification, Receive Media Request, Receive Media Response, Transmission Granted, Media Reception End Request, Media Reception End Response) }

(8)

with { the UE (MCVideo Client) having an ongoing Imminent Peril Chat Group Call, with implicit Reception Control and having sent a Receive Media Request message }

ensure that {

when { the UE (MCVideo Client) receives a Receive Media Response message from the SS (MCVideo Server) }

then {the UE (MCVideo Client) provides receive media success notification to the MCVideo User }

(9)

with { the UE (MCVideo Client) having an ongoing Imminent Peril Chat Group Call, with implicit Reception Control }

ensure that {

when {the UE (MCVideo Client) receives a Media Reception End Request message from the SS (MCVideo Server) }

then {the UE (MCVideo Client) notifies the MCVideo User, and upon receipt of user acceptance, sends a Media Reception End Response message to the SS (MCVideo Server }

(10)

with { the UE (MCVideo Client) having an ongoing Imminent Peril Chat Group Call with Automatic Commencement Mode }

ensure that {

when { the MCVideo User requests to end the Emergency Chat Group Call, }

then { the UE (MCVideo Client) sends a SIP BYE message to the SS (MCVideo Server, receives a SIP 200 (OK) message, and leaves the MCVideo session }

}

# 6.1.2.42	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281, clauses 1.2.1.2, 9.2.2.2.1.2, 9.2.2.2.1.6, 9.2.2.2.2.2, 6.2.6 and TS 24.581, clauses 6.2.5.3.2, 6.2.5.3.3, 6.2.5.4.5. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.281, clause 9.2.1.2.1.2]

In the procedures in this 2.2.1.6]

This procedure is used for MCVideo emergency and MCVideo imminent peril calls when the MCVideo client is affiliated but not joined to the chat group.

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcvideo-info+xml MIME body.

Upon receipt of an initial SIP INVITE request, the MCVideo client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [11] with the clarifications below.

The MCVideo client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCVideo client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCVideo function either with appropriate reject code as specified in 3GPP TS 24.229 [51] and warning texts as specified in subclause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorized to restrict the reason for failure and skip the rest of the steps of this subclause;

NOTE:	If the SIP INVITE request contains an emergency indication or imminent peril indication, the MCVideo client can by means beyond the scope of this specification choose to accept the request.

3)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [11];

4)	if the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP INVITE request for an MCVideo emergency group call and:

i)	should display the MCVideo ID of the originator of the MCVideo emergency group call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body;

ii)	should display the MCVideo group identity of the group with the emergency condition contained in the <mcvideo-calling-group-id> element; and

iii)	if the <alert-ind> element is set to "true", should display to the MCVideo user an indication of the MCVideo emergency alert and associated information;

b)	shall set the MCVideo emergency group state to "MVEG 2: in-progress";

c)	shall set the MCVideo imminent peril group state to "MVIG 1: no-imminent-peril"; and

d)	shall set the MCVideo imminent peril group call state to "MVIGC 1: imminent-peril-gc-capable"; otherwise

5)	if the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP INVITE request for an MCVideo imminent peril group call and:

i)	should display the MCVideo ID of the originator of the MCVideo imminent peril group call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	should display the MCVideo group identity of the group with the imminent peril condition contained in the <mcvideo-calling-group-id> element; and

b)	shall set the MCVideo imminent peril group state to "MVIG 2: in-progress";

6)	may display to the MCVideo user the MCVideo ID of the inviting MCVideo user;

7)	shall perform the automatic commencement procedures specified in subclause 6.2.3.1.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCVideo service setting at the invited MCVideo client for answering the call is set to automatic commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCVideo service setting at the invited MCVideo client for answering the call is set to manual commencement mode, yet the invited MCVideo client allows the call to be answered with automatic commencement mode;

8)	shall perform the manual commencement procedures specified in subclause 6.2.3.2.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCVideo service setting at the invited MCVideo client for answering the call is to use manual commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCVideo service setting at the invited MCVideo client for answering the call is set to automatic commencement mode, yet the invited MCVideo client allows the call to be answered with manual commencement mode; and

9)	when the SIP 200 (OK) response to the SIP INVITE request is sent, may subscribe to the conference event package as specified in subclause 9.1.3.1

[TS 24.281, clause 9.2.2.2.1.2]

This clause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request the MCVideo client:

1)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCVideo user the MCVideo ID of the originator of the MCVideo emergency group call and an indication that this is an MCVideo emergency group call;

b)	if the <mcvideoinfo> element containing the <mcvideo-Params> element contains an <alert-ind> element set to "true", should display to the MCVideo user an indication of the MCVideo emergency alert and associated information;

c)	shall set the MCVideo emergency group state to "MVEG 2: in-progress";

d)	shall set the MCVideo imminent peril group state to "MVIG 1: no-imminent-peril"; and

e)	shall set the MCVideo imminent peril group call state to "MIGC 1: imminent-peril-gc-capable";

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCVideo user the MCVideo ID of the originator of the MCVideo imminent peril group call and an indication that this is an MCVideo imminent peril group call; and

b)	shall set the MCVideo imminent peril group state to "MVIG 2: in-progress";

3)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "false":

a)	should display to the MCVideo user the MCVideo ID of the MCVideo user cancelling the MCVideo emergency group call;

b)	if the <mcvideoinfo> element containing the <mcvideo-Params> element contains an <alert-ind> element set to "false":

i)	should display to the MCVideo user an indication of the MCVideo emergency alert cancellation and the MCVideo ID of the MCVideo user cancelling the MCVideo emergency alert; and

ii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body including an <originated-by> element:

A)	should display to the MCVideo user the MCVideo ID contained in the <originated-by> element of the MCVideo user that originated the MCVideo emergency alert; and

B)	if the MCVideo ID contained in the <originated-by> element is the MCVideo ID of the receiving MCVideo user, shall set the MCVideo emergency alert state to "MVEA 1: no-alert";

c)	shall set the MCVideo emergency group state to "MVEG 1: no-emergency"; and

d)	if the MCVideo emergency group call state of the group is set to "MVEGC 3: emergency-call-granted", shall set the MCVideo emergency group call state of the group to "MVEGC 1: emergency-gc-capable";

4)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <imminentperil-ind> element set to a value of "false":

a)	should display to the MCVideo user the MCVideo ID of the MCVideo user cancelling the MCVideo imminent peril group call and an indication that this is an MCVideo imminent peril group call;

b)	shall set the MCVideo imminent peril group state to "MVIG 1: no-imminent-peril"; and

c)	shall set the MCVideo imminent peril group call state to "MIGC 1: imminent-peril-gc-capable";

5)	may check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [11];

6)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [11];

7)	shall include the g.3gpp.mcvideo media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" in the Contact header field of the SIP 200 (OK) response;

9)	if the SIP re-INVITE request was received within an on-demand session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [11] with the clarifications given in clause 6.2.2; and

10)	shall send the SIP 200 (OK) response towards the MCVideo server according to rules and procedures of 3GPP TS 24.229 [11].

[TS 24.281, clause 9.2.2.2.1.6]

This procedure is used for MCVideo emergency and MCVideo imminent peril calls when the MCVideo client is affiliated but not joined to the chat group.

In the procedures in this clause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcvideo-info+xml MIME body.

Upon receipt of an initial SIP INVITE request, the MCVideo client:

1)	may reject the SIP INVITE request if either of the following conditions is met:

a)	MCVideo client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCVideo function either with appropriate reject code as specified in 3GPP TS 24.229 [11] and warning texts as specified in clause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this clause;

NOTE 1:	if the SIP INVITE request contains an emergency indication or imminent peril indication, the MCVideo client can by means beyond the scope of this specification choose to accept the request.

3)	if the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP INVITE request for an MCVideo emergency group call and:

i)	should display the MCVideo ID of the originator of the MCVideo emergency group call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body;

ii)	should display the MCVideo group identity of the group with the emergency condition contained in the <mcvideo-calling-group-id> element; and

iii)	if the <alert-ind> element is set to "true", should display to the MCVideo user an indication of the MCVideo emergency alert and associated information;

b)	shall set the MCVideo emergency group state to "MVEG 2: in-progress";

c)	shall set the MCVideo imminent peril group state to "MVIG 1: no-imminent-peril"; and

d)	shall set the MCVideo imminent peril group call state to "MVIGC 1: imminent-peril-gc-capable"; otherwise

4)	if the SIP INVITE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCVideo user an indication that this is a SIP INVITE request for an MCVideo imminent peril group call and:

i)	should display the MCVideo ID of the originator of the MCVideo imminent peril group call contained in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body; and

ii)	should display the MCVideo group identity of the group with the imminent peril condition contained in the <mcvideo-calling-group-id> element; and

b)	shall set the MCVideo imminent peril group state to "MVIG 3: in-progress";

5)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [11];

6)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [11];

7)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.mcvideo media feature tag in the Contact header field of the SIP 200 (OK) response;

9)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" in the Contact header field of the SIP 200 (OK) response;

10)		shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [23]. If no "refresher" parameter was included in the received SIP INVITE request the "refresher" parameter in the Session-Expires header field shall be set to "uas", otherwise shall include a "refresher" parameter set to the value received in the Session-Expires header field the received SIP INVITE request;

11)		shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [11] with the clarifications given in clause 6.2.2;

12)	shall send the SIP 200 (OK) response towards the MCVideo server according to rules and procedures of 3GPP TS 24.229 [11]; and

13)	shall interact with the media plane as specified in 3GPP TS 24.581 [5].

[TS 24.281, clause 9.2.2.2.2.2]

Upon receiving a SIP BYE request for releasing the MCVideo chat session, the MCVideo client shall follow the procedures as specified in subclause 6.2.6.

[TS 24.281, clause 6.2.6]

Upon receiving a SIP BYE request, the MCVideo client:

1)	shall interact with the media plane as specified in 3GPP TS 24.581[5]; and

2)	shall send SIP 200 (OK) response towards MCVideo server according to 3GPP TS 24.229 [11].

[TS 24.581, clause 6.2.5.3.2]

Upon receiving the media transmission notification from the transmission control server, the transmission participant:

1.	if the first bit in the subtype of the media transmission notification message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '6' (Media transmission notification); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall provide media transmission notification to the user;

3.	shall store the User ID and the SSRC of the user transmitting the media;

4.	may display the details of the incoming media to the user; and

5.	shall remain in the 'U: reception controller' state.

[TS 24.581, clause 6.2.5.3.3]

Upon receiving an indication from the user to request permission to receive media, the transmission participant:

1.	shall send the Receive Media Request message toward the transmission control server; The Receive Media Request message:

a.	if a different priority than the normal priority is required, shall include the Reception Priority field with the priority not higher than negotiated with the transmission control server as specified in subclause 14.3.3; and

b.	if the receive media request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Transmission Indicator field indicating the relevant call types;

2.	shall create an instance of the 'Transmission participant state transition diagram for basic reception control operation';

3.	shall map the stored User ID and the SSRC of the user transmitting the media with instance of 'Transmission participant state transition diagram for basic reception control operation' created in step 2; and

4.	shall remain in the 'U: reception controller' state.

[TS 24.581, clause 6.2.5.4.5]

Upon receiving a granted response for Receive media request message, the transmission participant:

1.	if the first bit in the subtype of the Receive media response message is set to '1' (Acknowledgment is required) as described in subclause 9.2.2.1, shall send a Transmission control Ack message. The Transmission control Ack message:

a.	shall include the Message Type field set to '7' (Receive media response); and

b.	shall include the Source field set to '0' (the transmission participant is the source);

2.	shall provide receive media success notification to the user;

3.	if the Receive Media Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	shall stop timer T103 (Receive Media Request); and

5.	shall enter the 'U: has permission to receive' state.

# 6.1.2.4.3	Test description

# 6.1.2.4.3.1	Pre-test conditions

System Simulator:

-	SS (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2A.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCVideo Client Application has been activated and User has registered-in as the MCVideo User with the Server as active user at the Client.

6.1.2.4.3.2	Test procedure sequence

Table 6.1.2.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The E-UTRA/EPC actions that are related to the MCVideo call establishment are described in TS 36.579-1 [2], subclause 5.4.4 Generic Test Procedure for MCVideo CT communication in E-UTRA '. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 1 | The SS (MCVideo Server) sends an SIP INVITE to initiate an on-demand emergency chat group call with automatic commencement mode. | <-- | SIP INVITE | - | - |
|  | EXCEPTION: Steps 2a1 describes optional behaviour that depends on the UE (MCVideo Client)implementation; the "lower case letter" identifies a step sequence that take place if the UE (MCVideo Client) responds to a SIP INVITE with a SIP 100 (Trying) message. |  |  |  |  |
| 2a1 | The UE (MCVideo Client) sends a SIP 100 (Trying) message to the SS (MCVideo Server). | --> | SIP 100 (Trying) |  |  |
| 3 | Check: Does the UE (MCVideo Client) respond with a SIP 200 (OK)? | --> | SIP 200 (OK) | 1 | P |
| 4 | The SS (MCVideo Server) sends a SIP ACK to the UE (McVideo Client). | <-- | SIP ACK | - | - |
| 5 | Check: Does the UE (MCVideo client) notify the user that the Emergency Chat Group Call has been successfully established? (NOTE 1) | - | - | 1 | P |
| 6 | VOID |  |  |  |  |
| 6A | The SS (MCVideo Server) send a Media Transmission Notification message. | <-- | Media Transmission Notification |  |  |
| 6B | Check: Does the UE (MCVideo Client) notify the MCVideo User of the Media Transmission Notification? | - | - | 2 | P |
| 6C | Make the MCVideo User accept the transmission indicated by the Media Transmission Notification message from the SS (MCVideo Server) (NOTE 1) | - | - | - | - |
| 6D | Check: Does the UE (MCVideo Client) respond with a Receive Media Request message? | --> | Receive Media Request | 2 | P |
| 6E | The SS (MCVideo Server) sends a Receive Media Response message | <-- | Receive Media Response | 3 | P |
| 6F | Check: Does the UE (MCVideo Client) provide receive media success notification to the MCVideo User? | - | - | 3 | P |
| 6G | The SS (MCVideo Server) requests to terminate the RTP media reception by sending a Media Reception End Request message. | <-- | Media Reception End Request | - | - |
| 6H | Check: Does the UE (MCVideo Client) notify the MCVideo User of receipt of the Media Reception End Request? (NOTE 1) | - | - | 4 | P |
| 6I | Check: Does the UE (MCVideo Client) send a Media Reception End Response to the SS (MCVideo Server)? | --> | Media Reception End Response | 4 | P |
| 7 | Make the MCVideo User request the UE (MCVideo Client) to end the Emergency Chat Group Call. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCVideo Client) send a SIP BYE message to end the on-demand group call. | --> | SIP BYE | 5 | P |
| 9 | The SS (MCVideo Server) sends a SIP 200 (OK) to the UE (MCVideo Client). | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: The SS (MCVideo Server) waits 2 seconds before the SS (MCVideo Server) deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions that are related to the MCVideo call establishment are described in TS 36.579-1 [2], subclause 5.4.4 Generic Test Procedure for MCVideo CT communication in E-UTRA '. The test sequence below shows only the MCVideo relevant messages exchanged. | - | - | - | - |
| 10 | The SS (MCVideo Server) sends a SIP INVITE to initiate an Imminent Peril Chat Group Call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Steps 20a1 describes optional behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the SS responds to a SIP INVITE with a SIP 100 (Trying) message. | - | - | - | - |
| 11a1 | The UE (MCVideo Client) sends a SIP 100 (Trying) message to the SS (MCVideo Server). | --> | SIP 100 (Trying) |  |  |
| 12 | Check: Does the UE (MCVideo Client) send a SIP 200 (OK) message to the SS (MCVideo Server)? | --> | SIP 200 (OK) | 6 | P |
| 13 | The SS (MCVideo Server) sends a SIP ACK to the UE (MCVideo Client). | <-- | SIP ACK | - | - |
| 14 | Check: Does the UE (MCVideo client) notify the user that the Imminent Peril Group Call has been successfully established? (NOTE 1) | - | - | 6 | P |
| 14A | The SS (MCVideo Server) sends a Media Transmission Notification message. | <-- | Media Transmission Notification | - | - |
| 14B | Check: Does the UE (MCVideo Client) notifiy the MCVideo User of the Media Transmission Notification? (NOTE 1) | - | - | 7 | P |
| 14C | Make the MCVideo User accept the transmission indicated by the Media Transmission Notification message from the SS (MCVideo Server) (NOTE 1) | - | - | - | - |
| 14D | Check: Does the UE (MCVideo Client) respond with a Media Receive Request message? | --> | Receive Media Request | 7 | P |
| 14E | The SS (MCVideo Server) sends a Receive Media Response message. | <-- | Receive Media Response | - | - |
| 14F | Check: Does the UE (MCVideo Client) provide receive media success notification to the MCVideo User? (NOTE 1) | - | - | 8 | P |
| 14G | The SS (MCVideo Server) requests to terminate the RTP media reception by sending a Media Reception End Request message? | <-- | Media Reception End Request | - | - |
| 14H | Check: Does the UE (MCVideo Client) notify the MCVideo User of receipt of the Media Reception End Request? | - | - | 9 | P |
| 14I | Check: Does the UE (MCVideo Client) send a Media Reception End Response to the SS (MCVideo Server)? | --> | Media Reception End Response | 4 | P |
| 15 | VOID |  |  |  |  |
| 16 | Make the MCVideo User request to end an Imminent Peril Chat Group Call. (NOTE 1) | - | - | - | - |
| 17 | Check: Does the UE (MCVideo Client) send a SIP BYE (to terminate the media plane)? | --> | SIP BYE | 10 | P |
| 18 | The SS (MCVideo Server) sends SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: The SS (MCVideo Server) waits 2 seconds before the SS (MCVideo Server) deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2): | - | - | - | - |
| NOTE 1: This action is expected to be done via a suitable implementation-dependent MMI. |  |  |  |  |  |
| NOTE 2: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. |  |  |  |  |  |

6.1.2.4.3.3	Specific message contents

Table 6.1.2.4.3.3-1: SIP INVITE (Step 1, Table 6.1.2.4.3.2-1)

| Derivation Path: TS 36.579 [2], Table 5.5.2.5.2-1, condition MCVIDEO, EMERGENCY-CALL, GROUP-CALL, EMERGENCY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcvideo-info+xml" as described in Table 6.1.2.4.3.3-2 |  |  |  |

Table 6.1.2.4.3.3-2: MCVideo-INFO in SIP INVITE (Table 6.1.2.4.3.3-1)

| Derivation Path: TS 36.579-1, Table 5.5.3.2.2-2, condition MCVIDEO, GROUP-CALL, EMERGENCY-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
| emergency-ind | “true” |  |  |  |

Table 6.1.2.4.3.3-3: SIP 200 (OK) (Step 3, Table 6.1.2.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition MCVIDEO, GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcvideo-info+xml" as described in Table 6.1.2.4.3.3-4 |  |  |  |

Table 6.1.2.4.3.3-4: MCVideo-INFO (Table 6.1.2.4.3.3-3)VOID

Table: 6.1.2.4.3.3-5: Transmission Idle (Step 6, Table 6.1.2.4.3.2-1)VOID

Table 6.1.2.4.3.3-6: SIP BYE (Steps 8, 17, Table 6.1.2.4.3.2-1)

Derivation Path: TS 36.579-1, Table 5.5.2.2-1, condition MO_CALL

Table 6.1.2.4.3.3-7: SIP 200 (OK) (Steps 9, 18, Table 6.1.2.4.3.2-1)VOID

Table 6.1.2.4.3.3-8: SIP INVITE (Step 10, Table 6.1.2.4.3.2-1)

| Derivation Path: TS 36.579 [2], Table 5.5.2.17.1.2-1, condition MCVIDEO, GROUP-CALL, IMMPERIL-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.1.2.4.3.3-9 |  |  |  |

Table 6.1.2.4.3.3-9: MCVideo-INFO in SIP INVITE (Table 6.1.2.4.3.3-8)

| Derivation Path: TS 36.579-1, Table 5.5.3.2.2-2, condition MCVIDEO, GROUP-CALL, IMMPERIL-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
| session-type | "chat" |  |  |  |
| imminentperil-ind | “true” |  |  |  |

Table 6.1.2.4.3.3-10: SIP 200 (OK) (Step 12, Table 6.1.2.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition MCVIDEO, GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcvideo-info+xml" as described in Table 6.1.2.4.3.3-11 |  |  |  |

Table 6.1.2.4.3.3-11: MCVideo-INFO (Table 6.1.2.4.3.3-10)VOID

Table: 6.1.2.4.3.3-12: Transmission Idle (Step 15, Table 6.1.2.4.3.2-1)VOID

<END OF CHANGES>