3GPP TSG-RAN5 Meeting #92-e	R5-215654

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1450 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to RRC based BWP switch SA FR1 TC 6.5.6.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 38.508-1 Table 4.6.3-167 specifies initialDownlinkBWP to have a dedicated configuration. According to 38.331, this is considered as an RRC-configured BWP: initialDownlinkBWP The dedicated (UE-specific) configuration for the initial downlink bandwidth-part (i.e. DL BWP#0). If any of the optional IEs are configured within this IE, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. By default the UE is required to support a minimum of 1 BWP, however due to the initialDownlinkBWP dedicated configuration, the UE is required to support at least 2 BWPs in the current test description while the actual RAN4 requirement only needs 1 BWP. This causes this test, in its current form, to be not applicable to UE’s supporting only 1 BWP. To add, RRC-based BWP is not an optional feature, so no explicit applicability requirement is needed when only 1 BWP switch is configured and RRC is used to modify that BWP, as is the case for this test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the applicability, test procedure and message contents so at any given time there are not more than 2 RRC configured BWPs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

##### 6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX

6.5.6.2.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in TS 38.133 [6] clause 8.6.

6.5.6.2.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.5.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.2.1.

6.5.6.2.1.4	Test description

6.5.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.6.2.1.4.1-1.

Table 6.5.6.2.1.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Config | Description |
| --- | --- |
| 6.5.6.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.6.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.6.2.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.2.1.4.1-2.

Table 6.5.6.2.1.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.2.1.4.1-3.

2. Message contents are defined in clause 6.5.6.2.1.4.3.

3. The test scenario comprises of one NR Cell (Cell 1). Cell 1 is configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.2.1.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

6.5.6.2.1.4.2	Test procedure

The test consists of 1 time period, with duration of T1.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in Cell 1.

Cell 1 has constant signal level throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 6.5.6.2.1.4.1-3 and 6.5.6.2.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall send an RRCReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP. This message also configures another UE-specific bandwidth part, BWP-1 and indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id, according to the initial condition of Active BWP-1 in Table 6.5.6.2.1.5-1.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	The SS shall send an RRCReconfiguration message with updated bandwidth part configuration for DL BWP switch, change the BWP according to the final condition of Active BWP-1 in Table 6.5.6.2.1.5-1. T1 starts.

6.	The UE shall receive the RRCReconfiguration in PCell’s slot # denoted i and reconfigure its bandwidth part with the updated bandwidth part configuration.

7	If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot  i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After the SS receives the ACK/NACK in step 7) or when T1 expires, the SS shall transmit RRCRelease message to release the RRC connection.

9.	After the RRC connection release, the SS:

-transmits in NR Cell 1 a Paging message for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time in PCell by counting the slots from the time when the RRC Reconfiguration message including updated BWP configuration is sent till a valid ACK/NACK is received.

6.5.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.2.1.4.3-1: Common Exception messages for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.2.1.4.3-2: BWP

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set the value according to the configuration of active BWP-1 |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| cyclicPrefix | Not present |  |  |
| } |  |  |  |

Table 6.5.6.2.1.4.3-3: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| ServingCellConfig ::= SEQUENCE { |  |  |  |  |
| initialDownlinkBWP ::= SEQUENCE { |  |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |  |
| BWP-Downlink[1] | Downlink BWP-1 |  |  |  |
| } |  |  |  |  |
| firstActiveDownlinkBWP-Id | 1 |  |  |
| defaultDownlinkBWP-Id | 1 |  |  |
| uplinkConfig SEQUENCE { |  |  |  |  |
| initialUplinkBWP ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| srs-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |  |
| BWP-Uplink[1] | Uplink BWP-1 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.5.6.2.1.5	Test requirements

Tables 6.5.6.2.1.4.1-3 and 6.5.6.2.1.5-1 define the primary level settings including test tolerances.

Table 6.5.6.2.1.5-1: NR Cell specific test parameters for DL BWP switch

| Parameter |  |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  |  | FDD |
|  |  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  |  | Config 1 |  |  | Not Applicable |
|  |  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  |  | 1 |
| Initial DL BWP Configuration |  |  | Config 1,2,3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  |  | Config 1,2,3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration |  | Config 1,2,3 |  |  | DLBWP.1.3 |
|  | Active UL BWP-1 Configuration |  | Config 1,2,3 |  |  | ULBWP.1.3 |
| Final Condition | Active DL BWP-1 Configuration |  | Config 1,2,3 |  |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration |  | Config 1,2,3 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  | SR.1.1 FDD |
|  |  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  |  | Config 3 |  |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1 |  |  | CR.1.1 FDD |
|  |  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  |  | Config 3 |  |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1 |  |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  |  | CCR.2.3 TDD |
| OCNG Patterns |  |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  |  | dB | 17 |
| Ês/Noc |  |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant for Cell from the first DL slot that occurs right after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot  i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Changes >>