3GPP TSG-RAN WG5 Meeting #92-e	R5-215991

Electronic Meeting, August 16 – 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1303 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Tx test cases for PC2 CA_n28A-n79A with UL PC2 n79A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Tx test cases for PC2 CA_n28A-n79A with UL PC2 n79A needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tx test cases for PC2 CA_n28A-n79A with UL PC2 n79A has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.0.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2A.1.0.3	UE maximum output power for Inter-band CA

<<< UNCHAGED SECTIONS SKIPPED >>>

Table 6.2A.1.0.3-2 UE Power Class except class 3 for downlink inter-band CA (two bands DL/ 1 band UL)

| Downlink CA Configuration | Band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n28A-n79A | n79 |  |  | 264 | +2/-31 |
| NOTE 1: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: Power class 3 is the default power class unless otherwise stated. NOTE 4: The power class is supported by UE within NR TDD band. |  |  |  |  |  |

<< END OF CHANGES >>