3GPP TSG-RAN WG5 #92-e		R5-216083
Electronic Meeting, 16th – 27th August 2021

Agenda item:	5.3.2.17

Source:	Qualcomm Austria RFFE GmbH

Title:	Discussion of test applicability for different NS values

Document for:	Endorsement

# 1.	Introduction

UE additional maximum output power reduction (A_MPR) and additional spurious emissions for different NS values were introduced in different releases of TS 38.101-1 [1, 2]

The test applicability for different NS values for UE additional maximum output power reduction (A_MPR) and additional spurious emissions in TS 38.521-1 are not clearly noted and misaligned with core requirement.  In both cases, it is not clear which NS value applies to UE release 15 and forward, or UE release 16 and forward. This leads to different interpretations by different UE vendors.

In this discussion paper, the current discrepancies between core requirement and latest TS 38.521 are discussed and a way forward is proposed.

# 2. 	Discussion

## 2.1 UE additional maximum output power reduction (A_MPR) core requirement

From TS 38.101-1 V15.4.0 [1] and TS 38.101-1 V16.8.0 [2] clause 6.2.3, minimum requirement for different NS values is defined.  A-MPR for some NS values are only specified in release 16 core requirement.  NS values defined by both Release 15, Release 16, and Release 17 are listed in Table 2.1-1.  Additional NS values specified by Release 16 and Release 17 are listed in Table 2.1-2.  A “Yes” indicates NS value is available for that release listed. A “No” indicates NS value is not defined for that release listed.

Table 2.1-1 NS-values defined by Release 15, 16 and 17

| NS_Value | 4 | 10 | 05, 05U | 40 | 43, 43U | 03, 03U | 37 | 38 | 39 | 41 | 42 | 18 | 50 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR band | n41 | n20 | n1, n65, n84 | n51 | n8, n81 | n2, n25, n66, n86 | n74 | n74 | n74 | n50 | n50 | n28, n83 | n39 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 |
| Rel 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Rel 16 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Rel 17 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

Table 2.1-2 NS-values defined by Release 16 and 17 only

| NS_Value | 21 | 24 | 27 | 46 | 47 | 44 | 12 | 13 | 14 | 15 | 45 | 48 | 49 | 51 | 07 | 56 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR band | n30 | n65 | n48 | n7 | n41 | n38 | n26 | n26 | n26 | n26 | n53 | n1 | n1 | n65 | n13 | n24, n99 |
| Rel 15 | No | No | No | No | No | No | No | No | No | No | No | No | No | No | No | No |
| Rel 16 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | No | No |
| Rel 17 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

Clearly, all NS values listed in Table 2.1-1 shall apply to all types of NR UE release 15 and forward.

NR band highlighted in Table 2.1-2 are Release 15 bands, but their NS values (A-MPR) are only available and defined in Rel-16 of TS 38101-1 [2].   Therefore, these NS values shall only apply to NR UE release 16 and forward.

Further more, these new A-MPR requirement (for Rel-15 band) were introduced in Rel-16 apply to specific channel bandwidths which are also introduced in Rel-16.

In summary:

- Since NR bands are release agnostic, if the Rel-15 UE support a Rel-16 band then the Rel-16 NS-value also need to be supported.

- For NS values for Rel-15 NR bands n7, n41, n38 and n1, the NS values were only added in Rel-16 of 38.101-1 spec.  Rel-15 UE doesn’t have to support these new NS values.

- If a Rel-15 UE supports channel bandwidths introduced in Rel-16, it shall follow the Rel-16 requirements for the channel bandwidths supported.

## 2.2 UE additional maximum output power reduction in TS 38.521-1

In current TS 38.521-1 [3] clause 6.2.3.2 Test applicability, the following statement applies for tet 6.5.3.3

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49 and NS_50 to all types of NR Power Class 2 and 3 UE release 15 and forward.

From the comparison table above, A-MPR for NS_46, NS_47, NS_44, NS_48, and NS_49 are only defined by Release 16 core requirement even the corresponding bands are release 15 bands.  Therefore, for a Rel-15 complied UE, these NS values don’t apply.

Clearly, the test applicability for test case 6.5.3.3 Additional Spurious Emissions is not fully aligned with core requirement, and this leads different implementations by different UE vendors.

In order to avoid this ambiguity, the following changes are proposed based on email discussion

Proposal 1a: Add an Editor’s note to A-MPR test case

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

Proposal 1b: Added an Action Point to address applicability spec changes.

## 2.3 UE Additional spurious emissions in TS 38.521-1

Similarly, the Test applicability for Additional spurious emissions in current TS 38.521-1 [3] clause 6.5.3.3 needs to be revised in the same manner.

Proposal 2: Add an Editor’s note to Additional spurious emissions test

The requirements of this test case for NS_44, NS_46, NS_47, NS_48, and NS_49 apply to all types of NR UE release 16 forward, and release 15 if the corresponding channel bandwidths are supported.

# 3. 	Conclusion

Based on the discussion, the following modification of test applicability is proposed.

Proposal 1a: Add an Editor’s note to A-MPR test case

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidth are supported.

Proposal 1b: Added an Action Point to address applicability spec changes.

Proposal 2: Add an Editor’s note to Additional spurious emissions test

Test applicability exceptions:

NS_44, NS_46, NS_47, NS_48, and NS_49 apply to all types of NR UE release 16 forward, and release 15 if the corresponding channel bandwidth are supported.

# 4. 	References

- 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1, Standalone", Revision  V15.14.0 (2021-06)

- 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1, Standalone", Revision  V16.8.0 (2021-06)

- 3GPP TS 38.521-1:" User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone.