3GPP TSG-RAN5 Meeting #92-e	R5-216201

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2374 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 11.4.5-Handling of 5GS forbidden tracking areas for roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In step 8, the UE is in Limited Service state, therefore, the Emergency call without IMS registration shall be called, instead of the procedure with IMS registration. - Some RRC message is wrong. - When UE is in 5GMM-REGISTERED.LIMITED-SERVICE state and initiate an emergency call, the UE will perform emergency registration, and then after the emergency call is released, it’s likely that the UE will either stay in 5GMM-REGISTERED state or enter 5GMM-DEREGISTERED state. If the UE tries to enter 5GMM-DEREGISTERED, it’s possible that the UE will send a DEREGISTRATION REQ or perform locally release. But current steps couldn’t cover the condition when the UE perform locally deregistration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed step 8 to Emergency call without IMS registration. - Editorial corrections of RRC msg. - Change the steps between step 11~13 to firstly accomdate possible UE behaviour to send DeRegisterReq msg, then Release RRC connection and let the UE trigger a normal IMS call after the Emergency call released - Change Step 13 to let Cell-11 as serving cell and Cell-2 as suitable cell, so that can verify TP2 better, i.e although Cell-11 has better signal strength, UE still reselect to Cell-2 because Cell-11 is forbid cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-214789r1. |  |  |  |  |  |  |  |  |

<Start of modified section >

11.4.5	5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming

11.4.5.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED.LIMITED-SERVICE state and 5GMM-IDLE mode }

ensure that {

when { UE is requested to make an Emergency call }

then { UE establishes the Emergency call }

}

(2)

with { UE in 5GMM-REGISTERED.LIMITED-SERVICE state and 5GMM-IDLE mode having established an Emergency call }

ensure that {

when { UE is requested to release the Emergency call }

then { UE releases the Emergency call, and, the UE considers the current cell as belonging to 5GS forbidden tracking areas for roaming }

}

11.4.5.2	Conformance requirement

References: The conformance requirements covered in the present TC are specified in: TS 24.501 [22], subclause 5.5.1.3.5, TS 23.122 [38], subclause 3.4.2. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, subclause 5.5.1.3.5]

#15	(No suitable cells in tracking area).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2). The UE shall reset the registration attempt counter and shall enter the state 5GMM-REGISTERED.LIMITED-SERVICE. The UE shall search for a suitable cell in another tracking area according to 3GPP TS 38.304 [28].

The UE shall store the current TAI in the list of "5GS forbidden tracking areas for roaming" and shall remove the current TAI from the stored TAI list, if present.

If the UE is operating in single-registration mode, the UE shall handle the EMM parameters EMM state, EPS update status and tracking area updating attempt counter as specified in 3GPP TS 24.301 [15] for the case when the normal tracking area updating procedure is rejected with the EMM cause with the same value.

[TS 23.122, subclause 3.4.2]

The MS is not allowed to request 5GS services except emergency services when camped on a cell of a TA of which belongs to the list of "5GS forbidden tracking areas for regional provision of service".

11.4.5.3	Test description

11.4.5.3.1	Pre-test conditions

System Simulator:

-	3 NR Cells

-	NR Cell 1, NR Cell 2 and NR Cell 11 as defined in TS 38.508-1 [4], Table 4.4.2-3.

-	Maximum of 2 cells are active at any point of time.

-	On all cells when active: System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

None.

Preamble:

-	Cell configuration " in accordance with TS 38.508-1 [4], Table 4.4.2-3:

-	NR Cell 1 "Serving cell"

-	NR Cell 2 "Non-Suitable "Off" cell"

-	NR Cell 11 "Non-Suitable "Off" cell"

-	The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1

-	During the initial registration:

-	In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1.

11.4.5.3.2	Test procedure sequence

Table 11.4.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NR Cell 11 as "Serving cell" - NR Cell 1 as "Non-Suitable "Off" cell" - NR Cell 2 as "Non-Suitable "Off" cell". | - | - | - | - |
| - | EXCEPTION: The following messages are to be observed on NR Cell 11 unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5 | SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area). | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION REJECT | - | - |
| 6 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 7 | Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? | - | - | 1 | - |
| 9 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 10 | The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. | - | - | - | - |
| 11 | Void. | - | - | - | - |
| 12 | Start Timer T1=10 seconds. NOTE: This is an arbitrary value to wait for UE to initiate deregistration. | - | - | - | - |
| - | EXCEPTION: Steps 12a1-12b1 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 12a1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to “Normal de-registration”. | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 12a2 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |
| 12a3 | Stop Timer T1=10 seconds. | - | - | - | - |
| 12b1 | Timer T1=10 seconds expires | - | - | - | - |
| 12A | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 12B | Make the UE attempt an IMS non-emergency call. (NOTE 2) | - | - | - | - |
| 12C | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure on NR Cell 11 within the next 10 seconds? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 13 | The SS configures: - NR Cell 2 as "Suitable cell" - NR Cell 11 as "Serving cell" - NR Cell 1 as "Non-Suitable "Off" cell". | - | - | - | - |
| 14 | Check: Does the UE perform on NR Cell 2 the Registration procedure for initial registration as specified in TS 38.508-1 [4] subclause 4.5.2.2-2, 'connected without release'? | - | - | 2 | P |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

11.4.5.3.3	Specific message contents

Table 11.4.5.3.3-1: REGISTRATION REJECT (step 5, Table 11.4.5.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘00001111’B | #15 (No suitable cells in tracking area). |  |

<End of modified section >