3GPP TSG-RAN5 Meeting #92-e	R5-214406r1

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1965 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete CSI-ReportConfig for RRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CSI-ReportConfig in Chapter 7 for configuration “periodic” state that “set according to parameters provided by test cases”. However, the ReportConfig configuration comes in fact from the CSI-RS RMC. Therefore, a default configuration should be provided |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the CSI-RS.x.1 and CSI-RS.x.2 default report config to Chapter 7 of TS 38.508-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test cases using periodic CSI-RS reporting would not have default Report Config settings |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: removed overlap with R5-214978 and aligned Table 7.3.1-12G |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### –	CSI-ReportConfig for BM

Table 7.3.1-12G: CSI-ReportConfig for BM

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-BM |  |  |
|  | CSI-ReportConfigId-BM with condition APERIODIC |  | APERIODIC |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| reportConfigType CHOICE { |  |  |  |
| periodic |  |  |  |
| reportSlotConfig ::= CHOICE { |  |  |  |
| slots80 | 2 |  | SCS15 |
| slots80 | 4 |  | SCS30 OR SCS120 |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF{ |  |  |  |
| PUCCH_CSI_Resource[0] SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id |  |  |
| pucch_Resource | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| aperiodic SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RSRP | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

< End of changes >