3GPP TSG-RAN5 Meeting #92-e	R5-214768r1

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2356 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 8.1.4.1.9.1-Reestablish intra-band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1．To remove measurement reporting related massge contents for simplify. 2．To correct typos 3．To correct derivation paths 4．To simplify message contents. 5．The message structure of SIB1 in Table 8.1.4.1.9.1.3.3-1 does not align with the IE definition in TS 38.508-1 or TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Measurement reporting related massge contents are removed. - Several typos are corrected. - Derivation paths are corrected - message contents are simplified. - Specific message content of SIB1 was corrected with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: To merge R5-214944. Coversheet is updated accordingly. Measurement configuration is removed according to discussion. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.4.1.9.1	NR CA / Intra NR handover / Failure / Re-establishment successful / Intra-band Contiguous CA

8.1.4.1.9.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and having received an RRCReconfiguration message including a reconfigurationWithSync for PCell change and including sCellToReleaseList with an sCellIndex set to the configured Scell }

ensure that {

when { UE detects handover failure and the source PCell is selectable}

then { UE successfully performs an RRCReestablishment procedure on source Pcell}

}

(2)

with { UE in NR RRC_CONNECTED state and having received an RRCReconfiguration message including a reconfigurationWithSync for PCell change and including sCellToReleaseList with an sCellIndex set to the configured SCell }

ensure that {

when { UE detects handover failure and the initial SCell is selectable}

then { UE successfully performs an RRCReestablishment procedure on original SCell and the original SCell becomes the PCell}

}

8.1.4.1.9.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.3.5.5.2, 5.3.5.5.9, 5.3.5.5.8 and 5.3.7.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.5.5.2]

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	stop timer T310 for the corresponding SpCell, if running;

1>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

1>	if the frequencyInfoDL is included:

2>	consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

1>	else:

2>	consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId;

1>	start synchronising to the DL of the target SpCell;

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

NOTE 1:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

NOTE 2:	The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access.

1>	reset the MAC entity of this cell group;

1>	consider the SCell(s) of this cell group, if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

1>	configure lower layers in accordance with the received spCellConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

[TS 38.331, clause 5.3.5.5.9]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated.

[TS 38.331, clause 5.3.5.5.8]

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell.

[TS 38.331, clause 5.3.7.5]

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	store the nextHopChainingCount value indicated in the RRCReestablishment message;

1>	update the KgNB key based on the current KgNB key or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc and KUPenc keys associated with the previously configured cipheringAlgorithm, as specified in TS 33.501 [11];

1>	derive the KRRCint and KUPint keys associated with the previously configured integrityProtAlgorithm, as specified in TS 33.501 [11].

1>	request lower layers to verify the integrity protection of the RRCReestablishment message, using the previously configured algorithm and the KRRCint key;

1>	if the integrity protection check of the RRCReestablishment message fails:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure', upon which the procedure ends;

1>	configure lower layers to resume integrity protection for SRB1 using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to resume ciphering for SRB1 using the previously configured algorithm and, the KRRCenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	release the measurement gap configuration indicated by the measGapConfig, if configured;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

1>	the procedure ends.

8.1.4.1.9.1.3	Test Description

8.1.4.1.9.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and NR Cell 3 is the SCell

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in all NR cells.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A as defined in TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.4.1.9.1.3.2	Test procedure sequence

Table 8.1.4.1.9.1.3.2-1 and Table 8.1.4.1.9.1.3.2-1A illustrates the downlink power levels to be applied for NR Cell 1 and NR Cell 3 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1", "T2", "T3" and "T4" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.4.1.9.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -79 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -85 | “Off” | Power levels are assigned to satisfy SrxLevCell3 < 0 such that selecting Cell 1 is guaranteed |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -79 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -79 | Power levels are assigned to satisfy SrxLevCell1 < 0 such that selecting Cell 3 is guaranteed |

Table 8.1.4.1.9.1.3.2-1A: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | [FFS] | [FFS] |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | [FFS] | [FFS] |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | [FFS] | “Off” | Power levels are assigned to satisfy SrxLevCell3 < 0 such that selecting Cell 1 is guaranteed |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | [FFS] | [FFS] |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | [FFS] | Power levels are assigned to satisfy SrxLevCell1 < 0 such that selecting Cell 3 is guaranteed |

Table 8.1.4.1.9.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 to configure NR Cell 3 as an SCell | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3-4 | Void | - | - | - | - |
| 5 | The SS changes NR Cell 1 and NR Cell 3 parameters according to the row "T1" in table 8.1.4.1.9.1.3.2-1/1A | - | - | - | - |
| 6 | Void | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform inter frequency handover to NR Cell 3 and to release SCell NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 8 the steps specified in Table 8.1.4.1.9.1.3.2-3 should take place. | - | - | - | - |
| 8 | The SS changes NR Cell 1 and NR Cell 3 parameters according to the row "T2" in table 8.1.4.1.9.1.3.2-1/1A | - | - | - | - |
| 9 | Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? | --> | NR RRC: RRCReestablishmentRequest | 1 | P |
| 10 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 11 | The UE transmits an RRCReestablishmentComplete message | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 11A | The SS changes NR Cell 1 and NR Cell 3 parameters according to the row "T3" in table 8.1.4.1.9.1.3.2-1/1A. | - | - | - | - |
| 12 | The SS transmits an RRCReconfiguration message to resume existing radio bearer on NR Cell 1 and configure NR Cell 3 as an SCell | <-- | NR RRC: RRCReconfiguration | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message on NR Cell 1 | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 14-17 | Void | - | - | - | - |
| 18 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 3 and to release SCell NR Cell3. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 19 the steps specified in Table 8.1.4.1.9.1.3.2-3 should take place. | - | - | - | - |
| 19 | The SS changes NR Cell 1 and NR Cell 3 parameters according to the row "T4" in table 8.1.4.1.9.1.3.2-1/1A | - | - | - | - |
| 20 | Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 3? | --> | NR RRC: RRCReestablishmentRequest | 2 | P |
| 21 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 3. | <-- | NR RRC: RRCReestablishment | - | - |
| 22 | The UE transmits an RRCReestablishmentComplete message | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 23 | The SS transmits an RRCReconfiguration message to resume existing radio bearer on NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 24 | The UE transmits an RRCReconfigurationComplete message on NR Cell 3. | --> | NR RRC: RRCReconfigurationComplete | - | - |

Table 8.1.4.1.9.1.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 3. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.1.4.1.9.1.3.3	Specific message contents

Table 8.1.4.1.9.1.3.3-1: SIB1 for NR Cell 1 and NR Cell 3 (Preamble and all the steps in Table 8.1.4.1.9.1.3.2-2)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |  |
| rach-ConfigGeneric SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |

Table 8.1.4.1.9.1.3.3-2: RRCReconfiguration (step 1 and 12, Table 8.1.4.1.9.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.1.9.1.3.3-3: CellGroupConfig (Table 8.1.4.1.9.1.3.3-2)

| Derivation path: TS 38.508-1 [4], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | 1 |  |  |
| sCellConfigCommon | ServingCellConfigCommon |  |  |
| sCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| sCellToReleaseList | Not present |  |  |
| } |  |  |  |

Table 8.1.4.1.9.1.3.3-4: ServingCellConfigCommon (Table 8.1.4.1.9.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical layer cell identity of NR Cell 3 |  |  |
| } |  |  |  |

Table 8.1.4.1.9.1.3.3-5: Void

Table 8.1.4.1.9.1.3.3-6: Void

Table 8.1.4.1.9.1.3.3-7: Void

Table 8.1.4.1.9.1.3.3-8: Void

Table 8.1.4.1.9.1.3.3-9: Void

Table 8.1.4.1.9.1.3.3-10: Void

Table 8.1.4.1.9.1.3.3-11: RRCReconfiguration (steps 7 and 18, Table 8.1.4.1.9.1.3.2-2)

| Derivation Path: TS 38.508-1[4], table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.1.9.1.3.3-12: CellGroupConfig (Table 8.1.4.1.9.1.3.3-11)

| Derivation path: TS 38.508-1 [4] table 4.6.3-19 with condition PCell_Change |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  |  | Value/remark |  |  | Comment |  |  | Condition |  |  |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| physCellId |  |  |  | PhysCellId of NR Cell 3 |  |  |  |  |  |  |  |  |
| downlinkConfigCommon SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| frequencyInfoDL SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| absoluteFrequencySSB |  |  |  | Downlink ARFCN of NR Cell 3 SSB |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |  |  |  |  |  |  |  |  |  |
| uplink SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| cfra SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| resources CHOICE { |  |  |  |  |  |  |  |  |  |  |  |  |
| ssb SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { |  |  |  | 1 entry |  |  |  |  |  |  |  |  |
| CFRA-SSB-Resource[1] SEQUENCE { |  |  |  |  |  |  | entry 1 |  |  |  |  |  |
| ra-PreambleIndex |  |  |  | 63 |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex { |  |  |  | 1 entry |  |  |  |  |  |  |  |  |
| SCellIndex[1] |  |  |  | 1 |  |  | entry 1 SCellIndex of NR Cell 3 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |

Table 8.1.4.1.9.1.3.3-13: RRCReestablishmentRequest (steps 9 and 20, Table 8.1.4.1.9.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| physCellId | PhysCellId of NR Cell 1 |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |

Table 8.1.4.1.9.1.3.3-14: RRCReestablishment (steps 10 and 21, Table 8.1.4.1.9.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.4.1.9.1.3.3-15: RRCReconfiguration (step 12, Table 8.1.4.1.9.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-13 with condition REEST |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition SCell_add and REEST |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.4.1.9.1.3.3-16: RRCReconfiguration (step 23, Table 8.1.4.1.9.1.3.2-2)

Derivation Path: TS 38.508-1 [4], table 4.6.1-13 with condition REEST

<End of modified section 1>