3GPP TSG-RAN5 Meeting #92-e	R5-214803r1

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1982 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IEs for Not served by E-UTRA and not served by NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, NR V2X. default configuration is not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Define default configuration of IEs for Not served by E-UTRA and not served by NR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X test case will have no default configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR... CR... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: 1, Remove all Spare bits field to follow same scheme as other Tables in TS 38.508-1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.5.5	V2X information elements of UE policies for V2X communication over PC5

…

–	Not served by E-UTRA and not served by NR

Table 4.7.5.5-6: Not served by E-UTRA and not served by NR

| Derivation Path: TS 24.588 Figure 5.3.1.6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |  |
| Length of not served by E-UTRA and not served by NR contents |  |  | Set to the actual length of 'not served by E-UTRA and not served by NR contents' in bytes |  |  |  |  |  |
| VPNENNI |  |  | '1'B |  | UE is authorized to use V2X communication over PC5 |  |  |  |
| NPINENN |  | '1'B | UE is authorized to use V2X communication over NR-PC5 when not served by E-UTRA and not served by NR |  |
| EPINENN |  | '1'B | UE is authorized to use V2X communication over E-UTRA-PC5 when not served by E-UTRA and not served by NR |  |
| E-UTRA radio parameters per geographical area list |  | See Table 4.7.5.5-7 |  |  |
| NR radio parameters per geographical area list |  | See Table 4.7.5.5-7 |  |  |

–	Radio parameters per geographical area list

Table 4.7.5.5-7: Radio parameters per geographical area list

| Derivation Path: TS 24.588 Figure 5.3.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area list contents |  | Set to the actual length of 'radio parameters per geographical area list contents' in bytes |  |  |
| Radio parameters per geographical area info 1 |  | See Table 4.7.5.5-8 |  |  |
| Radio parameters per geographical area info 2 |  | FFS |  |  |

–	Radio parameters per geographical area info

Table 4.7.5.5-8: Radio parameters per geographical area info

| Derivation TS 24.588 Figure 5.3.1.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents |  | Set to the actual length of 'radio parameters per geographical area contents' in bytes |  |  |
| Geographical area |  | See Table 4.7.5.5-9 |  |  |
| Radio parameters |  | See Table 4.7.5.5-11 |  |  |
| MI |  | '1'B | Operator managed |  |

–	Geographical area

Table 4.7.5.5-9: Geographical area

| Derivation Path: TS 24.588 Figure 5.3.1.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical area contents |  | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 |  | See Table 4.7.5.5-10 |  |  |
| Coordinate 2 |  | FFS |  |  |

–	Coordinate area

Table 4.7.5.5-10: Coordinate area

| Derivation Path: TS 24.588 Figure 5.3.1.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Latitude |  | '40 00 00'H | +45°, see cl 6.1 of TS 23.032 |  |
| Longitude |  | 'C0 00 00'H | -90°, see cl 6.1 of TS 23.032 |  |

–	Radio parameters

Table 4.7.5.5-11: Radio parameters

| Derivation Path: TS 24.588 Figure 5.3.1.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters contents |  | Set to the actual length of 'radio parameters contents' in bytes |  |  |
| Radio parameters contents |  | See Table 4.10.1.1-1: SL-V2X-Preconfiguration in TS 36.508 [2] |  | E-UTRAN V2X |
|  |  | See Table 4.10.1-1: SL-PreconfigurationNR |  | NR V2X |

<End of modified section 1>