3GPP TSG-RAN5 Meeting #92-e	R5-214804r1

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1983 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IEs for V2X service identifier to PC5 RAT and Tx profiles mapping rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, NR V2X. default configuration is not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Define default configuration of IEs for V2X service identifier to PC5 RAT and Tx profiles mapping rules |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X test case will have no default configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR... CR... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.5.5	V2X information elements of UE policies for V2X communication over PC5

…

–	V2X service identifier to PC5 RAT and Tx profiles mapping rules

Table 4.7.5.5-12: V2X service identifier to PC5 RAT and Tx profiles mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rules contents |  | Set to the actual length of 'V2X service identifier to PC5 RAT and Tx profiles mapping rules contents' in bytes |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 1 |  | See Table 4.7.5.5-13 |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to PC5 RAT and Tx profiles mapping rule

Table 4.7.5.5-13: V2X service identifier to PC5 RAT and Tx profiles mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 RAT and Tx profiles mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7.5.5-14 |  |  |
| PC5 RAT |  | ‘00’B |  | E-UTRA-PC5 |
|  |  | ‘01’B |  | NR-PC5 |
| Spare |  | ‘0000 00’B |  |  |
| Length of E-UTRA-PC5 Tx profiles |  | Set to the actual length of 'E-UTRA-PC5 Tx profiles' in bytes |  |  |
| E-UTRA-PC5 Tx profiles |  | rel14 | UE shall use Release 14 compatible format (i.e. using MCS table in Table 8.6.1-1 with 64 QAM indices overridden by 16QAM in TS 36.213 [23] and not Rel-15 feature) to transmit the corresponding V2X packet, Refer to IE v2x-TxProfileList in TS 36.331 |  |

–	V2X service identifiers

Table 4.7.5.5-14: V2X service identifiers

| Derivation Path: TS 24.588 Figure 5.3.1.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifiers contents |  | Set to the actual length of 'V2X service identifiers contents' in bytes |  |  |
| V2X service identifier 1 |  | '00 00 00 01'H | 4 bytes for each V2X service identifier |  |
| V2X service identifier 2 |  | '00 00 00 02'H |  |  |

<End of modified section 1>