3GPP TSG-RAN5 Meeting #92-e	R5-214881r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0207 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5GS IMS test case 10.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New 5GS IMS test case 10.14 need to be added according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 10.14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0287 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Updated test purpose according to WP. Updated test procedure and added conditions for sepcific message contents. |  |  |  |  |  |  |  |  |

10.14	In parallel emergency and non-emergency registrations / 5GS

10.14.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to start an emergency call }

then { UE performs IMS emergency registration and sets up an emergency call }

}

(2)

with { Emergency call ongoing }

ensure that {

when { Network sends NOTIFY terminating non-emergency public user identities }

then { UE sends 200 OK without impacting ongoing emergency registration and call }

}

10.14.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.6.2]:

When the UE performs an initial emergency registration and whilst this emergency registration is active, the UE shall:

-	handle the emergency registration independently from any other ongoing registration to the IM CN subsystem;

-	handle any signalling or media related IP-CAN for the purpose of emergency calls independently from any other established IP-CAN for IM CN subsystem related signalling or media; and

-	handle all SIP signalling and all media related to the emergency call independently from any other ongoing IM CN subsystem signalling and media.

10.14.3	Test description

10.14.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS by executing the generic test procedure in Annex A.2 up to the last step..

10.14.3.2	Test procedure sequence

Table 10.14.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to make an emergency call | - | - | - | - |
| 2-14 | Steps 1 -13 in generic test procedure in TS 38.508-1 [21] Table 4.9.11.2.2-1 are performed. And steps in A.3 and step A.6 are performed in the generic test procedure, to initiate IMS emergency registration and IMS emergency voice call. | - | - | 1 | P |
| 15 | The SS sends a NOTIFY for registration event package, containing partial registration state information, with all previously registered non-emergency public user identities as "terminated" and "rejected" |  | NOTIFY | - | - |
| 16 | The UE responds the NOTIFY with 200 OK |  | 200 OK | - | - |
| 17 | Check: Does the UE maintain the emergency call? | - | - | 2 | P |
| 18 | Wait 10 seconds. | - | - | - | - |
| 19 | The SS releases the emergency call |  | BYE | - | - |
| 20 | The UE sends 200 OK for BYE |  | 200 OK | - | - |

10.14.3.3	Specific message content

Table 10.14.3.3-1: NOTIFY (Step 12, table 10.14.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.1.6 with condition A6. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| CSeq |  |  |  |  |
| Value |  | 2 |  |  |
| Message-body |  | <?xml version=”1.0” encoding="UTF-8"?> <reginfo xmlns=”urn:ietf:params:xml:ns:reginfo” version=”1” state=”partial”> <registration aor=”PublicUserIdentity2 (NOTE 1)” id=”a102” state=”terminated”> <contact id=”980” state=”terminated” event=”rejected”> <uri>same value as in Contact header of REGISTER request</uri> </contact> </registration> <registration aor=”AssociatedTelUri(NOTE 1)” id=”a101” state=”terminated”> <contact id=”981” state=”terminated” event=”rejected”> <uri>same value as in Contact header of REGISTER request</uri> </contact> </registration> </reginfo> |  |  |

NOTE 1:	The public user ids and the associated TEL URI are as returned to the UE in the P-Associated-URI header of the 200 (OK) response to the REGISTER request;
PublicUserId1 is the default public user id i.e. the first one contained in P-Associated-URI;
AssociatedTelUri is the same as used in P-Associated-URI
PublicUserId2 and PublicUserId3 are the remaining IMPUs of the P-Associated-URI header

Table 10.14.3.3-2: 200 OK for NOTIFY (Step 13, table 10.14.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.3.1 with conditions A11 and A22

Table 10.14.3.3-3: BYE (Step 15, table 10.14.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A1 and A8.

Table 10.14.3.3-4: 200 OK for BYE (Step 16, table 10.14.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.3.1 with conditions A11 and A22