3GPP TSG-RAN WG5 Meeting #92-e	R5-215139r1

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5051 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-16 RACS RRC test case 8.5.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of Rel-16 RACS RRC test case 8.5.5.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added LTE Rel-16 RACS RRC test case 8.5.5.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACS work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1357 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-215139r1: 1. Updates to Table 8.5.5.1.3.2-2: Main behaviour. |  |  |  |  |  |  |  |  |

#### <Start of modified section>

#### 8.5.5	RACS / UL Message Segment transfer

##### 8.5.5.1	RACS / UL Message Segment transfer / UECapabilityInformation / Success

8.5.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a UECapabilityEnquiry message with rrc-SegAllowed-r16 enabled }

then { UE successfully segments UECapabilityInformation message at RRC layer and sends ULDedicatedMessageSegment messages with rrc-MessageSegmentType set accordingly for the first and last segment and rrc-MessageSegmentContainer containing the segments of UECapabilityInformation message }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and having received UECapabilityEnquiry message with rrc-segAllowed-r16 enabled and having transmitted the firstSegment of UECapabilityInformation message within the ULDedicatedMessageSegment message }

ensure that {

when { UE detects radio link failure on expiry of timer T310 }

then { UE successfully completes RRC re-establishment procedure and discards the remaining segments of UECapabilityInformation message }

}

8.5.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.6.3, 5.6.22.2, 5.6.22.3 and 5.3.7.2. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.6.3]

The UE shall:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

[TS 36.331, clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

…

1>	if UE is not configured with conditionalReconfiguration:

2>	release uplinkDataCompression, if configured;

2>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

2>	reset MAC;

8.5.5.1.3	Test description

8.5.5.1.3.1	Pre-test conditions

System Simulator:

- 	Cell 1

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18].

8.5.5.1.3.2	Test procedure sequence

Table 8.5.5.1.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 |
| --- | --- | --- | --- |
| T1 | Cell-specific RS RPRE | dBm/15kHz | “Off” |
| T2 | Cell-specific RS RPRE | dBm/15kHz | -85 |

Table 8.5.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including eutra to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else step 2b1 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the ULDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| 2b1 | The UE sends UECapabilityInformation Message. | --> | UECapabilityInformation | - | - |
| - | EXCEPTION: Steps 3a1 to 3a4 describe behaviour that are executed IF pc_Set_UE_Cap_Info_LTE=True; the "lower case letter" identifies a step sequence that takes place depending on the UE implementation. Note: Steps 3a1 to 3a4 may only be executed IF step 2b1 was performed. | - | - | - | - |
| 3a1 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 3a2 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 3a3 | The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: Step 3a4 is repeated a maximum of 2 times. | - | - | - | - |
| 3a4 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | ULDedicatedMessageSegment | 1 | P |
| - | EXCEPTION: Steps 4a1 to 4a11 describe behaviour that are executed IF Step 2a1 was performed OR IF pc_Set_UE_Cap_Info_LTE=True. | - | - | - | - |
| - | EXCEPTION: Steps 4a1a1 to 4a1a2 describe behaviour that are executed IF pc_Set_UE_Cap_Info_LTE=True; the "lower case letter" identifies a step sequence that takes place depending on the UE implementation. Note: Steps 4a1a1 to 4a1a2 may only be executed IF step 2b1 was performed. | - | - | - | - |
| 4a1a1 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 4a1a2 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 4a2 | SS stops regular grant transmission upon scheduling request. | - | - | - | - |
| 4a3 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| 4a4 | The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 4a5 | The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded. | - | - | - | - |
| 4a6 | Wait for T310 to expire | - | - | - | - |
| 4a7 | The SS changes Cell 1 parameters according to the row "T2" in table 8.5.5.1.3.2-1. | - | - | - | - |
| 4a8 | The UE sends an RRCReestablishmentRequest message on Cell 1. | --> | RRCReestablishmentRequest | - | - |
| 4a9 | The SS transmits an RRCReestablishment message to resume SRB1 operation on Cell 1. | <-- | RRCReestablishment | - | - |
| 4a10 | The UE transmits an RRCReestablishmentComplete message on Cell 1? | --> | RRCReestablishmentComplete | - | - |
| 4a11 | Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and does not send any ULDedicatedMessageSegment message in the next 5 seconds? | - | - | 2 | P |

8.5.5.1.3.3	Specific message contents

FFS

<End of modifed section>