3GPP TSG-RAN5 Meeting #92-e	R5-215168r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2428 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 11.2.1 5G-SRVCC from NG-RAN to 3GPP UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SRVCC_NR_to_UMTS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The content of UE-CapabilityRAT is not align with the type definition. The UE should do a GPRS Attach procedure instead of RAU procedure when handover to a UTRAN cell from 5G cell. The behavior after UE release the CS call on UTRAN cell should be completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correting the content of UE-CapabilityRAT Updating to GPRS Attach procedure instead of RAU procedure when handover to a UTRAN cell from 5G cell. Completing the behavior after UE release the CS call on UTRAN cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Updating to GPRS Attach procedure instead of RAU procedure when handover to a UTRAN cell from 5G cell. Completing the behavior after UE release the CS call on UTRAN cell. |  |  |  |  |  |  |  |  |

11.2.1.3	Test Description

11.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 with system information combination NR-1 as defined in TS 38.508-1 [4].

-	UTRA Cell 5 as defined in TS 36.508 [7].

UE:

None.

Preamble:

- The UE is in test state 0N-B according to TS 38.508-1.

11.2.1.3.2	Test procedure sequence

Table 11.2.1.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration T0 indicates the initial conditions for preamble. Configurations marked "T1" and “T2” are applied at the points indicated in the Main behaviour description in Table 11.2.1.3.2-3.

Table 11.2.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | UTRA Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz |  | -64 |  |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz |  | Off |  |

Table 11.2.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | UTRA Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz |  | -64 |  |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz |  | Off |  |

Table 11.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
|  | The following messages are to be observed on NR Cell 1 unless explicitly stated otherwise. | - | - | - | - |
| 2-3 | The UE establishes RRC connection by executing steps 2-3 of Table 4.5.2.2-2 in TS38.508-1 [4]. | - | - | - | - |
| 4 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message indicating 5G-SRVCC from NG-RAN to UTRAN supported | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5-19 | Steps 5-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 20 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for NR and UTRA. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 21 | The UE transmits a UECapabilityInformation message. NOTE: The start-CS values received, should be used to configure ciphering on cell 5. | --> | NR RRC: UECapabilityInformation | - | - |
| 22 | The SS releases the RRC Connection | - | - | - | - |
| 23 | Perform Generic Test Procedure for IMS MO speech call establishment as specified in TS 38.508-1 [4], subclause 4.9.15 | - | - | - | - |
| 24 | Set the power levels according to “T1” as per Table 11.2.1.3.2-1/2. | - | - | - | - |
| 25 | The SS transmits a MobilityFromNRCommand message. | <-- | NR RRC: MobilityFromNRCommand | - | - |
|  | The following messages are to be observed on UTRA Cell 5 unless explicitly stated otherwise. | - | - | - | - |
| 26 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message on cell 5? | - | HANDOVER TO UTRAN COMPLETE | 1 | P |
| - | EXCEPTION: In parallel to the events described in step 27 to 32 the steps specified in table 11.2.1.3.2-4 takes place. | - | - | - | - |
| 27 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 28 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 29 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 30 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 31 | The SS transmits a TMSI REALLOCATION COMMAND message. | <-- | TMSI REALLOCATION COMMAND | - | - |
| 32 | The UE transmits a TMSI REALLOCATION COMPLETE message. | --> | TMSI REALLOCATION COMPLETE | - | - |
| 33 | Make the UE release the CS call. | - | - | - | - |
| 33 | Make the UE release the CS call. | - | - | - | - |
| 33A | The UE transmits a DISCONNECT message | --> | DISCONNECT |  |  |
| 33B | The SS transmits a RELEASE message | <-- | RELEASE | - | - |
| 33C | The UE transmits a RELEASE COMPLETE message. | --> | RELEASE COMPLETE | - | - |
| 33D | The SS releases the RRC connection. | - | - | - | - |
| 34 | Set the power levels according to “T2” as per Table 11.2.1.3.2-1/2. | - | - | - | - |
| 35-52 | Check: Does the UE perform initial registration procedure on NR Cell 1 as steps 2-19a1 of Table 4.5.2.2-2 NR RRC_IDLE in TS38.508-1 [4]? | - | - | 2 | P |

Table 11.2.1.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | - | - |
| 1AA | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 1AB | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 1A | The SS transmits a SECURITY MODE COMMAND message for the PS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 1B | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2 | The SS transmits an ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 3 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |

11.2.1.3.3	Specific message contents

Table 11.2.1.3.3-1: Message REGISTRATION REQUEST (step 4, Table 11.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GMM capability | '???? ???? ???? ???? ???? ???? ???? ???1 *'B | 5G-SRVCC from NG-RAN to UTRAN supported |  |

Table 11.2.1.3.3-2: UECapabilityEnquiry (step 20, Table 11.2.1.3.2-3)

| Derivation Path: 38.508-1 [4], Table 4.6.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |
| ue-CapabilityRAT-RequestList SEQUENCE (SIZE (1.. maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { | 2 entries |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| } |  |  |  |
| UE-CapabilityRAT-Request[2] SEQUENCE { |  | entry 2 |  |
| rat-Type | utra-fdd-v1610 |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| criticalExtensionsFuture |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2.1.3.3-3: UECapabilityInformation (step 21, Table 11.2.1.3.2-3)

| Derivation Path: 38.508-1 [4], Table 4.6.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { | 2 entries |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| ueCapabilityRAT-Container | Not checked |  |  |
| } |  |  |  |
| UE-CapabilityRAT-Request[2] SEQUENCE { |  | entry 2 |  |
| rat-Type | utra-fdd-v1610 |  |  |
| ueCapabilityRAT-Container | Octet string | INTER RAT HANDOVER INFO |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| criticalExtensionsFuture | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2.1.3.3-4: MobilityFromNRCommand (step 25, Table 11.2.1.3.2-3)

| Derivation Path: 38.508-1 [4] Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromNRCommand::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mobilityFromNRCommand ::= SEQUENCE { |  |  |  |
| targetRAT-Type | utra-fdd-v1610 |  |  |
| targetRAT-MessageContainer | HANDOVER TO UTRAN COMMAND(UTRA RRC message) |  |  |
| nas-SecurityParamFromNR | 4 LSB of the downlink NAS COUNT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2.1.3.3-5: HANDOVER TO UTRAN COMMAND (step 11, Table 11.2.1.3.2-3)

Derivation Path: 36.508 [7], Table 4.7B.1-1, condition UTRA Speech

Table 11.2.1.3.3-6: SECURITY MODE COMMAND  (step 12, Table 11.2.1.3.2-3)

| Derivation Path: 36.508 [7], Table 4.7B.1-n |  |  |
| --- | --- | --- |
| Information Element | Condition | Value/remark |
| Ciphering mode info |  | Not Present |

Table 11.2.1.3.3-7: ROUTING AREA UPDATE ACCEPT (step 2, 11.2.1.3.2-4)

| Derivation path: 36.508 [7], Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

Table 11.2.1.3.3-8: SECURITY MODE COMMAND (step 1A, 11.2.1.3.2-4)

| Derivation Path: 36.508 [7], Table 4.7B.1-n |  |  |
| --- | --- | --- |
| Information Element | Condition | Value/remark |
| Ciphering mode info |  | StartRestart |
| Integrity protection mode info |  | modify |
| CN Domain Identity |  | ps-domain |