3GPP TSG-RAN5 Meeting #92-e	R5-215173r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1473 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to annex A.2.19: MO INFO for eCall over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For MT SIP INFO TE populates the Request-uri with the uri that UE sent in the Contact header of INVITE - For A.2.19 condition A4, a different “purpose” is required in call-Info header as per RFC 8147and it should be “EmergencyCallData.Control” - For A.2.19 condition A4, expected contentType in Message-Body is “application/EmergencyCallData.eCall.Control+xml” but according to RFC 8147, it should be “application/EmergencyCallData.Control+xml”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changing the expected Request-URI with the value of Request-URI that TE sent in its Contact header of 200 OK(for INVITE) earlier in the dialog. - For condition A4, a new “purpose” has been added and its value is “EmergencyCallData.Control”. - Expected contentType in Message-Body, “application/EmergencyCallData.eCall.Control+xml” has been changed to “application/EmergencyCallData.Control+xml”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the eCall over IMS test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-215173->R5-215173r1, Corrected formatting issues, clarified the proposed value of Request-URI, corrected the release and reference columns to reflect the changes |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.2.19	MO INFO for eCall over IMS

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | INFO |  | RFC 6086 [139] |
| Request-URI |  |  |  | same URI as SS sent in its Contact header before when it sent 200 OK for INVITE |  |  |  |  |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Via |  | order of the parameters in this header must be like in this table |  | RFC 3261 [15] RFC 3581 [96] |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by |  | IP address or FQDN and protected server port of the UE |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE |  | TS 24.229 [10] |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | urn:service:sos.ecall.manual | Rel-14 | TS 24.229 [10] RFC 8147 [149] |
|  | A2 | urn:service:sos.ecall.automatic | Rel-14 | TS 24.229 [10] RFC 8147 [149] |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same as value received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |
| method |  | INFO |  |  |
| Call-Info |  |  |  |  |
| cid URL |  | any URL | Rel-14 | RFC 8147 [149] |
| purpose |  | A3 |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
|  | A4 | EmergencyCallData.Control | Rel-14 | RFC 8147 [149] |
| Info-Package |  | EmergencyCallData.eCall.MSD | Rel-14 | RFC 8147 [149] |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | multipart/mixed;boundary=any value |  | TS 24.229 [10] |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |
| Content-Disposition |  |  |  | RFC 3261 [15] |
| disp-type |  | Info-Package | Rel-14 | RFC 8147 [149] |
| Message-body |  |  |  |  |
|  | A3 | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: same cid as in Call-Info header Content-Disposition: by-reference MSD in ASN.1 PER encoding --boundary value (as provided in Content-Type) | Rel-14 | RFC 8147 [149] |
|  | A4 | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData..Control+xml Content-ID: same cid as in Call-Info header Content-Disposition: by-reference <?xml version=”1.0” encoding=”UTF-8”?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack ref=cid of the body part of corresponding INFO request from SS> <actionResult action=”send-data” success=”false” reason=any value/> </ack> </EmergencyCallData.Control> --boundary value (as provided in Content-Type) | Rel-14 | RFC 8147 [149] |

| Condition | Explanation |
| --- | --- |
| A1 | eCall over IMS was started manually |
| A2 | eCall over IMS was started automatically |
| A3 | UE able to provide an updated MSD |
| A4 | UE not able to provide an updated MSD |