3GPP TSG-RAN5 Meeting #92-e	R5-215273r1

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0595 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of FR2 Carrier Leakage Test Case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton, Keysight technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CBW in test configuration for FR2 Carrier Leakage Test case is unsynchronized with test point analysis in TR38.905 [21]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of CBW for FR2 Carrier Leakage Test Case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CBW for FR2 Carrier Leakage Teest Case will be only applicable to certain band combos. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2.2.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - T-doc number of CR is inconsistency with 3GU |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.4A.2.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.1.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | Default | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 |  |  |  |  |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

## << Unchanged sections omitted >>

Table 6.4A.2.2.2.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | Default | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 |  |  |  |  |  |
|  | SCC/CC3 |  |  |  |  |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_16RB_Left_Region2 for PC1, use Inner_16RB_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

## << End of changes >>