3GPP TSG-RAN5 Meeting #92-e	R5-215348r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2016 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Perf RI FR2 message contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Default Message Contents for FR2 RI reporting requirements are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added FR2 conditions to 5.4.2.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance Test cannot be correctly performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add ‘FR1’ to conditions in Table 5.4.2.6-5 |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

5.4.2.6	Message contents for RI reporting requirements

NZP-CSI-RS for Tracking

CSI-RS-ResourceMapping

Table 5.4.2.6-1: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 | k0=0 for CSI-RS resource 1,2,3,4 | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4 | TRS |
| cdm-Type | noCDM |  | TRS |
| density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  | TRS |
| } |  |  |  |

NZP CSI-RS for CSI Acquisition

CSI-RS-ResourceMapping

Table 5.4.2.6-2: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row4 | 001 | K0 = 0, row4 | 4Tx |
| other | 001000 | K0 = 6, row3 | 2Tx |
| } |  |  |  |
| nrofPorts | p4 |  | 4Tx |
|  | p2 |  | 2Tx |
| firstOFDMSymbolInTimeDomain | 13 | I0 = 13 | 2Tx, 4Tx |
| cdm-Type | fd-CDM2 |  | 2Tx, 4Tx |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2Tx | For test cases using 2 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |
| 4Tx | For test cases using 4 CSI-RS ports for NZP-CSI-RS for CSI acquisition. |

Table 5.4.2.6-3: CSI-ResourcePeriodicityAndOffset for CSI Acquisition

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots5 | 1 |  | FR1_FDD |
| Slots10 | 1 |  | FR1_TDD |
| } |  |  |  |

ZP CSI-RS for CSI Acquisition

CSI-RS-ResourceMapping

Table 5.4.2.6-4: ZP CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | K0 = 4, row5 | FR1 |
| row4 | 100 | K0 = 8, row4 | FR2 |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 | I0 = 9 | FR1 |
|  | 13 | I0 = 13 | FR2 |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | ZP CSI-FrequencyOccupation |  |  |
| } |  |  |  |

CSI-ResourcePeriodicityAndOffset

Table 5.4.2.6-5: ZP CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots5 | 1 |  | FR1 FDD |
| Slots10 | 1 |  | FR1 TDD |
| Slots8 | 1 |  | FR2 |
| } |  |  |  |

PDSCH-Config

Table 5.4.2.6-6: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| p-ZP-CSI-RS-ResourceSetsToAddModList | p-ZP-CSI-RS-ResourceSet |  |  |
| } |  |  |  |

p-ZP-CSI-RS-ResourceSet

Table 5.4.2.6-7: p-ZP-CSI-RS-ResourceSet

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-ZP-CSI-RS-ResourceSetsToAddModList ::= SEQUENCE { |  |  |  |
| zp_CSI-RS_ResourceSetId | 0 |  |  |
| zp-CSI-RS-ResourceIdList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId{ | 1 entry |  |  |
| ZP-CSI-RS-ResourceId[1] | 0 | entry 1 ZP CSI-RS resource 1 |  |
| } |  |  |  |
| } |  |  |  |

CSI-ResourceConfig

Table 5.4.2.6-8: CSI-ResourceConfig for CSI Acquisition

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 1 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 1 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  | FR1 |
|  | aperiodic |  | FR2 |
| } |  |  |  |

Table 5.4.2.6-8A: CSI-ResourceConfig for beam refinement

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  | FR2 |
| csi-ResourceConfigId | 3 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 2 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

CSI-IM Configuration

CSI-IM-Resource

Table 5.4.2.6-9: CSI-IM-Resource

| Derivation Path: Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| Pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  | FR1 |
|  | s8 |  | FR2 |
| symbolLocation-p0 | 9 |  | FR1 |
|  | 13 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  | FR1 |
|  | Not present |  | FR2 |
| } |  |  |  |

Table 5.4.2.6-10: CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots5 | 1 |  | FR1_FDD |
| Slots10 | 1 |  | FR1_TDD |
| } |  |  |  |

CSI-IM-ResourceId

Table 5.4.2.6-11: CSI-IM-ResourceId

| Derivation Path: Table 4.6.3-35 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | 0 |  |  |

CSI-IM-ResourceSet

Table 5.4.2.6-12: CSI-IM-ResourceSet

| Derivation Path: Table 4.6.3-36 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId |  |  |
| csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

CSI-IM-ResourceSetId

Table 5.4.2.6-13: CSI-IM-ResourceSetId

| Derivation Path: Table 4.6.3-37 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

CSI-IM-ResourceConfig

Table 5.4.2.6-14: CSI-IM-ResourceConfig

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| csi-IM-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  | FR1 |
|  | aperiodic |  | FR2 |
| } |  |  |  |

CSI-MeasConfig

Table 5.4.2.6-15: CSI-MeasConfig

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=5 for FR1 n=7 for FR2 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | NZP-CSI-RS-Resource for beam refinement | entry 6 CSI-RS resource 6 | FR2 |
| NZP-CSI-RS-Resource[7] | NZP-CSI-RS-Resource for beam refinement | entry 7 CSI-RS resource 7 | FR2 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | n entries | n=2 for FR1 n=3 for FR2 |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2 |  |
| NZP-CSI-RS-ResourceSet[3] | NZP-CSI-RS-ResourceSet for beam refinement | entry 3 | FR2 |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource |  |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | n entries | n=3 for FR1 n=4 for FR2 |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| CSI-ResourceConfig[2] | CSI-ResourceConfig for CSI Acquisition | entry 2 |  |
| CSI-ResourceConfig[3] | CSI-IM-ResourceConfig | entry 3 |  |
| CSI-ResourceConfig[4] | CSI-ResourceConfig for beam refinement | entry 4 | FR2 |
| } |  |  |  |
| } |  |  |  |

CSI-ReportConfig

Table 5.4.2.6-16: CSI-ReportConfig

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId for CSI Acquisition |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId for CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  | FR1 |
| reportSlotConfig | CSI-ReportPeriodicityAndOffset |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { |  |  |  |
| PUCCH-CSI-Resource [1] | PUCCH-CSI-Resource |  |  |
| } |  |  |  |
| aperiodic SEQUENCE { |  |  | FR2 |
| reportSlotOffsetList SEQUENCE { |  |  |  |
| reportSlotOffsetList[1] | 6 |  |  |
| reportSlotOffsetList[2] | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL, |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand CHOICE { |  |  | FR1 |
| subbands7 | '1111111'B |  |  |
| } |  |  |  |
| csi-ReportingBand CHOICE { |  |  | FR2 |
| subbands9 | '111111111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table2 |  | FR1 |
|  | table1 |  | FR2 |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

CodebookConfig

Table 5.4.2.6-17: CodebookConfig

| Derivation Path: Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| two SEQUENCE { |  |  | 2Tx |
| twoTX-CodebookSubsetRestriction | '010000'B |  | Fixed rank2, 2x2, 2x4 |
|  | ‘010011’B |  | Follow rank, 2x2, 2x4 |
|  | ‘000011’B |  | Fixed rank1, 2x2, 2x4 |
| } |  |  |  |
| moreThanTwo SEQUENCE { |  |  | 4Tx |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | ‘11111111’B |  | 4x4 |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | ‘11111111’B |  | 2x2, 2x4 |
|  | ‘00000010’B |  | Fixed rank2, 4x4 |
|  | ‘00001111’B |  | Follow RI, 4x4 |
| } |  |  |  |
| } |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

CSI-ReportPeriodicityAndOffset

Table 5.4.2.4-18: CSI-ReportPeriodicityAndOffset

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportPeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 9 |  | FR1_TDD |
| slots5 | 0 |  | FR1_FDD |
| } |  |  |  |

PUCCH-CSI-Resource

Table 5.4.2.4-19: PUCCH-CSI-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-CSI-Resource ::= SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-id |  | FR1 |
| pucch-Resource | 8 |  | FR1 |
| } |  |  |  |

<<End of change>>