3GPP TSG-RAN5 Meeting #92-e	R5-215372r3

# Electronic Meeting, 16th August – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0101 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to applicability spec for 5G test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas, China Mobile, MediaTek Inc., Huawei, HiSilicon, Ericsson, ROHDE & SCHWARZ, TTA, Qualcomm Incorporated, Anritsu, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Some of the sub-title are needed for groups of test cases; - Some of RRM test cases with incorrect applicability indicated; - According to 38.101-1 clause 6.4.2.1, power boosting is only applicable to TDD bands n40, n41, n77, n78 and n79. The tested bands for FR1 TC 6.4.2.5 were not in line with the core requirements. - For inter-band EN-DC, MPR, SEM and ACLR test cases are applied to LTE anchor agnostic approach due to no exception requirements. Therefore, similar to the principle in LTE and NR SA, MPR test case can be covered by SEM or ACLR case as well for inter-band EN-DC within FR1. Likewise, A-MPR also can be covered by A-SEM case for inter-band EN-DC within FR1; - Some of test cases still missing condition and/or complete status in applicability table; - Some of applicability are incorrectly mapping to the RRM test cases; - The current title of the inter-RAT test cases in Chapter 8 is misleading and seems to indicate that there is an active NR SA connection at some point in the test; - There’re some terms used in the condition is not clear enough. - Table 4.0-3 Tested CA/DC Configuration Selection Criteria condition/comment is not proper adapted to the test cases; |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added sub-title to TS38.521-3 test cases; - Some editorial correction; - Update to Table 4.0-1: Applicability of conformance test cases conditions: - New C021b, C041b, C042b for RRM TC used for EN-DC FDD FR1 only; - New C064a, C064b for EN-DC FR1 with 4 and 5 NR DL CCs tests; - New C012f for EN-DC TC6.4B.2.4.5; - New C043a for RRM TC5.6.1.4; - Updated C065/C066 for EN-DC BWP switching TC; - New C080, C080a for inter-RAT SFTD measurements test; - New C082, C082a, C083, C083a RRM EN-DC link recovery TC4.5.5.1 / 4.5.5.2 / 4.5.5.3 / 4.5.5.4; - New C084, C084a, C085, C085a for RRM SA link recovery TC6.5.5.1 / 6.5.5.2 / 6.5.5.3 / 6.5.54 - New C030a; - Typo correct to C012z; - New C086, C086a, C080, C080a for Inter-RAT measurement TC8.4.2.x and 8.5.2.x; - New C065a and C066a - Update to Table 4.0-2: Tested Bands Selection Criteria: - New D017 for limiting test band for FR1 TC6.4.2.5; - Deleted redundant sentence; - Update to Table 4.0-3: Tested CA/DC Configuration Selection Criteria: - New E028a, E028b, E030a, E030b for EN-DC FR1 with 4 and 5 NR DL CCs tests; - Added new NOTE for UL; UL_nCC; DL_nCC; ULTxSwitching; - Corrected multiple Exxx with proper information and comment used for the test cases. - Update to Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases: - Update to Table 4.1.2-1: Applicability of RF SA FR2 conformance test cases: - Additional information updated for TC6.4.2.5; - Added 6.4D.1 with NOTE 1; - Addditional information update for TC6.4.3.2; - Update to Table 4.1.3-1: Applicability of RF EN-DC FR1 and FR2 conformance test cases; - MPR test applicabiliy for inter-band EN-DC to specify that MPR can be covered by SEM or ACLR test; (depends on R5-215196) - A-MPR test applicabiliy for inter-band EN-DC to specify that MPR can be covered by A-SEM test; (depends on R5-215196) - Added 7.5B.3_1.3 with complete status; - Added 6.6B.4 / 7.5B.3_1.4 with NOTE 1; - Added 6.4B.2.4.5 with NOTE 1 and NOTE 5; - Added 6.5.B.3.4.2 with NOTE 1 and removed NOTE 1; - Update to Table 4.1.4-1: Applicability of performance test cases: - Update to Table 4.2-x: Applicability of RRM conformance test cases: - TC4.6.1.5 / 4.6.1.6 / 6.6.1.6 with EN-DC FDD FR1 only condition; - TC5.6.1.2 / 5.6.1.4 / 5.6.2.1 to 5.6.2.8 / 5.6.3.1 / 5.6.3.2 / 7.6.2.1 to 7.6.2.8 / 7.6.3.1 / 7.6.3.2 removed NOTE 1; - Added TC5.3.2.2.1 / 5.3.2.2.1 with completed status; - Applicability/Comment updated to TC4.5.6.1.1 / 4.5.6.1.2 / 4.5.6.2.1 / 6.5.6.1.1 / 6.5.6.2.1 / 8.4.1.1 / 8.4.1.2 / 8.5.1.1; - Applicability/Comment updated to TC4.5.5.1 / 4.5.5.2 / 4.5.5.3 / 4.5.5.4 / 6.5.5.1 / 6.5.5.2 / 6.5.5.3 / 6.5.5.4 to added the corresponding applicability conditions to include CSI-RS configuration as RLM-RS; - The title of chapter 8 inter-RAT test cases has been modified to better reflect the test configuration; - Updated codntions/comment to 8.4.2.1 to 8.4.2.8 / 8.5.2.1.1.1 / 8.5.2.1.2 / 8.5.2.2.1 / 8.5.2.2.2 / 8.5.2.3.1 / 8.5.2.3.2. - Updated 7.5.5.3 / 7.5.5.4 condition/comment to FFS as not complete TCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cases can not be selected correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0, 4.1.1, 4.1.2, 4.1.3, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A.4.3.6-1/xx01 and A.4.3.6-1/xx02 used in C081 and C081a are define in TS38.508-2 CR 0228 (R5-214979); A.4.3.6-1/Z1, A.4.3.6-1/Z2 and A.4.3.6-1/Z3 used in C082, C082a, C083, C083a, C084, C084a, C085, and C085a are defined in TS38.508-2 CR 0224 (R5-214679); |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: added RRM TC5.3.2.2.1 and 5.3.2.2.2 with NOTE 1; RRM TC5.6.1.2/5.6.1.4 removed NOTE 1; merged content of R5-214980(HW), R5-214950(QC), R5-214380 (R&S) and R5-215435(R&S); Added source company; r2: deleted redundant wording in NOTE 1 of Table A.4.0-2; Moved new defination into NOTE under Table A.4.0-3; Updated 7.5.5.3 / 7.5.5.4 condition/comment to FFS as not complete TCs; Some Exxx corrected. r3: updated C041b condition and removed ICS related to CA, as TC not for CA but for intra-frequency measurements; re-organized the Cxxx numbering mapping to the TC, update TC4.5.6.1.1 / 4.5.6.1.2 / 4.5.6.2.1 / 6.5.6.1.1 with update C065/C066 and new C065a/C066a. |  |  |  |  |  |  |  |  |

## {Start of changes}

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

The present document is one part of a multi-part Technical Specification (TS) covering the New Radio (NR) User Equipment (UE) conformance specification, which is divided in the following parts:

3GPP TS 38.521-1 [1]: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone;

3GPP TS 38.521-2 [2]: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone;

3GPP TS 38.521-3 [3]: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios;

3GPP TS 38.521-4 [4]: NR; User Equipment conformance specification; Radio transmission and reception; Part 4: Performance;

3GPP TS 38.522: NR; User Equipment (UE) conformance specification; Applicability of RF and RRM test cases;

3GPP TS 38.533 [5]: NR; User Equipment (UE) conformance specification; Radio resource management;

# 1	Scope

The present document specifies the recommended applicability statement and completion status for the test cases included in 3GPP TS 38.521-1 [1], TS 38.521-2 [2], TS 38.521-3 [3], TS 38.521-4 [4] and TS 38.533 [5]. These applicability statements are based on the features implemented in the UE.

Special conformance testing functions can be found in 3GPP TS 38.509 [6] and the common test environments are included in 3GPP TS 38.508-1 [7]. Common implementation conformance statement (ICS) proforma can be found in 3GPP TS 38.508-2 [8].

The present document is valid for UE implemented according to 3GPP releases starting from Release 15 up to the Release indicated on the cover page of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TS 38.521-1: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[2]	3GPP TS 38.521-2: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[3]	3GPP TS 38.521-3: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

[4]	3GPP TS 38.521-4: NR; User Equipment conformance specification; Radio transmission and reception; Part 4: Performance

[5]	3GPP TS 38.533: NR; User Equipment (UE) conformance specification; Radio resource management

[6]	3GPP TS 38.509: 5GS; Special conformance testing functions for User Equipment (UE)

[7]	3GPP TS 38.508-1: 5GS; User Equipment (UE) conformance specification; Part 1: Common test environment

[8]	3GPP TS 38.508-2: 5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma

[9]	3GPP TR 21.905: Vocabulary for 3GPP Specifications

[10]	3GPP TS 36.521-2: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Implementation Conformance Statement (ICS)

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [9] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [9].

EIRP(Link=Link angle, Meas=Link angle): measurement of the UE such that the link angle is aligned with the measurement angle. EIRP (indicator to be measured) can be replaced by EIS, Frequency, EVM, carrier Leakage, In-band emission and OBW. Beam peak search grids, TX beam peak direction, and RX beam peak direction can be selected to describe Link.

EIRP(Link=Link angle, Meas=beam peak direction): measurement of the EIRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement error uncertainty.

Implementation Conformance Statement (ICS): statement made by the supplier of an implementation or system claimed to conform to a given specification, stating which capabilities have been implemented

ICS proforma: document, in the form of a questionnaire, which when completed for an implementation or system becomes an ICS

Implementation extra Information for Testing (IXIT): A statement made by a supplier or implementer of an UEUT which contains or references all of the information (in addition to that given in the ICS) related to the UEUT and its testing environment, which will enable the test laboratory to run an appropriate test suite against the UEUT

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

IXIT proforma: A document, in the form of a questionnaire, which when completed for an UEUT becomes an IXIT

Protocol Implementation Conformance Statement (PICS): An ICS for an implementation or system claimed to conform to a given protocol specification

Protocol Implementation eXtra Information for Testing (PIXIT): An IXIT related to testing for conformance to a given protocol specification

Static conformance review: A review of the extent to which the static conformance requirements are claimed to be supported by the UEUT, by comparing the answers in the ICS(s) with the static conformance requirements expressed in the relevant specification(s)

TRP(Link=Link angle): measurement of the TRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement uncertainty. TX beam peak direction and RX beam peak direction can be selected to describe Link.

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

UL: UL(table_index) includes all supported CA Configurations where at least one UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8].

UL_nCC: UL_nCC(table_index) includes all supported CA Configurations where at least one n-carrier UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8].

DL_nCC: DL_nCC(table_index) includes all supported n-carrier CA/DC Configurations in Table table_index in TS 38.508-2 [8].

ULTxSwitching: ULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported ULTxSwitching Band Pair" in Table table_index in TS 38.508-2 [8].

## 3.2	Symbols

No specific symbols have been identified so far.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [9] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [9].

For the purposes of the present document, the following abbreviations apply:

BWP	Bandwidth Part

CA	Carrier Aggregation

DRX	Discontinuous Reception

EN-DC	E-UTRA NR-Dual Connection

FR1	Frequency Range 1 (410 MHz - 7125 MHz)

FR2	Frequency Range 2 (24250 MHz - 52600 MHz)

HST	High Speed Train

ICS	Implementation Conformance Statement

IXIT	Implementation eXtra Information for Testing

NR	New Radio

PIXIT	Protocol Implementation eXtra Information for Testing

RLM	Radio Link Monitoring

RRM	Radio Resource Management

SCS	System Conformance Statement

SUL	Supplementary UpLink

TC	Test Case

TRP	Total Radiated Power

UEUT	User Equipment Under Test

# 4	Recommended test case applicability

The applicability of each individual test is identified in the tables 4.1.1-1 / 4.1.2-1 / 4.1.3-1 / 4.1.4-1 / 4.2-1 / 4.2-2 / 4.2-3 / 4.2-4. This is just a recommendation based on the purpose for which the test case was written.

The applicability of every test is formally expressed by the use of Boolean expressions that are based on parameters (ICS). The parameters (ICS) included in TS 38.508-2 [8] are used in the test case applicability condition without reference. Parameters (ICS) specified in TS 36.521-2 [10] shall be referred with proper reference.

Selection criteria of tested bands and tested CA configurations for each applicable test is formally expressed using group theory based on parameters (ICS) included in annex A of TS 38.508-2 [8] without reference.

Additional information related to the Test Case (TC), e.g. affecting its dynamic behaviour or its execution may be provided as well.

The columns in tables 4.1.1-1 / 4.1.2-1 / 4.1.3-1 / 4.1.4-1 / 4.2-1 / 4.2-2 / 4.2-3 / 4.2-4 have the following meaning:

Clause

The clause column indicates the clause number in TS 38.521-1 [1], TS 38.521-2 [2], TS 38.521-3 [3], TS 38.521-4 [4] and TS 38.533 [5] that contains the test body.

TC Title

The TC Title column describes the name of the test and contains the clause title of the clause in TS 38.521-1 [1], TS 38.521-2 [2], TS 38.521-3 [3], TS 38.521-4 [4] and TS 38.533 [5] that contains the test body.

Release

The release column indicates the earliest release from which each test case is applicable. It may also indicate a range of releases or a single release to which a test case is applicable.

Applicability - Condition

The following notations are used for the applicability column:

R	recommended - the test case is recommended to all terminals supporting NR

O	optional - the test case is optional

N/A	not applicable - in the given context, the test case is not recommended.

Ci	conditional - the test is recommended ("R") or not ("N/A") depending on the support of other items. "i" is an integer identifying a unique conditional status expression which is defined in Table 4.0-1. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is used to avoid ambiguities.

Applicability - Comment

This comment column contains a verbal description of the condition included in the applicability column.

Tested Bands / CA/DC Configurations Selection

This column defines a set of bands / CA/DC Configurations the test is to be run for, if the test is applicable. If the set is empty, the test is considered as not applicable.

The following notations are used in the tested bands selection column:

Di	Derive the set based on Band Selection Criteria Di defined in table 4.0-2.

Ei	Derive the set based on CA/DC Configurations Selection Criteria Ei defined in table 4.0-3.

TBD	Band selection not defined at this time, in the meantime test all Bands / CA/DC Configurations

Text	For more complex selection criteria, or if the criteria are already specified somewhere else in the spec, text reference to the clause is given.

Branch

This column contains indication if the test case may perform differently depending on the UE capabilities.

NOTE 1:	Void.

NOTE 2:	Void.

Additional Information

This column contains indication if the test case may perform differently depending on the UE capabilities and the measurement execution.

This column also contains indication of the completion status of the test case.

## 4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2 and Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-1: Applicability of conformance test cases conditions

C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A

C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C002 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C003 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C003a IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C004 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-2/1 THEN R ELSE N/A

C005 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.1-2/4 AND A.4.3.2A.1-1/1 AND A.4.1-3/1 THEN R ELSE N/A

C006 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 THEN R ELSE N/A

C006a IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C006b IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/31a THEN R ELSE N/A

C007 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/22 THEN R ELSE N/A

C008 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND NOT(A.4.3.2-1/22) THEN R ELSE N/A

C009 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C009a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C009z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C010 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C010a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C010z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C011 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C011a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C011b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2A/1 THEN R ELSE N/A

C011c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1A/1 THEN R ELSE N/A

C011z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2A/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C012 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C012a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/1 THEN R ELSE N/A

C012b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A

C012c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/3 THEN R ELSE N/A

C012d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/4 THEN R ELSE N/A

C012e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A

C012f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/31a THEN R ELSE N/A

C012z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C013 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C014 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C015 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/33 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C016 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016y IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/20 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C017 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C017b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/6 THEN R ELSE N/A

C017x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.9-1/1 THEN R ELSE N/A

C017y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/33 THEN R ELSE N/A

C018 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C019 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C019b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.2-1/6 THEN R ELSE N/A

C019x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.9-1/1 THEN R ELSE N/A

C020 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) THEN R ELSE N/A

C021 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C021a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C021b IF A.4.1-1/1 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C022 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C022a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C023 IF A.4.1-4/5 AND A.4.1-3/2 THEN R ELSE N/A

C023a IF A.4.1-4/5 AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C024 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C025 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 THEN R ELSE N/A

C025a IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C026 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND 4.3.6-1/11 THEN R ELSE N/A

C027 Void

C028 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.6-1/11 THEN R ELSE N/A

C029 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C030 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND 4.3.2-1/9 THEN R ELSE N/A

C030a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/9 THEN R ELSE N/A

C031 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C032 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C033 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/2 THEN R ELSE N/A

C034 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/6 THEN R ELSE N/A

C035 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/6 THEN R ELSE N/A

C036 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/3 THEN R ELSE N/A

C037 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C037a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C038 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C038a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C039 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C040 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C041 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C041a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C041b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C042 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C042a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C042b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C043 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (4.3.6-1/43 OR 4.3.6-1/44) THEN R ELSE N/A

C043a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C044 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.6-1/42 THEN R ELSE N/A

C045 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND (A.4.1-4/1 OR (A.4.1-4/3 AND A.4.3.2B.2.0-1A/2)) THEN R ELSE N/A

C046 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND (A.4.1-4/1 OR (A.4.1-4/3 AND A.4.3.2B.2.0-1A/3)) THEN R ELSE N/A

C047 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/4 AND (A.4.1-4/3 AND A.4.3.2B.2.0-1A/4) THEN R ELSE N/A

C048 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND A.4.1-4/1 THEN R ELSE N/A

C049 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND A.4.1-4/1 THEN R ELSE N/A

C050 IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/36 THEN R ELSE N/A

C051 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.3.2A.1-2/1 AND A.4.3.2-1/37 THEN R ELSE N/A

C052 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.11-1/1 THEN R ELSE N/A

C053 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/1 THEN R ELSE N/A

C054 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/2 THEN R ELSE N/A

C055 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/3 THEN R ELSE N/A

C056 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/4 THEN R ELSE N/A

C057 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/5 THEN R ELSE N/A

C058 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/6 THEN R ELSE N/A

C059 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/7 THEN R ELSE N/A

C060 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C061 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) THEN R ELSE N/A

C062 IF A.4.1-1/2 AND A.4.1.2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 THEN R ELSE N/A

C063 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A

C064 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/3 THEN R ELSE N/A

C064a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/4 THEN R ELSE N/A

C064b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/5 THEN R ELSE N/A

C065 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) THEN R ELSE N/A

C065a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C066 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) THEN R ELSE N/A

C066a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C067 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.1-3/2 THEN R ELSE N/A

C068 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND [10] A.4.6-1/1 AND A.4.1-3/2 THEN R ELSE N/A

C069 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/6 THEN R ELSE N/A

C070 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C071 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C072 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C073 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C074 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C075 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C076 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C077 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C078 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) AND A.4.1-4A/1 AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C079 IF A.4.1-1/3 AND A.4.1-2/7 THEN R ELSE N/A

C080 IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND A.4.1-1/2 AND A.4.1-2/8 THEN R ELSE N/A

C080a IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND A.4.1-1/2 AND A.4.1-2/8 AND A.4.3.5-1/1 THEN R ELSE N/A

C081 IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/5) AND (A.4.3.6-1/xx01 OR A.4.3.6-1/xx02) THEN R ELSE N/A

C081a IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/5) AND [10]A.4.4-1a/5 AND (A.4.3.6-1/xx01 OR A.4.3.6-1/xx02) THEN R ELSE N/A

C082 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/Z1 AND A.4.3.2-1/Z3 THEN R ELSE N/A

C082a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/Z1 AND A.4.3.2-1/Z3 AND A.4.3.5-1/1 THEN R ELSE N/A

C083 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/41 AND A.4.3.2-1/Z2 AND A.4.3.2-1/Z3 THEN R ELSE N/A

C083a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/41 AND A.4.3.2-1/Z2 AND A.4.3.2-1/Z3 AND A.4.3.5-1/1 THEN R ELSE N/A

C084 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/Z1 AND A.4.3.2-1/Z3 THEN R ELSE N/A

C084a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/Z1 AND A.4.3.2-1/Z3 AND A.4.3.5-1/1THEN R ELSE N/A

C085 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 AND A.4.3.2-1/Z2 AND A.4.3.2-1/Z3 THEN R ELSE N/A

C085a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 AND A.4.3.2-1/Z2 AND A.4.3.2-1/Z3 AND A.4.3.5-1/1 THEN R ELSE N/A

C086 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 THEN R ELSE N/A

C086a IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 AND A.4.3.5-1/1 THEN R ELSE N/A

NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25).

For the purposes of the present document, the tested bands selection criteria given in Table 4.0-2 apply. The ICS proformas used in Table 4.0-2 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-2: Tested Bands Selection Criteria

| Code | Tested Bands Selection Criteria | Comment |
| --- | --- | --- |
| D001 | (A.4.3.1-1 OR A.4.3.1-2) AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FR1 Bands without SUL/SDL bands |
| D002 | A.4.3.1-4 | All supported FR1 PC2 Bands |
| D003 | A.4.3.1-5 | All supported FR1 SUL Bands |
| D004 | {1,2,3,5,7,8,12,14,20,25,28,30,34,38,39,40,41,50,51,65,66,70,71,74,75,76} | UE supported bands among n1,n2,n3,n5,n7,n8,n12,n14,n20,n25,n28,n30,n34,n38,n39,n40,n41,n50,n51,n65,n66,n70,n71,n74,n75,n76 |
| D005 | A.4.3.1-3 | All supported FR2 Bands |
| D006 | A.4.3.1-1 OR A.4.3.1-2 | All supported FR1 Bands |
| D007 | A.4.3.1-1 OR A.4.3.1-2 OR A.4.3.1-3 | All supported NR Bands |
| D008 | ANY((A.4.3.1-1) AND 10MHz) | Any FDD FR1 band within the set supporting 10 MHz UE Channel BW |
| D009 | ANY((A.4.3.1-2) AND 20MHz) | Any TDD FR1 band within the set supporting 20 MHz UE Channel BW |
| D010 | ANY((A.4.3.1-2) AND 40MHz) | Any TDD FR1 band within the set supporting 40 MHz UE Channel BW |
| D011 | A.4.3.9-4a OR A.4.3.9-4b | All supported 4 Rx antenna ports Bands |
| D012 | A.4.3.1-1 AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FDD bands without SUL/SDL bands |
| D013 | ANY((A.4.3.1-3) AND 50MHz) | Any TDD FR2 band within the set supporting 50 MHz UE Channel BW |
| D014 | ANY((A.4.3.1-3) AND 100MHz) | Any TDD FR2 band within the set supporting 100 MHz UE Channel BW |
| D015 | ANY((A.4.3.1-3) AND 200MHz) | Any TDD FR2 band within the set supporting 200 MHz UE Channel BW |
| D016 | A.4.3.1-9 | All supported FR1 sidelink bands |
| D017 | {40, 41, 77, 78, 79} | UE supported TDD bands among n40, n41, n77, n78, n79 |
| NOTE 1: Band Selection is based on set theory. For each feature, item number shall correspond to the Band number. The result is the set of bands for which the test shall be conducted. The following operators are used: AND: Set intersection ( ). {1,2} AND {2,3} = {2} OR: Set union ( ). {1,2} OR {2,3} = {1,2,3} NOT: Set complement (\), full set being all bands. NOT{1} = {2 ...256} Also note that this is set without repetitions so {1} AND {1} = {1} The following basic sets are used: {1,2}: Explicitly given band set 10MHz: All bands supporting 10 MHz |  |  |

For the purposes of the present document, the tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The ICS proformas used in Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-3: Tested CA/DC Configuration Selection Criteria

| Code | Tested CA Configuration Selection Criteria | Comment |
| --- | --- | --- |
| E001 | DL_2CC(A.4.3.2A.2.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.2.1-2) | All supported intra-band contiguous CA Configurations with 2 carriers in DL but no CA in UL |
| E002 | DL_2CC(A.4.3.2A.4.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.4.1-2) | All supported inter-band CA Configurations with 2 carriers in DL but no CA in UL |
| E003 | UL_2CC(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band contiguous EN-DC configurations in FR1 (2UL CCs) |
| E003a | DL_2CC(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-1/1 | All supported Intra-band contiguous EN-DC configurations in FR1 (2DL CCs) |
| E004 | UL_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2UL CCs) |
| E004a | DL_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2DL CCs) |
| E005 | UL_2CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations within FR1 (2UL CCs) |
| E005a | DL_2CC(A.4.3.2B.2.3.1-2) AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations within FR1 (2DL CCs) |
| E005b | UL_NR_1CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-2A/1 | All supported Inter-band EN-DC configurations within FR1 with 1 NR UL CC and one or more LTE UL CC(s) |
| E005c | DL_NR_1CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/1 | All supported Inter-band EN-DC configurations within FR1 with 1 NR DL CC and one or more LTE DL CC(s) |
| E006 | DL_3CC(A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/2 | All supported EN-DC configurations within FR1 (3DL CCs) |
| E007 | DL_4CC(A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/3 | All supported EN-DC configurations within FR1 (4DL CCs) |
| E008 | DL_5CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/4 | All supported EN-DC configurations within FR1 (5DL CCs) |
| E009 | DL_6CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/5 | All supported EN-DC configurations within FR1 (6DL CCs) |
| E010 | UL_NR_1CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-2 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations including FR2 (1NR UL CC) |
| E010a | DL_NR_1CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-2 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations including FR2 (1NR DL CC) |
| E011 | UL_NR_2CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/2 AND A.4.3.2B.2.0-2A/2 | All supported Inter-band EN-DC configurations including FR2 (2UL NR CCs) |
| E011a | DL_NR_2CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/2 AND A.4.3.2B.2.0-1A/2 | All supported Inter-band EN-DC configurations including FR2 (2DL NR CCs) |
| E012 | UL_NR_3CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/3 AND NR_A.4.3.2B.2.0-2A/3 | All supported Inter-band EN-DC configurations including FR2 (3UL NR CCs) |
| E012a | DL_NR_3CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/3 AND A.4.3.2B.2.0-1A/3 | All supported Inter-band EN-DC configurations including FR2 (3DL NR CCs) |
| E013 | UL_NR_4CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/4 AND A.4.3.2B.2.0-2A/4 | All supported Inter-band EN-DC configurations including FR2 (4UL NR CCs) |
| E013a | DL_NR_4CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/4 AND A.4.3.2B.2.0-1A/4 | All supported Inter-band EN-DC configurations including FR2 (4DL NR CCs) |
| E014 | DL_NR_5CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/5 AND A.4.3.2B.2.0-1A/5 | All supported Inter-band EN-DC configurations including FR2 (5DL NR CCs) |
| E015 | UL_2CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-2/1 | All supported FR1 2UL CA configurations |
| E016 | DL_2CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3) AND A.4.3.2A.1-1/1 | All supported FR1 2DL CA configurations |
| E017 | DL_3CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-1/2 | All supported FR1 3DL CA configurations |
| E018 | DL_4CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-1/3 | All supported FR1 4DL CA configurations |
| E019 | ULTxSwitching(A.4.3.2A.4.1-3) | All supported FR1 2UL CA configurations with ULTxSwitching capability |
| E020 | UL_2CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/1 | All supported FR2 2UL CA configurations |
| E021 | UL_3CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/2 | All supported FR2 3UL CA configurations |
| E022 | UL_4CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/3 | All supported FR2 4UL CA configurations |
| E023 | UL_5CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/4 | All supported FR2 5UL CA configurations |
| E024 | UL_6CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/5 | All supported FR2 6UL CA configurations |
| E025 | UL_7CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/6 | All supported FR2 7UL CA configurations |
| E026 | UL_8CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/7 | All supported FR2 8UL CA configurations |
| E027 | DL_NR_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/2 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2DL NR CCs) |
| E028 | DL_NR_3CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/3 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (3DL NR CCs) |
| E028a | DL_NR_4CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/4 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (4DL NR CCs) |
| E028b | DL_NR_5CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/5 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (5DL NR CCs) |
| E029 | DL_NR_2CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/2 | All supported Inter-band EN-DC configurations within FR1 (2DL NR CCs) |
| E030 | DL_NR_3CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/3 | All supported Inter-band EN-DC configurations within FR1 (3DL NR CCs) |
| E030a | DL_NR_4CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/4 | All supported Inter-band EN-DC configurations within FR1 (4DL NR CCs) |
| E030b | DL_NR_5CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/5 | All supported Inter-band EN-DC configurations within FR1 (5DL NR CCs) |
| E031 | A.4.3.2C.3-2 | All supported FR1 intra-band contiguous 2DL CA with SUL in uplink Configurations |
| NOTE 1: UL(table_index) includes all supported CA Configurations where at least one UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 2: UL_nCC(table_index) includes all supported CA or DC Configurations where at least one n-carrier UL CA or DC configuration was declared in column "Supported CA Bandwidth Class(es) in UL" or “Supported EN-DC Bandwidth Class(es) in UL” in Table table_index in TS 38.508-2 [8]. NOTE 3: UL_NR_nCC(table_index) includes all supported DC Configurations where at least one DC configuration with n-carrier NR UL CA configuration was declared in column "Supported EN-DC Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 4: DL_nCC(table_index) includes all supported n-carrier CA/DC Configurations in Table table_index in TS 38.508-2 [8]. NOTE 5: DL_NR_nCC(table_index) includes all supported DC Configurations with n-carrier NR DL CA configuration in Table table_index in TS 38.508-2 [8]. NOTE 6: ULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported ULTxSwitching Band Pair" in Table table_index in TS 38.508-2 [8]. |  |  |

## 4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, supported CBW and SCS in the RF-Parameters IE (see TS 38.331 [11]) which conveys RF related capabilities for NR operation is taken into account.

### 4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 |  |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 has been executed. |
| 6.2.3 | UE additional maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 PC3 | D001 | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 6.5.2.3 and 6.5.3.3 are executed. Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.3 has been executed. |
| 6.2.4 | Configured transmitted power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC2 PC3 |  |
| 6.2A.1.1 | UE maximum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.2A.2.1 | UE maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  | Test execution is not necessary if TS 38.521-1 6.5A.2.4.1.1 is executed. |
| 6.2A.3.1 | UE additional maximum output power reduction for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  | Test execution is not necessary if TS 38.521-1 6.5A.2.3 and 6.5A.3.3 are executed. |
| 6.2A.4.1 | Configured transmitted power for CA (2UL CA) | Rel-15 | C004 | Ues supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.2C.1 | Configured transmitted power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.3 | UE maximum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.4 | UE maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.5 | UE additional maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2D.1 | UE maximum output power for UL MIMO | Rel-15 | N/A | Not recommended due to no test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4. | D001 |  | Maximum Output Power for UL MIMO is tested as part of the MPR test case with using MPR=1.5dB suggested by RAN4. |
| 6.2D.2 | UE maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  | Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed. |
| 6.2D.3 | UE additional maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  | Test execution is not necessary if TS 38.521-1 6.5D.2.3 and 6.5D.3.3 are executed. |
| 6.2D.4 | Configured transmitted power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.2E.2.1 | UE maximum output power for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |
| 6.3.1 | Minimum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3.3.2 | General ON/OFF time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.3.3.2 if UE supports NSA and TS 38.521-3 TC 6.3B.3.1 or 6.3B.3.2 or 6.3B.3.3 has been executed. |
| 6.3.3.4 | PRACH time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.3.3.4 if UE supports NSA and TS 38.521-3 TC 6.3B.4.1 or 6.3B.4.2 or 6.3B.4.3 has been executed. |
| 6.3.3.6 | SRS time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3.4.2 | Absolute power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3.4.3 | Relative power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3.4.4 | Aggregate power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3A.1.1 | Minimum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.3.1 | Transmit ON/OFF time mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.3.1_1 | Time mask for switching between two uplink carriers | Rel-16 | C051 | UEs supporting 5GS FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching | E019 |  | NOTE 1 |
| 6.3A.4.1 | Absolute power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.4.2 | Power control relative power tolerane for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.4.3 | Aggregate power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3C.1 | Minimum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.3 | Transmit ON/OFF time mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.4.1 | Absolute power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.4.2 | Relative power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.4.3 | Aggregate power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3D.1 | Minimum output power for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.3D.3 | Transmit ON/OFF time mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.3D.4.1 | Absolute power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.3D.4.2 | Relative power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.3D.4.3 | Aggregate power tolerance for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.3E.1.1 | Minimum output power for V2X / non-concurrent operation | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  | NOTE 1 |
| 6.4.1 | Frequency error | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.1 or 6.4B.1.2 or 6.4B.1.3 has been executed. |
| 6.4.2.1 | Error vector magnitude | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.1 or 6.4B.2.2.1 or 6.4B.2.3.1 has been executed. |
| 6.4.2.2 | Carrier leakage | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.2 or 6.4B.2.2.2 or 6.4B.2.3.2 has been executed. |
| 6.4.2.3 | In-band emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.4B.2.2.3 or 6.4B.2.3.3 has been executed. |
| 6.4.2.4 | EVM equalizer spectrum flatness | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.4 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.4 or 6.4B.2.2.4 or 6.4B.2.3.4 has been executed. |
| 6.4.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK | Rel-15 | C050 | UEs supporting 5GS FR1 and pi/2-BPSK modulation scheme for power boosting in FR1 | D017 |  |  |
| 6.4A.1.1 | Frequency error for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4A.2.1.1 | Error vector magnitude for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4C.1 | Frequency error for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.1 | Error vector magnitude for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.2 | Carrier leakage for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.3 | In-band emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.4 | EVM equalizer spectrum flatness for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK for SUL | Rel-16 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 1 |
| 6.4D.1 | Frequency error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.4D.2.1 | Error vector magnitude for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.4D.2.2 | Carrier leakage for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.4D.2.3 | In-band emissions for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.4D.2.4 | EVM equalizer spectrum flatness for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.4D.3 | Time alignment error for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.4D.4 | Requirements for coherent UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4E.2.2.1 | Error Vector Magnitude for V2X / non-concurrent operation | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4E.2.2.1D | Error Vector Magnitude for V2X / non-concurrent operation / SL-MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4E.2.4.1 | In-band emissions for V2X / non-concurrent operation | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4E.2.4.1D | In-band emissions for V2X / non-concurrent operation / SL-MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.5.1 | Occupied bandwidth | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.2 or 6.5B.1.3 has been executed. |
| 6.5.2.2 | Spectrum emission mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | Skip TC 6.5.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.1 or 6.5B.2.3.1 has been executed. |
| 6.5.2.3 | Additional spectrum emission mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | NOTE 1 Skip TC 6.5.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.2 has been executed. |
| 6.5.2.4.1 | NR ACLR | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | Skip TC 6.5.2.4.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.3 or 6.5B.2.3.3 has been executed. |
| 6.5.2.4.2 | UTRA ACLR | Rel-15 | C001a | UEs supporting 5GS FR1 PC3 | D001 |  |  |
| 6.5.3.1 | General spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.1.1 or 6.5B.3.2.1 has been executed. |
| 6.5.3.2 | Spurious emissions for UE co-existence | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.5.3.3 | Additional spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.3 has been executed. |
| 6.5.4 | Transmit intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.4 if UE supports NSA and TS 38.521-3 TC 6.5B.5.3 has been executed. |
| 6.5A.1.1 | Occupied bandwidth for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.2.2.1 | Spectrum emission mask for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.2.4.1.1 | NR ACLR for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.2.4.2.1 | UTRA ACLR for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.3.2.1 | Spurious emissions for UE co-existence for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.4.1 | Transmit intermodulation for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5C.1 | Occupied bandwidth for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.2 | Spectrum emission mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.3 | Additional spectrum emission mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.4.1 | NR ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.4.2 | UTRA ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.3.1 | General spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.3.2 | Spurious emissions for UE co-existence for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.3.3 | Additional spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.4 | Transmit intermodulation for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5D.1 | Occupied bandwidth for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.1_1 | Occupied bandwidth for UL MIMO (Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.2.2 | Spectrum Emission Mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.2.3 | Additional spectrum emission mask for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.2.4.1 | NR ACLR for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.2.4.2 | UTRA ACLR for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.2_1.4.2 | UTRA ACLR for UL MIMO (Rel-16 onward) | Rel-16 | FFS | FFS | FFS |  | NOTE 1 |
| 6.5D.3.1 | General spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.3.2 | Spurious emissions for UE co-existence for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 6.5D.3_1.1 | General spurious emissions for UL MIMO (Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.3_1.2 | Spurious emission for UE co-existence for UL MIMO (Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.3_1.3 | Additional spurious emissions for UL MIMO(Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.4 | Transmit intermodulation for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 |  |  |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | 2Rx 4Rx |  |
| 7.3A.1 | Reference sensitivity power level for 2DL CA without exception | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.3A.1_1 | Reference sensitivity power level for 2DL CA exceptions | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.3A.2 | Reference sensitivity level for CA (3DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.3 | Reference sensitivity level for CA (4DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3C.2 | Reference sensitivity power level for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.3D.2 | Reference sensitivity power level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.4 | Maximum input level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.4 if UE supports NSA and TS 38.521-3 TC 7.4B.3 or 7.4B.4 has been executed. |
| 7.4A.1 | Maximum input level for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.4A.2 | Maximum input level for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.4A.3 | Maximum input level for CA (4DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4D | Maximum input level for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.5 | Adjacent channel selectivity | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | NOTE 1 Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.2 or 7.5B.3 has been executed. |
| 7.5A.1 | Adjacent channel selectivity for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.5A.2 | Adjacent channel selectivity for 3DL CA | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5A.3 | Adjacent channel selectivity for 4DL CA | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5D | Adjacent channel selectivity for UL MIMO | Rel-15 | C003a | UEs supprtoing 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.6.2 | In-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.2 or 7.6B.2.3 has been executed. |
| 7.6.3 | Out-of-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 7.6.4 | Narrow band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D004 |  | Skip TC 7.6.4 if UE supports NSA and TS 38.521-3 TC 7.6B.4.2 or 7.6B.4.3 has been executed. |
| 7.6A.2.1 | In-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.2.2 | In-band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.2.3 | In-band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | NOTE 1 Skip TC 7.6A.2.3 if UE supports NSA and TS 38.521-3 TC 7.6B.2.3_1.3 has been executed. |
| 7.6A.3.1 | Out-of-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.3.2 | Out-of-band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.3.3 | Out-of-band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |
| 7.6A.4.1 | Narrow band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.4.2 | Narrow band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.4.3 | Narrow band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | Skip TC 7.6A.4.3 if UE supports NSA and TS 38.521-3 TC 7.6B.4.3_1.3 has been executed. |
| 7.6C.2 | In-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.6C.2_1 | Inband Blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |
| 7.6C.3 | Out-of-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.6C.3_1 | Out-of-band blocking for SUL with DL CA | Rel-17 | C078 | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E031 |  |  |
| 7.6D.2 | In-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.6D.3 | Out-of-band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.6D.4 | Narrow band blocking for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.7 | Spurious response | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 7.7A.1 | Spurious response for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.7A.2 | Spurious response for CA (3DL CA) | Rel-16 | FFS | UEs supporting 5GS FR1 and CA (3DL CA) | FFS |  | NOTE 1 |
| 7.7A.3 | Spurious response for CA (4DL CA) | Rel-16 | FFS | UEs supporting 5GS FR1 and CA (4DL CA) | FFS |  | NOTE 1 |
| 7.7D | Spurious response for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.8.2 | Wide band Intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.8.2 if UE supports NSA and TS 38.521-3 TC 7.8B.2.2 or 7.8B.2.3 has been executed. |
| 7.8A.2.1 | Wide band Intermodulation for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.8A.2.2 | Wide band Intermodulation for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.8A.2.3 | Wide band Intermodulation for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |
| 7.8D.2 | Wide band Intermodulation for UL MIMO | Rel-15 | C003a | UEs supporting 5GS FDD FR1 and UL MIMO | D012 |  |  |
| 7.9 | Spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.1 or 7.9B.2 or 7.9B.3 has been executed. |
| 7.9A.1 | Spurious emissions for CA (2DL CA) | Rel-15 | C005 | UEs supporting 5GS FR1 and inter-band 2DL CA with a DL-only band | E002 |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-1. |  |  |  |  |  |  |  |

Table 4.1.1-1a: Void

Table 4.1.1-1b: Void

Table 4.1.1-1c: Void

### 4.1.2	FR2 standalone conformance test cases

Table 4.1.2-1: Applicability of RF SA FR2 conformance test cases, ref. TS 38.521-2 [2]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1.1 | UE maximum output power - EIRP and TRP | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | Skip TC 6.2.1.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4.1 has been executed. |
| 6.2.1.2 | UE maximum output power - Spherical coverage | Rel-15 | C007 | UEs supporting 5GS FR2 and beam correspondence without UL beam sweeping | D005 | PC1 PC2 PC3 PC4 | Skip TC 6.2.1.2 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4.2 has been executed. |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.4 has been executed. |
| 6.2.3 | UE maximum output power with additional requirements | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.4 has been executed. |
| 6.2A.1.1.1 | UE maximum output power - EIRP and TRP for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | Skip TC 6.2A.1.1.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.1.1 has been executed. |
| 6.2A.1.1.2 | UE maximum output power - EIRP and TRP for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | Skip TC 6.2A.1.1.2 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.2.1 has been executed. |
| 6.2A.1.1.3 | UE maximum output power - EIRP and TRP for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | Skip TC 6.2A.1.1.3 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.3.1 has been executed. |
| 6.2A.1.1.4 | UE maximum output power - EIRP and TRP for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.1.1.5 | UE maximum output power - EIRP and TRP for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.1.1.6 | UE maximum output power - EIRP and TRP for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.1.1.7 | UE maximum output power - EIRP and TRP for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.1.2.1 | UE maximum output power - Spherical coverage for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | Skip TC 6.2A.1.2.1 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.1.2 has been executed. |
| 6.2A.1.2.2 | UE maximum output power - Spherical coverage for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | Skip TC 6.2A.1.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.2.2 has been executed. |
| 6.2A.1.2.3 | UE maximum output power - Spherical coverage for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | Skip TC 6.2A.1.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.1.4_1.3.2 has been executed. |
| 6.2A.1.2.4 | UE maximum output power - Spherical coverage for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.1.2.5 | UE maximum output power - Spherical coverage for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.1.2.6 | UE maximum output power - Spherical coverage for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.1.2.7 | UE maximum output power - Spherical coverage for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2A.2.1 | UE maximum output power reduction for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3.1 | Minimum output power | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | Skip TC 6.3.1 if UE supports NSA and TS 38.521-3 TC 6.3B.1.4 has been executed. |
| 6.3.2 | Transmit OFF power | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  |  |
| 6.3.3.2 | General ON/OFF time mask | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.3.3.4 | PRACH time mask | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.3.4.2 | Absolute power tolerance | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.3.4.3 | Relative power tolerance | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.3.4.4 | Aggregate power tolerance | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.3A.1.1 | Minimum output power for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3A.1.2 | Minimum output power for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3A.1.3 | Minimum output power for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3A.1.4 | Minimum output power for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3A.1.5 | Minimum output power for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3A.1.6 | Minimum output power for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3A.1.7 | Minimum output power for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3A.2.1 | Void |  |  |  |  |  |  |
| 6.3A.2.2 | Void |  |  |  |  |  |  |
| 6.3A.2.3 | Void |  |  |  |  |  |  |
| 6.3A.3.1.1 | General ON/OFF time mask for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 |
| 6.3A.4.2.1 | Absolute power tolerance for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 |
| 6.3A.4.2.2 | Absolute power tolerance for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 |
| 6.3A.4.2.3 | Absolute power tolerance for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 |
| 6.3A.4.2.4 | Absolute power tolerance for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.3A.4.2.5 | Absolute power tolerance for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.3A.4.2.6 | Absolute power tolerance for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.3A.4.2.7 | Absolute power tolerance for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.3A.4.4.1 | Aggregate power tolerance for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 |
| 6.3A.4.4.2 | Aggregate power tolerance for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 |
| 6.3A.4.4.3 | Aggregate power tolerance for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 |
| 6.3A.4.4.4 | Aggregate power tolerance for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.3A.4.4.5 | Aggregate power tolerance for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.3A.4.4.6 | Aggregate power tolerance for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.3A.4.4.7 | Aggregate power tolerance for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.3D.1 | Minimum output power for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.3D.2 | Transmit OFF power for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.3D.3.1 | General ON/OFF time mask for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.3D.3.4 | SRS time mask for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.4.1 | Frequency error | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.4 has been executed. |
| 6.4.2.1 | Error vector magnitude | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 6.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.1 has been executed. |
| 6.4.2.2 | Carrier leakage | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 6.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.2 has been executed. |
| 6.4.2.3 | In-band emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | NOTE 1 Skip TC 6.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.3 has been executed. |
| 6.4.2.4 | EVM equalizer spectrum flatness | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 Skip TC 6.4.2.4 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.4 has been executed. |
| 6.4.2.5 | EVM spectral flatness for pi/2 BPSK modulation | Rel-15 | C006b | UEs supporting 5GS FR2 and pi/2 BPSK modulation | D005 |  | NOTE 1 Skip TC 6.4.2.5 if UE supports NSA and TS 38.521-3 TC 6.4B.2.4.5 has been executed. |
| 6.4A.1.1 | Frequency error for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 |
| 6.4A.1.2 | Frequency error for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 |
| 6.4A.1.3 | Frequency error for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 |
| 6.4A.2.1.1 | Error vector magnitude for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.2 | Error vector magnitude for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.3 | Error vector magnitude for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.4 | Error vector magnitude for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.5 | Error vector magnitude for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.6 | Error vector magnitude for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.1.7 | Error vector magnitude for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.2 | Carrier leakage for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.3 | Carrier leakage for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.4 | Carrier leakage for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.5 | Carrier leakage for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.6 | Carrier leakage for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.2.7 | Carrier leakage for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.2 | In-band emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.3 | In-band emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.4 | In-band emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.5 | In-band emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.6 | In-band emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4A.2.3.7 | In-band emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.4D.1 | Frequency error for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.4D.3 | Time alignment error for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.5.1 | Occupied bandwidth | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4 has been executed. |
| 6.5.2.1 | Spectrum Emission Mask | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.2.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1 has been executed. |
| 6.5.2.3 | Adjacent channel leakage ratio | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3 has been executed. |
| 6.5.3.1 | Transmitter Spurious emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1 has been executed. |
| 6.5.3.2 | Spurious emission band UE co-existence | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 6.5.3.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.2 has been executed. |
| 6.5.3.3 | Additional spurious emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 |
| 6.5A.1.1 | Occupied bandwidth for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 Skip TC 6.5A.1.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4_1.1 has been executed. |
| 6.5A.1.2 | Occupied bandwidth for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 Skip TC 6.5A.1.2 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4_1.2 has been executed. |
| 6.5A.1.3 | Occupied bandwidth for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 Skip TC 6.5A.1.3 if UE supports NSA and TS 38.521-3 TC 6.5B.1.4_1.3 has been executed. |
| 6.5A.1.4 | Occupied bandwidth for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.1.5 | Occupied bandwidth for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.1.6 | Occupied bandwidth for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.1.7 | Occupied bandwidth for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.2.1.1 | Spectrum Emission Mask for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | Skip TC 6.5A.2.1.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1_1.1 has been executed. |
| 6.5A.2.1.2 | Spectrum Emission Mask for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 Skip TC 6.5A.2.1.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1_1.2 has been executed. |
| 6.5A.2.1.3 | Spectrum Emission Mask for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 Skip TC 6.5A.2.1.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.1_1.3 has been executed. |
| 6.5A.2.1.4 | Spectrum Emission Mask for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.2.1.5 | Spectrum Emission Mask for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.2.1.6 | Spectrum Emission Mask for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.2.1.7 | Spectrum Emission Mask for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.2.2.1 | Adjacent channel leakage ratio for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | Skip TC 6.5A.2.2.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3_1.1 has been executed. |
| 6.5A.2.2.2 | Adjacent channel leakage ratio for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | Skip TC 6.5A.2.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3_1.2 has been executed. |
| 6.5A.2.2.3 | Adjacent channel leakage ratio for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | Skip TC 6.5A.2.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.4.3_1.3 has been executed. |
| 6.5A.2.2.4 | Adjacent channel leakage ratio for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.2.2.5 | Adjacent channel leakage ratio for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.2.2.6 | Adjacent channel leakage ratio for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.2.2.7 | Adjacent channel leakage ratio for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 and CA (2UL CA) | E020 |  | NOTE 1 Skip TC 6.5A.3.1.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.1 has been executed. |
| 6.5A.3.1.2 | Generalspurious emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 Skip TC 6.5A.3.1.2 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.2 has been executed. |
| 6.5A.3.1.3 | Generalspurious emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 Skip TC 6.5A.3.1.3 if UE supports NSA and TS 38.521-3 TC 6.5B.3.4.1_1.3 has been executed. |
| 6.5A.3.1.4 | Transmitter Spurious emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.3.1.5 | Transmitter Spurious emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.3.1.6 | Transmitter Spurious emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.3.1.7 | Transmitter Spurious emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.3.2.1 | Spurious emission band UE co-existence for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 |  | NOTE 1 |
| 6.5A.3.2.2 | Spurious emission band UE co-existence for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 |
| 6.5A.3.2.3 | Spurious emission band UE co-existence for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 |
| 6.5A.3.2.4 | Spurious emission band UE co-existence for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.3.2.5 | Spurious emission band UE co-existence for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.3.2.6 | Spurious emission band UE co-existence for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.3.2.7 | Spurious emission band UE co-existence for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5A.3.3.1 | Additional spurious emissions for CA (2UL CA) | Rel-15 | C053 | UEs supporting 5GS FR2 CA (2UL CA) | E020 |  | NOTE 1 |
| 6.5A.3.3.2 | Additional spurious emissions for CA (3UL CA) | Rel-15 | C054 | UEs supporting 5GS FR2 and CA (3UL CA) | E021 |  | NOTE 1 |
| 6.5A.3.3.3 | Additional spurious emissions for CA (4UL CA) | Rel-15 | C055 | UEs supporting 5GS FR2 and CA (4UL CA) | E022 |  | NOTE 1 |
| 6.5A.3.3.4 | Additional spurious emissions for CA (5UL CA) | Rel-15 | C056 | UEs supporting 5GS FR2 and CA (5UL CA) | E023 |  | NOTE 1 |
| 6.5A.3.3.5 | Additional spurious emissions for CA (6UL CA) | Rel-15 | C057 | UEs supporting 5GS FR2 and CA (6UL CA) | E024 |  | NOTE 1 |
| 6.5A.3.3.6 | Additional spurious emissions for CA (7UL CA) | Rel-15 | C058 | UEs supporting 5GS FR2 and CA (7UL CA) | E025 |  | NOTE 1 |
| 6.5A.3.3.7 | Additional spurious emissions for CA (8UL CA) | Rel-15 | C059 | UEs supporting 5GS FR2 and CA (8UL CA) | E026 |  | NOTE 1 |
| 6.5D.1 | Occupied bandwidth for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.5D.2.1 | Spectrum Emission Mask for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.5D.2.2 | Adjacent channel leakage ratio for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.5D.3.1 | Transmitter Spurious emissions for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.5D.3.2 | Spurious emission band UE co-existence for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO | Rel-15 | C060 | UEs supporting 5GS FR2 and UL MIMO | D005 |  | NOTE 1 |
| 6.6.1 | Beam correspondence - EIRP | Rel-15 | C008 | UEs supporting 5GS FR2 and not beam correspondence without UL beam sweeping | D005 |  |  |
| 6.6.2 | Enhanced Beam correspondence - EIRP | Rel-15 | TBD | TBD |  |  | NOTE 1 |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 | PC1 PC2 PC3 PC4 | Skip TC 7.3.2 if UE supports NSA and TS 38.521-3 TC 7.3B.2.4 has been executed. |
| 7.3A.2.1 | Reference sensitivity power level for CA (2DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.2.2 | Reference sensitivity power level for CA (3DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.2.3 | Reference sensitivity power level for CA (4DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.2.4 | Reference sensitivity power level for CA (5DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.2.5 | Reference sensitivity power level for CA (6DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.2.6 | Reference sensitivity power level for CA (7DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.2.7 | Reference sensitivity power level for CA (8DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.3.4 | EIS spherical coverage | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 7.3.4 if UE supports NSA and TS 38.521-3 TC 7.3B.4 has been executed. |
| 7.4 | Maximum input level | Rel-15 | N/A | not recommended due to testability issues | N/A |  | NOTE 1 |
| 7.4A.1 | Maximum input level for CA (2DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.4A.2 | Maximum input level for CA (3DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.4A.3 | Maximum input level for CA (4DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.4A.4 | Maximum input level for CA (5DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.4A.5 | Maximum input level for CA (6DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.4A.6 | Maximum input level for CA (7DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.4A.7 | Maximum input level for CA (8DL CA) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 7.5 | Adjacent channel selectivity | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.4.1 has been executed. |
| 7.6.2 | In-band blocking | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | NOTE 1 Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.4 has been executed. |
| 7.9 | Spurious emissions | Rel-15 | C006 | UEs supporting 5GS FR2 | D005 |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.4 has been executed. |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band/CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-2. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. |  |  |  |  |  |  |  |

Table 4.1.2-1a: Void

Table 4.1.2-1b: Void

Table 4.1.2-1c: Void

### 4.1.3	NR interworking between NR FR1 and NR FR2 and between NR and LTE conformance test cases

Table 4.1.3-1: Applicability of RF EN-DC FR1 and FR2 conformance test cases, ref. TS 38.521-3 [3]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA/DC-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2B | Transmitter Characteristics for DC |  |  |  |  |  |  |
| 6.2B.1.1 | UE Maximum Output Power for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  | NOTE 1 |
| 6.2B.1.2 | UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010 | UEs supporting Intra-Band non-contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.2B.1.3 | UE Maximum Output Power for Inter-Band EN-DC within FR1 | Rel-15 | C011 | UEs supporting Inter-Band EN-DC within FR1 (2UL CCs) | E005 | PC3 PC2 |  |
| 6.2B.1.4.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TRP | Rel-15 | C012 | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC | E010 | PC1 PC2 PC3 PC4 | NOTE 5 Skip TC 6.2B.1.4.1 if UE supports SA and TSC 38.521-2 TC 6.2.1.1 has been executed. |
| 6.2B.1.4.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC)- Spherical Coverage | Rel-15 | C012 | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC | E010 | PC1 PC2 PC3 PC4 | NOTE 5 Skip TC 6.2B.1.4.2 if UE supports SA and TSC 38.521-2 TC 6.2.1.2 has been executed. |
| 6.2B.1.4_1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.2B.1.4_1.1.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP | Rel-15 | FFS | FFS | FFS | PC1 PC2 PC3 PC4 | NOTE 1 NOTE 5 Skip TC 6.2B.1.4_1.1.1 if UE supports SA and TS 38.521-2 TC 6.2A.1.1.1 has been executed. |
| 6.2B.1.4_1.1.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage | Rel-15 | FFS | FFS | FFS | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2B.1.4_1.2.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP | Rel-15 | FFS | FFS | FFS | PC1 PC2 PC3 PC4 | NOTE 1 NOTE 5 Skip TC 6.2B.1.4_1.2.1 if UE supports SA and TS 38.521-2 TC 6.2A.1.1.2 has been executed. |
| 6.2B.1.4_1.2.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - Spherical Coverage | Rel-15 | FFS | FFS | FFS | PC1 PC2 PC3 PC4 | NOTE 1 |
| 6.2B.1.4_1.3.1 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP | Rel-15 | FFS | FFS | FFS | PC1 PC2 PC3 PC4 | NOTE 1 NOTE 5 Skip TC 6.2B.1.4_1.3.1 if UE supports SA and TS 38.521-2 TC 6.2A.1.1.3 has been executed. |
| 6.2B.1.4_1.3.2 | UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - Spherical Coverage | Rel-15 | FFS | FFS | FFS | PC1 PC2 PC3 PC4 | NOTE 1 NOTE 5 Skip TC 6.2B.1.4_1.3.1 if UE supports SA and TS 38.521-2 TC 6.2A.1.2.3 has been executed. |
| 6.2B.1.4D | UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.2B.2.1 | UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC | Rel-15 | C009z | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) and modified MPR behaviour | E003 |  |  |
| 6.2B.2.2 | UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010z | UEs supporting Intra-Band non-contiguous EN-DC (2UL CCs) and modified MPR behaviour | E004 |  |  |
| 6.2B.2.3 | UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC) | Rel-15 | C011z | UEs supporting Inter-Band EN-DC within FR1 with 1 NR UL CC and modified MPR behaviour | E005b | PC3 PC2 | NOTE 5 Test execution is not necessary if TS 38.521-3 6.5B.2.3.1 and 6.5B.2.3.3 is executed. Skip TC 6.2B.2.3 if UE supports SA and TS 38.521-1 TC 6.2.2 has been executed. |
| 6.2B.2.4 | UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012z | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC and modified MPR behaviour | E010 |  | NOTE 1 NOTE 5 Skip TC 6.2B.2.4 if UE supports SA and TS 38.521-2 TC 6.2.2 has been executed. |
| 6.2B.3.1 | UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC | Rel-15 | C009z | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) and modified MPR behaviour | E003 |  |  |
| 6.2B.3.2 | UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 6.2B.3.3 | UE Additional Maximum Output power reduction for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011z | UEs supporting Inter-Band EN-DC within FR1 with 1 NR UL CC and modified MPR behaviour | E005b | PC3 PC2 | NOTE 5 Test execution is not necessary if TS 38.521-3 6.5B.2.3.2 is executed. Skip TC 6.2B.3.3 if UE supports SA and TS 38.521-1 TC 6.2.3 has been executed. |
| 6.2B.3.4 | UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012z | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC and modified MPR behaviour | E010 | PC3 PC2 | NOTE 5 Skip TC 6.2B.3.4 if UE supports SA and TS 38.521-2 TC 6.2.3 has been executed. |
| 6.2B.4.1.1 | Configured Output Power Level for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  | NOTE 1 |
| 6.2B.4.1.2 | Configured Output Power for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010 | UEs supporting Intra-Band Non-Contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 |
| 6.2B.4.1.3 | Configured Output Power for Inter-Band EN-DC within FR1 | Rel-15 | C011 | UEs supporting Inter-Band EN-DC within FR1 (2UL CCs) | E005 |  |  |
| 6.2B.4.1.4 | Configured Output Power for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC | E005 |  |  |
| 6.3B | Output power dynamics for DC |  |  |  |  |  |  |
| 6.3B.1.1 | Minimum Output power for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.3B.1.2 | Minimum output power for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.1.2 if UE supports SA and TS 38.521-1 TC 6.3.1 has been executed. |
| 6.3B.1.3 | Minimum output power for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.1.3 if UE supports SA and TS 38.521-1 TC 6.3.1 has been executed. |
| 6.3B.1.4 | Minimum Output Power for EN-DC Interband including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.3B.1.4 if UE supports SA and TS 38.521-2 TC 6.3.1 has been executed. |
| 6.3B.1.4_1 | Minimum output power for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.3B.1.4_1.1 | Minimum output power for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.3B.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.3A.1.1 has been executed. |
| 6.3B.1.4_1.2 | Minimum output power for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.3B.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.3A.1.2 has been executed. |
| 6.3B.1.4_1.3 | Minimum output power for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.3B.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.3A.1.3 has been executed. |
| 6.3B.1.4D | Minimum output power for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.3B.1.4 if UE supports SA and TS 38.521-2 TC 6.3D.1 has been executed. |
| 6.3B.2.4 | Transmit OFF Power for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting inter-band EN-DC including FR2 with 1 NR UL CC | E005 |  |  |
| 6.3B.2.4_1.1 | Void |  |  |  |  |  |  |
| 6.3B.2.4_1.2 | Void |  |  |  |  |  |  |
| 6.3B.2.4_1.3 | Void |  |  |  |  |  |  |
| 6.3B.3.1 | Tx ON/OFF time mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.3.1 if UE supports SA and TS 38.521-1 TC 6.3.3.2 has been executed. |
| 6.3B.3.2 | Tx ON/OFF time mask for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.3.2 if UE supports SA and TS 38.521-1 TC 6.3.3.2 has been executed. |
| 6.3B.3.3 | Tx ON/OFF time mask for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.3.3 if UE supports SA and TS 38.521-1 TC 6.3.3.2 has been executed. |
| 6.3B.3.4 | Tx ON/OFF time mask for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 6.3B.4.1 | PRACH time mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.4.1 if UE supports SA and TS 38.521-1 TC 6.3.3.4 has been executed. |
| 6.3B.4.2 | PRACH Time Mask for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.4.2 if UE supports SA and TS 38.521-1 TC 6.3.3.4 has been executed. |
| 6.3B.4.3 | PRACH Time Mask for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.4.3 if UE supports SA and TS 38.521-1 TC 6.3.3.4 has been executed. |
| 6.3B.8.1.1 | Absolute Power Tolerance for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2 UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.8.1.1 if UE supports SA and TS 38.521-1 TC 6.3.4.2 has been executed. |
| 6.3B.8.1.2 | Absolute Power Tolerance for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2 UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.8.1.2 if UE supports SA and TS 38.521-1 TC 6.3.4.2 has been executed. |
| 6.3B.8.1.3 | Absolute Power Tolerance for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.1.3 if UE supports SA and TS 38.521-1 TC 6.3.4.2 has been executed. |
| 6.3B.8.1.4 | Absolute Power Tolerance for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 6.3B.8.2.1 | Relative Power Tolerance for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2 UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.8.2.1 if UE supports SA and TS 38.521-1 TC 6.3.4.3 has been executed. |
| 6.3B.8.2.2 | Relative Power Tolerance for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2 UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.8.2.2 if UE supports SA and TS 38.521-1 TC 6.3.4.3 has been executed. |
| 6.3B.8.2.3 | Relative Power Tolerance for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.2.3 if UE supports SA and TS 38.521-1 TC 6.3.4.3 has been executed. |
| 6.3B.8.2.4 | Relative Power Tolerance for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 6.3B.8.3.1 | Aggregate Power Tolerance for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2 UL CCs) | E003 |  | NOTE 5 Skip TC 6.3B.8.3.1 if UE supports SA and TS 38.521-1 TC 6.3.4.4 has been executed. |
| 6.3B.8.3.2 | Aggregate Power Tolerance for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2 UL CCs) | E004 |  | NOTE 5 Skip TC 6.3B.8.3.2 if UE supports SA and TS 38.521-1 TC 6.3.4.4 has been executed. |
| 6.3B.8.3.3 | Aggregate Power Tolerance for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.3B.8.3.3 if UE supports SA and TS 38.521-1 TC 6.3.4.4 has been executed. |
| 6.3B.8.3.4 | Aggregate Power Tolerance for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B | Transmit Signal Quality for DC |  |  |  |  |  |  |
| 6.4B.1.1 | Frequency Error for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.1.1 if UE supports SA and TS 38.521-1 TC 6.4.1 has been executed. |
| 6.4B.1.2 | Frequency Error for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.1.2 if UE supports SA and TS 38.521-1 TC 6.4.1 has been executed. |
| 6.4B.1.3 | Frequency error for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.1.3 if UE supports SA and TS 38.521-1 TC 6.4.1 has been executed. |
| 6.4B.1.4 | Frequency Error for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-Band EN-DC including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.4B.1.4 if UE supports SA and TS 38.521-2 TC 6.4.1 has been executed. |
| 6.4B.1.4_1 | Frequency Error for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.4B.1.4_1.1 | Frequency Error for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 |
| 6.4B.1.4_1.2 | Frequency Error for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 |
| 6.4B.1.4_1.3 | Frequency Error for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 |
| 6.4B.2.1.1 | Error Vector Magnitude for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.2.1.1 if UE supports SA and TS 38.521-1 TC 6.4.2.1 has been executed. |
| 6.4B.2.1.2 | Carrier Leakage for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.2.1.2 if UE supports SA and TS 38.521-1 TC 6.4.2.2 has been executed. |
| 6.4B.2.1.3 | In-band Emissions for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.4B.2.1.4 | EVM Equalizer Flatness for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.4B.2.1.4 if UE supports SA and TS 38.521-1 TC 6.4.2.4 has been executed. |
| 6.4B.2.2.1 | Error Vector Magnitude for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.1 if UE supports SA and TS 38.521-1 TC 6.4.2.1 has been executed. |
| 6.4B.2.2.2 | Carrier Leakage for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.2 if UE supports SA and TS 38.521-1 TC 6.4.2.2has been executed. |
| 6.4B.2.2.3 | In-band Emissions for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.3 if UE supports SA and TS 38.521-1 TC 6.4.2.3 has been executed. |
| 6.4B.2.2.4 | EVM Equalizer Flatness for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.4B.2.2.4 if UE supports SA and TS 38.521-1 TC 6.4.2.4 has been executed. |
| 6.4B.2.3.1 | Error Vector Magnitude for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.1 if UE supports SA and TS 38.521-1 TC 6.4.2.1 has been executed. |
| 6.4B.2.3.2 | Carrier Leakage for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.2 if UE supports SA and TS 38.521-1 TC 6.4.2.2 has been executed. |
| 6.4B.2.3.3 | In-band Emissions for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.3 if UE supports SA and TS 38.521-1 TC 6.4.2.3 has been executed. |
| 6.4B.2.3.4 | EVM Equalizer Flatness for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.4B.2.3.4 if UE supports SA and TS 38.521-1 TC 6.4.2.4 has been executed. |
| 6.4B.2.4.1 | Error Vector Magnitude for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1 if UE supports SA and TS 38.521-2 TC 6.4.2.1 has been executed. |
| 6.4B.2.4.1_1 | Error Vector Magnitude for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.4B.2.4.1_1.1 | Error Vector Magnitude for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1_1.1 if UE supports SA and TS 38.521-2 TC 6.4A.2.1.1 has been executed. |
| 6.4B.2.4.1_1.2 | Error Vector Magnitude for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.4A.2.1.2 has been executed. |
| 6.4B.2.4.1_1.3 | Error Vector Magnitude for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.4A.2.1.3 has been executed. |
| 6.4B.2.4.1D | Error Vector Magnitude for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.2 | Carrier Leakage for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.2 if UE supports SA and TS 38.521-2 TC 6.4.2.2 has been executed. |
| 6.4B.2.4.2_1 | Carrier Leakage for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.4B.2.4.2_1.1 | Carrier Leakage for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.2_1.1 if UE supports SA and TS 38.521-2 TC 6.4A.2.2.1 has been executed. |
| 6.4B.2.4.2_1.2 | Carrier Leakage for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.2_1.2 if UE supports SA and TS 38.521-2 TC 6.4A.2.2.2 has been executed. |
| 6.4B.2.4.2_1.3 | Carrier Leakage for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.2_1.3 if UE supports SA and TS 38.521-2 TC 6.4A.2.2.3 has been executed. |
| 6.4B.2.4.2D | Carrier Leakage for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.3 | In-band Emissions for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.3 if UE supports SA and TS 38.521-2 TC 6.4.2.3 has been executed. |
| 6.4B.2.4.3D | In-band Emissions for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.4 | EVM Equalizer Flatness for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.4 if UE supports SA and TS 38.521-2 TC 6.4.2.4 has been executed. |
| 6.4B.2.4.4D | EVM Equalizer Flatness for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4B.2.4.5 | EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012f | UEs supporting Inter-band including FR2 with 1 NR UL CC and pi/2 BPSK modulation | E010 |  | NOTE 1 NOTE 5 Skip TC 6.4B.2.4.5 if UE supports SA and TS 38.521-2 TC 6.4.2.5 has been executed. |
| 6.5B | Output RF spectrum emissions for DC |  |  |  |  |  |  |
| 6.5B.1.1 | Occupied bandwidth for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 1 |
| 6.5B.1.2 | Occupied bandwidth for Intra-Band Non-Contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 NOTE 5 Skip TC6.5B.1.2 if UE supports SA and TS 38.521-1 TC 6.5.1 has been executed. |
| 6.5B.1.3 | Occupied bandwidth for Inter-Band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC6.5B.1.3 if UE supports SA and TS 38.521-1 TC 6.5.1 has been executed. |
| 6.5B.1.4 | Occupied bandwidth for Inter-Band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4 if UE supports SA and TS 38.521-2 TC 6.5.1 has been executed. |
| 6.5B.1.4D | Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.5B.1.4_1 | Occupied bandwidth for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.1.4_1.1 | Occupied bandwidth for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.1.1 has been executed. |
| 6.5B.1.4_1.2 | Occupied bandwidth for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.1.2 has been executed. |
| 6.5B.1.4_1.3 | Occupied bandwidth for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.1.4_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.1.3 has been executed. |
| 6.5B.2.1.1 | Spectrum emissions mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.2.1.2 | Additional spectrum emissions mask for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.2.1.3 | Adjacent channel leakage ratio for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.2.2.1 | Spectrum emissions mask for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 |
| 6.5B.2.2.2 | Additional Spectrum emissions mask for intra-band non-contiguous EN-DC | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.5B.2.2.3 | Adjacent channel leakage ratio for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 |
| 6.5B.2.3.1 | Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.2.3.1 if UE supports SA and TS 38.521-1 6.5.2.2 has been executed. |
| 6.5B.2.3.2 | Additional Spectrum emissions mask for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.2.3.2 if UE supports SA and TS 38.521-1 6.5.2.2 has been executed. |
| 6.5B.2.3.3 | Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.2.3.3 if UE supports SA and TS 38.521-1 6.5.2.4.1 has been executed. |
| 6.5B.2.4.1 | Spectrum emissions mask for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.2.4.1 if UE supports SA and TS 38.521-2 TC 6.5.2.1 has been executed. |
| 6.5B.2.4_1 | Spectrum emissions mask for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.2.4.1_1.1 | Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012b | UEs supporting Inter-Band EN-DC including FR2 with 2 NR UL CCs | E011 |  | NOTE 5 Skip TC 6.5B.2.4.1_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.2.1.1 has been executed. |
| 6.5B.2.4.1_1.2 | Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | C012c | UEs supporting Inter-Band EN-DC including FR2 with 3 NR UL CCs | E012 |  | NOTE 5 Skip TC 6.5B.2.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.2.1.2 has been executed. |
| 6.5B.2.4.1_1.3 | Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | C012d | UEs supporting Inter-Band EN-DC including FR2 with 4 NR UL CCs | E013 |  | NOTE 5 Skip TC 6.5B.2.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.2.1.3 has been executed. |
| 6.5B.2.4.3 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.2.4.3 if UE supports SA and TS 38.521-2 TC 6.5.2.3 has been executed. |
| 6.5B.2.4.3_1 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 6.5B.2.4.3_1.1 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.2.4.3_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.1 has been executed. |
| 6.5B.2.4.3_1.2 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.2.4.3_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.2 has been executed. |
| 6.5B.2.4.3_1.3 | Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.2.4.3_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.2.2.3 has been executed. |
| 6.5B.2.4D.3 | Adjacent channel leakage ratio for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.5B.3.1.1 | General spurious emissions for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  | NOTE 5 Skip TC 6.5B.3.1.1 if UE supports SA and TS 38.521-1 TC 6.5.3.1 has been executed. |
| 6.5B.3.1.2 | Spurious emission band UE co-existence for intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.3.2.1 | General spurious emissions for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 5 Skip TC 6.5B.3.2.1 if UE supports SA and TS 38.521-1 TC 6.5.3.1 has been executed. |
| 6.5B.3.2.2 | Spurious emission band UE co-existence for intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  |  |
| 6.5B.3.3.1 | General spurious emissions for Inter-band EN-DC within FR1 | Rel-15 | C011 | UEs supporting Inter-band EN-DC within FR1 (2UL CCs) | E005 |  |  |
| 6.5B.3.3.2 | Spurious emission band UE co-existence for Inter-band within FR1 | Rel-15 | C011 | UEs supporting Inter-band EN-DC within FR1 (2UL CCs) | E005 |  |  |
| 6.5B.3.4.1 | General Spurious Emissions for Inter-band including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.3.4.1 if UE supports SA and TS 38.521-2 TC 6.5.3.1 has been executed. |
| 6.5B.3.4.1_1.1 | General Spurious Emissions for Inter-band including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.1_1.1 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.1 has been executed. |
| 6.5B.3.4.1_1.2 | General Spurious Emissions for Inter-band including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.1_1.2 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.2 has been executed. |
| 6.5B.3.4.1_1.3 | General Spurious Emissions for Inter-band including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 6.5B.3.4.1_1.3 if UE supports SA and TS 38.521-2 TC 6.5A.3.1.3 has been executed. |
| 6.5B.3.4.1D | General Spurious Emissions for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.5B.3.4.2 | Spurious emission band UE co-existence for Inter-band including FR2 (1 NR CC) | Rel-15 | C012 | UEs supporting Inter-band including FR2 with 1 NR UL CC | E010 |  | NOTE 5 Skip TC 6.5B.3.4.2 if UE supports SA and TC 38.521-2 TC 6.5.3.2 has been executed. |
| 6.5B.4.1 | Additional Spurious Emissions for Intra-band contiguous EN-DC | Rel-15 | C009 | UEs supporting intra-band contiguous EN-DC (2UL CCs) | E003 |  |  |
| 6.5B.4.2 | Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Rel-15 | C010 | UEs supporting intra-band non-contiguous EN-DC (2UL CCs) | E004 |  | NOTE 1 |
| 6.5B.4.3 | Additional Spurious Emissions for Inter-band EN-DC (1 NR CC) | Rel-15 | C011b | UEs supporting inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.4.3 if UE supports SA and TC 38.521-1 TC 6.5.3.3 has been executed. |
| 6.5B.5.3 | Transmit Intermodulation for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 5 Skip TC 6.5B.5.3 if UE supports SA and TC 38.521-1 TC 6.5.4 has been executed. |
| 6.6B.4 | Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Rel-15 | C011b | UEs supporting Inter-band EN-DC within FR1 with 1 NR UL CC | E005b |  | NOTE 1 NOTE 5 Skip TC 6.6B.4 if UE supports SA and TC 38.521-1 TC 6.6.1 has been executed. |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3B | Reference sensitivity level for DC |  |  |  |  |  |  |
| 7.3B.2.1 | Reference sensitivity for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting intra-band contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.3B.2.2 | Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting intra-band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.3B.2.2 if UE supports SA and TS 38.521-1 TC 7.3.2 has been executed. |
| 7.3B.2.3 | Reference sensitivity for Inter-band EN-DC within FR1 | Rel-15 | C011a | UEs supporting inter-band EN-DC within FR1 (2DL CCs) | E005a |  |  |
| 7.3B.2.3_1 | Reference sensitivity for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.3B.2.3_1.1 | Reference sensitivity for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs or inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs) | E006 |  |  |
| 7.3B.2.3_1.2 | Reference sensitivity for EN-DC within FR1 (4 CCs) | Rel-15 | C046 | UEs supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs or inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs) | E007 |  |  |
| 7.3B.2.3_1.3 | Reference sensitivity for EN-DC within FR1 (5 CCs) | Rel-15 | C047 | UEs supporting inter-band EN-DC within FR1 with 5 DL CCs (4 NR DL CCs) | E008 |  |  |
| 7.3B.2.4 | Reference sensitivity for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting inter-band EN-DC including FR2 with 1 NR DL CC | E010a |  | NOTE 5 Skip TC 7.3B.2.4 if UE supports SA and TS 38.521-2 TC 7.3.2 has been executed. |
| 7.3B.2.4_1 | Reference sensitivity for Inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.3B.2.4_1.1 | Reference sensitivity for Inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | C012e | UEs supporting inter-band EN-DC including FR2 with 2 NR DL CCs | E011a |  | NOTE 1 NOTE 5 Skip TC 7.3B.2.4_1.1 if UE supports SA and TS 38.521-2 TC 7.3A.2.1 has been executed. |
| 7.3B.2.4_1.2 | Reference sensitivity for Inter-band EN-DC including FR2 (3 NR CCs) | FFS | FFS | UEs supporting inter-band EN-DC including FR2 with 3 NR DL CCs | FFS |  | NOTE 1 |
| 7.3B.2.4_1.3 | Reference sensitivity for Inter-band EN-DC including FR2 (4 NR CCs) | FFS | FFS | UEs supporting inter-band EN-DC including FR2 with 4 NR DL CCs | FFS |  | NOTE 1 |
| 7.3B.2.4_1.4 | Reference sensitivity for Inter-band EN-DC including FR2 (5 NR CCs) | FFS | FFS | UEs supporting inter-band EN-DC including FR2 with 5 NR DL CCs | FFS |  | NOTE 1 |
| 7.3B.2.4D | Reference sensitivity for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3B.4 | EIS Spherical Coverage for Inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting inter-band EN-DC including FR2 with 1 NR DL CC | E010a |  | NOTE 5 Skip TC 7.3B.4 if UE supports SA and TS 38.521-2 TC 7.3.4 has been executed. |
| 7.4B | Maximum Input Level for DC |  |  |  |  |  |  |
| 7.4B.1 | Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.4B.2 | Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  |  |
| 7.4B.3 | Maximum Input Level for Inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting Inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.4B.3 if UE supports SA and TS 38.521-1 TC 7.4 has been executed. |
| 7.4B.3_1 | Maximum Input Level for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.4B.3_1.1 | Maximum Input Level for EN-DC within FR1 (3 CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.3_1.2 | Maximum Input Level for EN-DC within FR1 (4 CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.3_1.3 | Maximum Input Level for EN-DC within FR1 (5 CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.3_1.4 | Maximum Input Level for EN-DC within FR1 (6 CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4 | Maximum Input Level for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | C012a | UEs supporting Inter-band including FR2 with 1 NR DL CC | E010a |  | NOTE 1 NOTE 5 Skip TC 7.4B.4 if UE supports SA and TS 38.521-1 7.4 has been executed. |
| 7.4B.4_1 | Maximum Input Level for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.4B.4_1.1 | Maximum Input Level for inter-band EN-DC including FR2 (2 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4_1.2 | Maximum Input Level for inter-band EN-DC including FR2 (3 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4_1.3 | Maximum Input Level for inter-band EN-DC including FR2 (4 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4_1.4 | Maximum Input Level for inter-band EN-DC including FR2 (5 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4B.4D | Maximum Input Level for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5B | Adjacent channel selectivity for DC |  |  |  |  |  |  |
| 7.5B.1 | Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting intra-band contiguous EN-DC (2DL CCs) | E003a |  | NOTE 1 |
| 7.5B.2 | Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting intra-band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 1 NOTE 5 Skip TC 7.5B.2 if UE supports SA and TS 38.521-1 TC 7.5 has been executed. |
| 7.5B.3 | Adjacent Channel Selectivity for inter-band EN-DC within FR1 (2 CCs) | Rel-15 | C011c | UEs supporting inter-band EN-DC within FR1 with 1 NR DL CCs and one or more LTE DL CC(s) | E005c |  | NOTE 5 Skip TC 7.5B.3 if UE supports SA and TS 38.521-1 TC 7.5 has been executed. |
| 7.5B.3_1 | Adjacent Channel Selectivity for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.5B.3_1.1 | Adjacent Channel Selectivity for EN-DC within FR1 (2 NR CCs) | Rel-15 | C063 | UEs supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 2 NR DL CCs | E027 E029 |  | NOTE 5 Skip TC 7.5B.3_1.1 if UE supports SA and TS 38.521-1 TC 7.5A.1 has been executed. |
| 7.5B.3_1.2 | Adjacent Channel Selectivity for EN-DC within FR1 (3 NR CCs) | Rel-15 | C064 | UEs supporting inter-band or intra-band non-contiguous EN-DC within FR1 with 3 NR DL CCs | E028 E030 |  | NOTE 5 Skip TC 7.5B.3_1.2 if UE supports SA and TS 38.521-1 TC 7.5A.2 has been executed. |
| 7.5B.3_1.3 | Adjacent Channel Selectivity for EN-DC within FR1 (4 NR CCs) | Rel-15 | C064a | UEs supporting intra-band non-contiguous EN-DC within FR1 with 4 NR DL CCs | E028a E030a |  | NOTE 5 Skip TC 7.5B.3_1.3 if UE supports SA and TS 38.521-1 TC 7.5A.3 has been executed. |
| 7.5B.3_1.4 | Adjacent Channel Selectivity for EN-DC within FR1 (5 NR CCs) | Rel-15 | C064b | UEs supporting intra-band non-contiguous EN-DC within FR1 with 5 NR DL CCs | E028b E030b |  | NOTE 1 NOTE 5 Skip TC 7.5B.3_1.4 if UE supports SA and TS 38.521-1 TC 7.5A.4 has been executed. |
| 7.5B.4 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.5B.4 if UE supports SA and TS 38.521-2 TC 7.5 has been executed. |
| 7.5B.4_1 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.5B.4_1.1 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.5B.4_1.1 if UE supports SA and TS 38.521-2 TC 7.5A.1 has been executed. |
| 7.5B.4_1.2 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (3 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.5B.4_1.2 if UE supports SA and TS 38.521-2 TC 7.5A.2 has been executed. |
| 7.5B.4_1.3 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (4 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.5B.4_1.3 if UE supports SA and TS 38.521-2 TC 7.5A.3 has been executed. |
| 7.5B.4_1.4 | Adjacent Channel Selectivity for inter-band EN-DC including FR2 (5 NR CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.5B.4_1.4 if UE supports SA and TS 38.521-2 TC 7.5A.4 has been executed. |
| 7.5B.4D | Adjacent Channel Selectivity for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B | Blocking characteristics for DC |  |  |  |  |  |  |
| 7.6B.2.1 | Inband blocking for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.6B.2.2 | Inband blocking for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.6B.2.2 if UE supports SA and TS 38.521-1 TC 7.6.2 has been executed. |
| 7.6B.2.3 | Inband blocking for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting Inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.6B.2.3 if UE supports SA and TS 38.521-1 TC 7.6.2 has been executed. |
| 7.6B.2.3_1 | Inband blocking for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.6B.2.3_1.1 | Inband blocking for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs or inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs) | E006 |  |  |
| 7.6B.2.3_1.2 | Inband blocking for EN-DC within FR1 (4 CCs) | Rel-16 | C046 | UEs supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs or inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs) | E007 |  |  |
| 7.6B.2.3_1.3 | Inband blocking for EN-DC within FR1 (5 CCs) | Rel-16 | C047 | UEs supporting inter-band EN-DC within FR1 with 5 DL CCs (4 NR DL CCs) | E008 |  | Skip TC 7.6B.2.3_1.3 if UE supports SA and TS 38.521-1 TC 7.6A.2.3 has been executed. |
| 7.6B.2.4 | Inband blocking for inter-band EN-DC including FR2 (1 NR CC) | Rel-15 | FFS | UEs supporting Inter-band including FR2 with 1 NR DL CC | FFS |  | NOTE 1 NOTE 5 Skip TC 7.6B.2.4 if UE supports SA and TS 38.521-2 TC 7.6.2 has been executed. |
| 7.6B.2.4_1 | Inband blocking for inter-band EN-DC including FR2 (>1 NR CC) |  |  |  |  |  |  |
| 7.6B.2.4_1.1 | Inband blocking for inter-band EN-DC including FR2 (2 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B.2.4_1.2 | Inband blocking for inter-band EN-DC including FR2 (3 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B.2.4_1.3 | Inband blocking for inter-band EN-DC including FR2 (4 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B.2.4_1.4 | Inband blocking for inter-band EN-DC including FR2 (5 NR CCs) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B.2.4D | Inband blocking for inter-band EN-DC including FR2 for UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.6B.3.1 | Out-of-band blocking for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.6B.3.2 | Out-of-band blocking for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.6B.3.2 if UE supports SA and TS 38.521-1 TC 7.6.3 has been executed. |
| 7.6B.3.3 | Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) | Rel-15 | C011a | UEs supporting Inter-band EN-DC within FR1 (2DL CCs) | E005a |  |  |
| 7.6B.3.3_1 | Out-of-band blocking for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.6B.3.3_1.1 | Out-of-band blocking for EN-DC within FR1 (3 CCs) | Rel-15 | C048 | UEs supporting intra-band contiguous EN-DC in FR1 with 3 DL CCs | E006 |  |  |
| 7.6B.3.3_1.2 | Out-of-band blocking for EN-DC within FR1 (4 CCs) | Rel-16 | C049 | UEs supporting intra-band contiguous EN-DC in FR1 with 4 DL CCs | E007 |  |  |
| 7.6B.4.1 | Narrow band blocking for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.6B.4.2 | Narrow band blocking for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.6B.4.2 if UE supports SA and TS 38.521-1 TC 7.6.4 has been executed. |
| 7.6B.4.3 | Narrow band blocking for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting Inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.6B.4.3 if UE supports SA and TS 38.521-1 TC 7.6.4 has been executed. |
| 7.6B.4.3_1 | Narrow band blocking for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.6B.4.3_1.1 | Narrow band blocking for EN-DC within FR1 (3 CCs) | Rel-15 | C045 | UEs supporting intra-band contiguous EN-DC within FR1 with 3 DL CCs or inter-band EN-DC within FR1 with 3 DL CCs (2NR DL CCs) | E006 |  |  |
| 7.6B.4.3_1.2 | Narrow band blocking for EN-DC within FR1 (4 CCs) | Rel-16 | C046 | UEs supporting intra-band contiguous EN-DC within FR1 with 4 DL CCs or inter-band EN-DC within FR1 with 4 DL CCs (3NR DL CCs) | E007 |  |  |
| 7.6B.4.3_1.3 | Narrow band blocking for EN-DC within FR1 (5 CCs) | Rel-16 | C047 | UEs supporting inter-band EN-DC within FR1 with 5 DL CCs (4 NR DL CCs) | E008 |  | Skip TC 7.6B.4.3_1.3 if UE supports SA and TS 38.521-1 TC 7.6A.4.3 has been executed. |
| 7.7B | Spurious response for DC |  |  |  |  |  |  |
| 7.7B.1 | Spurious Response for intra-band contiguous EN-DC (2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  |  |
| 7.7B.2 | Spurious Response for intra-band non-contiguous EN-DC (2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band Non-Contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.7B.2 if UE supports SA and TS 38.521-1 TC 7.7 has been executed. |
| 7.7B.3 | Spurious Response for inter-band EN-DC within FR1 (2 CCs) | Rel-15 | C011a | UEs supporting Inter-band EN-DC within FR1 (2DL CCs) | E005a |  |  |
| 7.7B.3_1 | Spurious Response for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.7B.3_1.1 | Spurious Response for EN-DC within FR1 (3 CCs) | Rel-15 | C048 | UEs supporting intra-band contiguous EN-DC in FR1 with 3 DL CCs | E006 |  |  |
| 7.7B.3_1.2 | Spurious Response for EN-DC within FR1 (4 CCs) | Rel-16 | C049 | UEs supporting intra-band contiguous EN-DC in FR1 with 4 DL CCs | E007 |  |  |
| 7.8B | Intermodulation characteristics for DC in FR1 |  |  |  |  |  |  |
| 7.8B.2.1 | Wideband Intermodulation for intra-band contiguous EN-DC in FR1 | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  | NOTE 1 |
| 7.8B.2.2 | Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 | Rel-15 | C010a | UEs supporting Intra-Band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.8B.2.2 if UE supports SA and TS 38.521-1 TC 7.8.2 has been executed. |
| 7.8B.2.3 | Wideband Intermodulation for inter-band EN-DC in FR1 (2 CCs) | Rel-15 | C011a | UEs supporting inter-band EN-DC within FR1 (2DL CCs) | E005c |  | NOTE 5 Skip TC 7.8B.2.3 if UE supports SA and TS 38.521-1 TC 7.8.2 has been executed. |
| 7.8B.2.3_1 | Wideband Intermodulation for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.8B.2.3_1.1 | Wideband Intermodulation for EN-DC within FR1 (3 CCs) | Rel-15 | FFS | FFS | FFS |  | NOTE 1 NOTE 5 Skip TC 7.8B.2.3_1.1 if UE supports SA and TS 38.521-1 TC 7.8A.2.1 has been executed. |
| 7.9B | Spurious emissions for DC in FR1 |  |  |  |  |  |  |
| 7.9B.1 | Spurious Emissions for intra-band contiguous EN-DC in FR1(2 CCs) | Rel-15 | C009a | UEs supporting Intra-Band Contiguous EN-DC (2DL CCs) | E003a |  | NOTE 5 Skip TC 7.9B.1 if UE supports SA and TS 38.521-1 TC 7.9 has been executed. |
| 7.9B.2 | Spurious Emissions for intra-band non-contiguous EN-DC in FR1(2 CCs) | Rel-15 | C010a | UEs supporting Intra-Band non-contiguous EN-DC (2DL CCs) | E004a |  | NOTE 5 Skip TC 7.9B.2 if UE supports SA and TS 38.521-1 TC 7.9 has been executed. |
| 7.9B.3 | Spurious Emissions for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011c | UEs supporting inter-band EN-DC within FR1 with 1 NR DL CC | E005c |  | NOTE 5 Skip TC 7.9B.3 if UE supports SA and TS 38.521-1 TC 7.9 has been executed. |
| 7.9B.3_1 | Spurious Emissions for EN-DC within FR1 (>2 CCs) |  |  |  |  |  |  |
| 7.9B.3_1.1 | Spurious Emissions for EN-DC within FR1 (3 CCs) | Rel-15 | C048 | UEs supporting EN-DC within FR1 with 1 LTE DL CC and 2 inter-band NR DL CCs with DL-only NR band | E006 |  |  |
| 7.9B.4 | Spurious Emissions for inter-band EN-DC including FR2 (2 CCs) | Rel-15 | C012a | UEs supporting Inter-band including FR2 with 1 NR DL CC and one or more LTE DL CC(s) | E010a |  | NOTE5 Skip TC 7.9B.4 if UE supports SA and TS 38.521-2 TC 7.9 has been executed. |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-3. NOTE 2: Void. NOTE 3: Void. NOTE 4: Void. NOTE 5: Test only one EN-DC combination per 5G NR band as LTE anchor agnostic approach is applied. |  |  |  |  |  |  |  |

Table 4.1.3-1a: Void

Table 4.1.3-1b: Void

Table 4.1.3-1c: Void

### 4.1.4	Performance conformance test cases

Table 4.1.4-1: Applicability of performance test cases, ref. TS 38.521-4 [4]

| Clause | TC Title | Release | Applicability |  | Tested Bands Selection | Additional Information |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 5 | Demodulation performance requirements (Conducted requirements) |  |  |  |  |  |
| 5.2 | PDSCH demodulation requirements |  |  |  |  |  |
| 5.2.2.1.1_1 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 D010 |  |
| 5.2.2.1.1_2 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C015x | UEs supporting 5GS FDD FR1 and Enhanced Receiver Type 1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.2_1 | 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.3_1 | 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015b | UEs supporting 5GS FDD FR1 and PDSCH mapping Type B but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.4_1 | 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015y | UEs supporting 5GS FDD FR1 and additional DMRS for coexistence with LTE CRS but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.5_1 | 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C074 | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.11_1 | 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C070 | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.2.1_1 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D009 D010 |  |
| 5.2.2.2.1_2 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C016x | UEs supporting 5GS TDD FR1 and Enhanced Receiver Type 1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.2_1 | 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.3_1 | 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016b | UEs supporting 5GS TDD FR1 and PDSCH mapping Type B but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.4_1 | 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C016y | UEs supporting 5GS TDD FR1 and additional DMRS for coexistence with LTE CRS but not supporting TDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.2.5_1 | 2Rx TDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C075 | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5, but not supporting TDD bands with 4Rx UE capability | D009 |  |
| 5.2.2.2.11_1 | 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C071 | UEs supporting 5GS TDD FR1 and single DCI based spatial division multiplexing scheme, but not supporting TDD bands with 4Rx UE capability | D009 |  |
| 5.2.3.1.1_1 | 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D009 |  |
| 5.2.3.1.1_2 | 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D009 |  |
| 5.2.3.1.1_4 | 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C017x | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and Enhanced Receiver Type 1 | D008 |  |
| 5.2.3.1.2_1 | 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.2.3.1.3_1 | 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017b | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and PDSCH mapping Type B | D008 D011 |  |
| 5.2.3.1.4_1 | 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017y | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and LTE-NR coexistence | D008 |  |
| 5.2.3.1.5_1 | 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C076 | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports | D008 |  |
| 5.2.3.1.11_1 | 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 | C072 | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports | D008 |  |
| 5.2.3.2.1_1 | 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D009 D010 |  |
| 5.2.3.2.1_2 | 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 5.2.3.2.1_4 | 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C019x | UEs supporting 5GS TDD FR1 and Enhanced Receiver Type 1 and 4Rx antenna ports | D010 |  |
| 5.2.3.2.2_1 | 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D009 D010 |  |
| 5.2.3.2.3_1 | 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019b | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and PDSCH mapping Type B | D009 D011 |  |
| 5.2.3.2.5_1 | 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C077 | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports | D009 |  |
| 5.2.3.2.11_1 | 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C073 | UEs supporting 5GS TDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports | D009 |  |
| 5.3.2.1.1 | 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.3.2.1.2 | 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.3.2.2.1 | 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.3.2.2.2 | 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.3.3.1.1 | 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.3.3.1.2 | 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.3.3.2.1 | 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 5.3.3.2.2 | 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 5.5.1 | FR1 Sustained downlink data rate performance for single carrier | Rel-15 | C001 | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) | D008 D009 D010 |  |
| 6 | CSI reporting requirements (Conducted requirements) |  |  |  |  |  |
| 6.2.2.1.1.1 | 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.2.2.1.2.1 | 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.2.2.1.2.2 | 2Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.2.2.2.1.1 | 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.2.2.2.2.1 | 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.2.2.2.2.2 | 2Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.2.3.1.1.1 | 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 6.2.3.1.2.1 | 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 6.2.3.1.2.2 | 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 6.2.3.2.1.1 | 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.2.3.2.2.1 | 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.2.3.2.2.2 | 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.3.2.1.1 | 2Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.2 | 2Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.3 | 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.4 | 2Rx FDD FR1 Single PMI with 32Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.2.1 | 2Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.2 | 2Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.3 | 2Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.4 | 2Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.3.1.1 | 4Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.2 | 4Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.3 | 4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.4 | 4Rx FDD FR1 Single PMI with 32Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.2.1 | 4Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D010 D011 |  |
| 6.3.3.2.2 | 4Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D010 D011 |  |
| 6.3.3.2.3 | 4Rx TDD FR1 Single PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.4.2.1_1 | 2Rx FDD FR1 RI reporting for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.4.2.2_1 | 2Rx TDD FR1 RI reporting for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.4.3.1_1 | 4Rx FDD FR1 RI reporting for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.4.3.2_1 | 4Rx TDD FR1 RI reporting for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 D011 |  |
| 7 | Demodulation performance requirements (Radiated requirements) |  |  |  |  |  |
| 7.2.2.2.1_1 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D013 D014 D015 |  |
| 7.2.2.2.1_2 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with enhanced type 1 receiver for SA and NSA | Rel-15 | C062 | UEs supporting 5GS TDD FR2 and Enhanced Receiver Type 1 | D014 | NOTE 1 |
| 7.3.2.2.1 | 2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 7.3.2.2.2 | 2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 8 | CSI reporting requirements (Radiated requirements) |  |  |  |  |  |
| 8.2.2.2.1.1 | 2Rx TDD FR2 periodic CQI reporting under AWGN performance for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 8.2.2.2.2.1 | 2Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA | FFS | FFS | FFS | FFS | NOTE 1 |
| 8.3.2.2.1 | 2Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 8.4.2.2.1 | 2Rx TDD FR2 RI reporting for both SA and NSA | FFS | FFS | FFS | FFS | NOTE 1 |
| 9 | Demodulation performance requirements for interworking |  |  |  |  |  |
| 9.4B.1.1 | Sustained downlink data rate performance for EN-DC within FR1 | Rel-15 | C020 | UEs supporting 5GS FDD FR1 or TDD FR1 (NSA) | D008 D009 D010 |  |
| 9.4B.1.2 | Sustained downlink data rate performance for EN-DC including FR2 NR carrier | FFS | FFS | FFS | FFS | NOTE 1 |
| 10 | CSI reporting requirements for interworking |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-4. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

Table 4.1.4-1a: Void

Table 4.1.4-1b: Void

Table 4.1.4-1c: Void

## 4.2	RRM conformance test cases

Table 4.2-1: Applicability of RRM EN-DC FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 4.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 4.3.2 | RRC connection mobility control |  |  |  |  |  |
| 4.3.2.2 | Random access |  |  |  |  |  |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | Rel-15 | C030 | UEs supporting EN-DC FR1 and CSI-RS based PRACH |  | 2Rx 4Rx |
| 4.4 | Timing |  |  |  |  |  |
| 4.4.1 | UE Transmit Timing |  |  |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.4.2 | UE timer accuracy |  |  |  |  |  |
| 4.4.3 | Timing Advance |  |  |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5 | Signalling characteristics |  |  |  |  |  |
| 4.5.1 | Radio link monitoring |  |  |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2 | Interruption |  |  |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |
| 4.5.3 | SCell activation and deactivation delay |  |  |  |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C032 | UEs supporting EN-DC FR1 and SUL |  | 2Rx 4Rx |
| 4.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C082 | UEs supporting EN-DC FR1 and link recovery |  | 2Rx 4Rx |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C082a | UEs supporting EN-DC FR1 and long DRX cycle and link recovery |  | 2Rx 4Rx |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C083 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C083a | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 4.5.6 | Active BWP switch delay |  |  |  |  |  |
| 4.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065 | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | Rel-15 | C065a | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |
| 4.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6 | Measurement procedures |  |  |  |  |  |
| 4.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C042 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | Rel-15 | C042a | UEs supporting EN-DC FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle |  | 2Rx 4Rx |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Rel-15 | C021b | UEs supporting EN-DC FDD FR1 |  | 2Rx 4Rx |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Rel-15 | C042b | UEs supporting EN-DC FDD FR1 and CSI-RS based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 4.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.7 | Measurement performance requirements |  |  |  |  |  |
| 4.7.1 | SS-RSRP |  |  |  |  |  |
| 4.7.1.1 | Intra-frequency measurements |  |  |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.1.2 | Inter-frequency measurements |  |  |  |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.2 | SS-RSRQ |  |  |  |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.3 | SS-SINR |  |  |  |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 4.7.4 | L1-RSRP |  |  |  |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.4.1.2 | EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.5 | SFTD |  |  |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | Rel-15 | C043 | UEs supporting EN-DC FR1 and SFTD measurements between E-UTRA PCell and NR PSCell |  | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |

Table 4.2-1a: Void

Table 4.2-2: Applicability of RRM EN-DC FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 5.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 5.3.2 | RRC connection mobility control |  |  |  |  |  |
| 5.3.2.2 | Random access |  |  |  |  |  |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | Rel-16 | C022 | UEs supporting EN-DC FR2 |  | 2Rx 4Rx |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | Rel-16 | C030a | UEs supporting EN-DC FR2 and CSI-RS based PRACH |  | 2Rx 4Rx |
| 5.4 | Timing |  |  |  |  |  |
| 5.4.1 | UE transmit timing |  |  |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.4.2 | UE timer accuracy |  |  |  |  |  |
| 5.4.3 | Timing advance |  |  |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.5 | Signalling characteristics |  |  |  |  |  |
| 5.5.1 | Radio link monitoring |  |  |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.2 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.4 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2 | Interruption |  |  |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.3 | SCell activation and deactivation delay |  |  |  |  |  |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 5.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.5.6 | Active BWP switch delay |  |  |  |  |  |
| 5.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 5.5.6.1.1 | EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.6.1.2 | EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 5.5.6.2.1 | EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 5.5.7.1 | EN-DC FR2 addition and release delay of known PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.8 | Active TCI state switch delay |  |  |  |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.6 | Measurement procedures |  |  |  |  |  |
| 5.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | Rel-15 | C043a | UEs supporting EN-DC FR2, long DRX cycle, CSI-RS based RLM and BWP operation without BW restriction |  | 2Rx |
| 5.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | C023 | UEs supporting EN-DC FR1 and FR2 |  | 2Rx |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | Rel-15 | C023a | UEs supporting EN-DC FR1 and FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C023 | UEs supporting EN-DC FR1 and FR2 |  | 2Rx |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C023a | UEs supporting EN-DC FR1 and FR2 and long DRX cycle |  | 2Rx |
| 5.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 5.7 | Measurement performance requirements |  |  |  |  |  |
| 5.7.1 | SS-RSRP |  |  |  |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.7.2 | SS-RSRQ |  |  |  |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.2.2 | EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.7.3 | SS-SINR |  |  |  |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas |  | 2Rx |
| 5.7.3.2 | EN-DC FR2-FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas | NOTE 1 | 2Rx |
| 5.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

Table 4.2-2a: Void

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST | NOTE 1 | 2Rx 4Rx |
| 6.1.2 | NR – E-UTRA cell re-selection |  |  |  |  |  |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.2 | RRC_INACTIVE state mobility |  |  |  |  |  |
| 6.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 6.3.1 | Handover |  |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.3 | NR SA FR1-FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.2 | RRC connection mobility control |  |  |  |  |  |
| 6.3.2.1 | RRC re-establishment |  |  |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.1.2 | NR SA FR1 – FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.2 | Random access |  |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | Rel-15 | C029 | UEs supporting 5GS NR SA FR1 and CSI-RS based PRACH |  | 2Rx 4Rx |
| 6.3.2.3 | RRC connection release with redirection |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.3.2 | NR SA FR1 – E-UTRA RRC connection release with redirection | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.4 | Timing |  |  |  |  |  |
| 6.4.1 | UE transmit timing |  |  |  |  |  |
| 6.4.1.1 | NR SA FR1 transmit timing accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.4.2 | UE timer accuracy |  |  |  |  |  |
| 6.4.3 | Timing advance |  |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.5 | Signalling characteristics |  |  |  |  |  |
| 6.5.1 | Radio Link Monitoring |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 6.5.2 | Interruption |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3 | Scell activation and deactivation delay |  |  |  |  |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C002 | UEs supporting 5GS NR SA FR1 and SUL |  | 2Rx 4Rx |
| 6.5.5 | Link recovery procedures |  |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C084 | UEs supporting 5GS NR SA FR1 and link recovery |  | 2Rx 4Rx |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C084a | UEs supporting 5GS NR SA FR1 and long DRX cycle and link recovery |  | 2Rx 4Rx |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C085 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C085a | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 6.5.6 | Active BWP switch delay |  |  |  |  |  |
| 6.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066a | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066 | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 6.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6 | Measurement procedures |  |  |  |  |  |
| 6.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 6.6.1.1 | NR SA FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.1.2 | NR SA FR1 event-triggered reporting without gap in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.1.3 | NR SA FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C041 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 6.6.1.4 | NR SA FR1 event-triggered reporting with gap in DRX | Rel-15 | C041a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle |  | 2Rx 4Rx |
| 6.6.1.5 | NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading | Rel-15 | C024 | UEs supporting 5GS NR FDD SA FR1 |  | 2Rx 4Rx |
| 6.6.1.6 | NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | Rel-15 | C041b | UEs supporting 5GS NR FDD SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  | 2Rx 4Rx |
| 6.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.3 | Inter-RAT measurements |  |  |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRAN |  | 2Rx 4Rx |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Rel-15 | C025a | UEs supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle |  | 2Rx 4Rx |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Rel.16 | C052 | UEs supporting 5GS NR SA FR1 and E-UTRAN configured with highSpeedMeasFlag-r16 |  | 2Rx 4Rx |
| 6.6.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.7 | Measurement performance requirements |  |  |  |  |  |
| 6.7.1 | SS-RSRP |  |  |  |  |  |
| 6.7.1.1 | Intra-frequency measurements |  |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.1.2 | Inter-frequency measurements |  |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.2 | SS-RSRQ |  |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.3 | SS-SINR |  |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 6.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB-based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.4.1.2 | NR SA FR1 SSB-based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.4.2.1 | NR SA FR1 CSI-RS-based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.4.2.2 | NR SA FR1 CSI-RS-based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.5 | E-UTRAN RSRP |  |  |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.6 | E-UTRAN RSRQ |  |  |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.7 | E-UTRAN RS-SINR |  |  |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |

Table 4.2-3a: Void

Table 4.2-4: Applicability of RRM NR SA FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 7.1 | RRC_IDLE state mobility |  |  |  |  |  |
| 7.1.1 | NR cell re-selection |  |  |  |  |  |
| 7.1.1.1 | NR SA FR2 cell re-selection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.2 | RRC_INACTIVE state mobility |  |  |  |  |  |
| 7.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 7.3.1 | Handover |  |  |  |  |  |
| 7.3.2 | RRC connection mobility control |  |  |  |  |  |
| 7.3.2.1 | RRC re-establishment |  |  |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.3.2.1.2 | NR SA FR2 - FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.3.2.2 | Random access |  |  |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.2.3 | RRC connection release with redirection |  |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2 RRC connection release with redirection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.4 | Timing |  |  |  |  |  |
| 7.4.1 | UE transmit timing |  |  |  |  |  |
| 7.4.2 | UE timer accuracy |  |  |  |  |  |
| 7.4.3 | Timing advance |  |  |  |  |  |
| 7.5 | Signalling characteristics |  |  |  |  |  |
| 7.5.1 | Radio Link Monitoring |  |  |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.2 | Interruption |  |  |  |  |  |
| 7.5.3 | Scell activation and deactivation delay |  |  |  |  |  |
| 7.5.3.1 | NR SA FR2-FR2 intra-band SCell activation and deactivation delay | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.3.2 | NR SA FR1-FR2 inter-band SCell activation and deactivation delay | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 7.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.5.5 | NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.6 | Active BWP switch delay |  |  |  |  |  |
| 7.5.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 7.5.6.1.1 | NR SA FR2 2DL CA DCI-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.6.1.2 | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.6.1.3 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 7.5.6.2.1 | NR SA FR2 RRC-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 7.5.7.1 | NR SA FR2 addition and release delay of known PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.7.2 | NR SA FR2 addition and release delay of unknown PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.6 | Measurement procedures |  |  |  |  |  |
| 7.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.7 | Measurement performance requirements |  |  |  |  |  |
| 7.7.1 | SS-RSRP |  |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.1.3 | Inter-frequency measurements between FR1 and FR2 |  |  |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.2 | SS-RSRQ |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.3 | SS-SINR |  |  |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

Table 4.2-4a: Void

Table 4.2-5: Applicability of E-UTRA – NR Inter-RAT conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 8.2 | RRC_IDLE state mobility |  |  |  |  |  |
| 8.2.1 | Inter-RAT cell re-selection |  |  |  |  |  |
| 8.2.1.1 | E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 8.2.1.2 | E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | Rel.16 | C052 | UEs supporting 5GS NR SA FR1 and E-UTRAN configured with highSpeedMeasFlag-r16 | NOTE 1 | 2Rx 4Rx |
| 8.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 8.3.1 | Inter-RAT cell handover |  |  |  |  |  |
| 8.3.1.1 | E-UTRA – NR FR1 handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 8.4 | Measurement procedures |  |  |  |  |  |
| 8.4.1 | SFTD measurement delay |  |  |  |  |  |
| 8.4.1.1 | E-UTRA – NR FR1 SFTD measurement delay in non-DRX | Rel-15 | C081 | UEs supporting EN-DC and E-UTRA and SFTD measurements between E-UTRA PCell and NR neighbour cell |  | 2Rx 4Rx |
| 8.4.1.2 | E-UTRA – NR FR1 SFTD measurement delay in DRX | Rel-15 | C081a | UEs supporting EN-DC and E-UTRA and long DRX cycle and SFTD measurements between E-UTRA PCell and NR neighbour cell |  | 2Rx 4Rx |
| 8.4.2 | Inter-RAT measurements |  |  |  |  |  |
| 8.4.2.1 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.4.2.2 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Rel-15 | C086a | UEs supporting E-UTRA and NR FR1 measurement and long DRX cycle |  | 2Rx 4Rx |
| 8.4.2.3 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.4.2.4 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Rel-15 | C086a | UEs supporting E-UTRA and NR FR1 measurement and long DRX cycle |  | 2Rx 4Rx |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement | NOTE 1 | 2Rx |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | Rel-15 | C080a | UEs supporting E-UTRA and NR FR2 measurement and long DRX cycle | NOTE 1 | 2Rx |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement | NOTE 1 | 2Rx |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | Rel-15 | C080a | UEs supporting E-UTRA and NR FR2 measurement and long DRX cycle | NOTE 1 | 2Rx |
| 8.5 | Measurement performance requirements |  |  |  |  |  |
| 8.5.1 | SFTD measurement accuracy |  |  |  |  |  |
| 8.5.1.1 | E-UTRA – NR FR1 SFTD measurement accuracy | Rel-15 | C081 | UEs supporting EN-DC and E-UTRA and SFTD measurements between E-UTRA PCell and NR neighbour cell | NOTE 1 | 2Rx 4Rx |
| 8.5.2 | Inter-RAT |  |  |  |  |  |
| 8.5.2.1 | SS-RSRP |  |  |  |  |  |
| 8.5.2.1.1.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.1.2 | E-UTRA SS-RSRP measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| 8.5.2.2 | SS-RSRQ |  |  |  |  |  |
| 8.5.2.2.1 | E-UTRA SS-RSRQ measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.2.2 | E-UTRA SS-RSRQ measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| 8.5.2.3 | SS-SINR |  |  |  |  |  |
| 8.5.2.3.1 | E-UTRA SS-SINR measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.3.2 | E-UTRA SS-SINR measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. |  |  |  |  |  |  |

Annex A (informative): Change history

## {End of changes}