3GPP TSG-RAN5 Meeting #92-e	R5-215611r1

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0224 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case 10.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the WP test case 10.9 (based on similar test case 19.4.5 in TS 34.229-1) needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 10.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage will remaing incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-215611->R5-215611r1: -test sequence updated with RRC steps - parallel behaviour added - specific mesage contents re-formatted |  |  |  |  |  |  |  |  |

## 10.9	Emergency call without emergency registration / UE credentials are not accepted / 5GS

10.9.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being made to initiate an emergency call }

ensure that {

when { UE attempts IMS emergency registration and network declines by sending 403 Forbidden }

then { UE initiates and completes IMS emergency calls on non-protected ports }

}

10.9.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 33.203 clause 7.1]

The P-CSCF is allowed to receive only REGISTER messages, messages relating to emergency services in accordance with TS 23.167 [31] and TS 24.229 [8], and error messages related to unprotected messages on unprotected ports. All other messages not arriving on a protected port shall be either discarded or rejected by the P-CSCF.

4.	The UE is allowed to receive only the following messages on an unprotected port:

-	responses to unprotected REGISTER messages;

-	messages relating to emergency services in accordance with TS 23.167 [31] and TS 24.229 [8];

-	error messages related to unprotected messages.

All other messages not arriving on a protected port shall be rejected or silently discarded by the UE.

[TS 24.229 clause 5.1.6.1]

If the IM CN subsystem is selected and the UE is currently attached to its home operator's network (e.g. HPLMN) and the UE is currently registered and the IP-CAN defines emergency bearers and the core network has indicated that it supports emergency bearers, the UE shall:

1)	perform an initial emergency registration, as described in subclause 5.1.6.2; and

2)	attempt an emergency call as described in subclause 5.1.6.8.3.

10.9.3	Test description

10.9.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

10.9.3.2	Test procedure sequence

Table 10.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE to initiate an emergency call. | - | - | - | - |
| 2-9 | Steps 1-8 of annex table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. | - | - | - | - |
|  | EXCEPTION: In parallel to the events described in steps 10-11 below the events specified in steps 1a1 to 5 of Table 10.9.3.2-2 take place. | - | -- | - | - |
| 10-11 | Steps 9-10 of annex table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. | - | - | - | - |
|  | EXCEPTION: In parallel to the events described in steps 12-14 below the events specified in steps 5 to 9 of Table 10.9.3.2-2 take place. |  |  |  |  |
| 12-14 | Steps 11-13-10 of annex table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. |  |  |  |  |
| 15 | SS sends BYE. (Step 1 of annex A.8) | <-- | BYE | - | - |
| 16 | UE sends 200 OK. (Step 1 of annex A.8) | --> | 200 OK | - | - |

Table 10.9.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. |  |  |  |  |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 in TS 38.508-1 [21] is performed. |  |  |  |  |
| 1 | UE sends REGISTER (Step 1 of Annex A.3) | --> | REGISTER | - | - |
| 2 | SS sends 401 Unauthorized (Step 2 of Annex A.3) | <-- | 401 Unauthorized |  |  |
| 3 | UE sends REGISTER (Step 3 of Annex A.3) | --> | REGISTER |  |  |
| 4 | SS sends 403 Forbidden | <-- | 403 Forbidden |  |  |
| 5 | Check: Does the UE send a correctly composed INVITE request? (Step 1 of annex A.6) | --> | INVITE | 1 | P |
| 6 | SS sends 100 Trying. (Step 2 of annex A.6) | <-- | 100 Trying |  |  |
| 7 | SS sends 180 Ringing. (Step 3 of annex A.6) | <-- | 180 Ringing |  |  |
| 8 | SS sends 200 OK. (Step 4 of annex A.6) | <-- | 200 OK |  |  |
| 9 | Check: Does the UE send ACK? (Step 5 of annex A.6) | --> | ACK | 1 | P |

10.9.3.3	Specific message contents

Table 10.9.3.3-1: INVITE (step 5, table 10.9.3.2-2)

| Derivation Path: TS 34.229-1 [2], Annex A.2.1, Conditions A6, A7, A8, A28. |  |  |
| --- | --- | --- |
| Header/param | Cond | Value/Remark |
| Message-body |  |  | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] Time description: t= (start-time) (stop-time) Media description: m=audio (transport port) [Note 2] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: EVS codec shall be present in the media attributes, optionally including channel number "/1". |  |  |

Table 10.9.3.3-2: 180 Ringing (step 7, table 10.9.3.2-2)

Derivation Path: TS 34.229-1 [2], Annex A.2.6, Conditions A4, A7, A14.

Table 10.9.3.3-3: 200 OK (step 8, table 10.9.3.2-2)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, Conditions A6, A7, A22. |  |  |
| --- | --- | --- |
| Header/param | Cond | Value/Remark |
| Content-Type media-type |  | application/sdp |
| Content-Length value |  | length of message-body |
| Message-body |  | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:37 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1] b=AS:37 b=RS:0 b=RR:0 Attributes for media: a=rtpmap: (payload type) EVS/16000/1 [Note 1] a=fmtp: (format) mode-change-capability=2; max-red=220 a=ptime:20 a=maxptime:240 Note 1: The value for fmt, payload type and format is copied from step 1. |

Table 10.9.3.3-4: BYE (step 15, table 10.9.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8, Conditions A3, A6, A8.