3GPP TSG-RAN WG5 Meeting #92-e	R5-21abcd

# Electronic Meeting, 16th – 27th August 2021

Agenda Item:	TTCN sidebar

Source:	MCC TF160

Title:	NR Cell configuration types

Document for:	Discussion and endorsement

# 1	Introduction

In TS 38.508-1 clause 6.3.1, an Editor’s Note remains, which should be resolved:

Editor’s Note: To define different types of SS cell configurations. It may be similar as defined in 3GPP TS 36.508 [2], clause 6.3.3 and 6.3.4 i.e. full, minimum uplink, broadcast only and virtual cell configuration. But details are FFS and depending on different connectivity options (MR-DC and SA).

# 2	Current status

## 2.1	In LTE

For LTE, at some point in time the concept of different E-UTRA cell configuration types had been introduced on request from some SS Vendor(s), see TS 36.508 clauses 6.3.3 / 6.3.4, TS 36.523-3 clause 7.4.1.

In the LTE ASPs it is configurable using CellConfigCapability_Type and CA_CellInitialConfig_Type. To be used in TTCN it is required that first the cell configuration type is explicitly specified in the test case prose.

CellConfigCapability_Type:

- In TS 36.523-1 ‘DL only’ is used in multiple measurement test cases, ‘Min UL’ is used only once (TC 6.1.2.8).

CA_CellInitialConfig_Type:

- By default an E-UTRA cell used as SCell in the test case is configured as Scell_Inactive. When required by the test case sequence, it is configured as Scell_Active.

## 2.2	In NR

For NR, TS 38.508-1 only contain the Editor’s Note mentioned in section 1.

In the NR ASPs the above concepts used for LTE were ported during initial NR ASPs creation. We have the types NR_CellCapability_Type and NR_CellInitialCAConfig_Type.

NR_CellCapability_Type:

- In TS 38.523-1 ‘DL only’ can only be found being used in 5 measurement test cases. ‘Min UL’ cannot be found at all.

NR_CellInitialCAConfig_Type:

- By default an NR cell used as SCell in the test case is configured as Scell_Inactive. When required by the test case sequence, it is configured as Scell_Active.

# 3	Discussion

For each of NR_CellCapability_Type and NR_CellInitialCAConfig_Type, there are two options: add in TS 38.508-1 the specification matching with the usage of the ASP type, or delete both Editor’s Note + the ASP type.

NR_CellCapability_Type seems to be very seldomly used => It can be deleted.

NR_CellInitialCAConfig_Type is used consistently by TTCN for NR CA test cases, but it is not clear to TF160 whether it is used by any SS adaptation layer.

If we were to delete both ASP types, it would mean the following ASP updates:

NR_CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigInfo_Type |  |  |
| Comment | common information for initial cell configuration or reconfiguration; in case of reconfiguration omit means 'keep configuration as it is' |  |  |
| CellConfigCommon | NR_CellConfigCommon_Type | opt | common configuration parameters which are not specific to physical layer (or any other layer) |
| PhysicalLayer | NR_CellConfigPhysicalLayer_Type | opt | Physical layer configuration: physical channels, signals and BWPs for UL and DL; DCI |
| BcchConfig | NR_BcchConfig_Type | opt | configuration of BCCH/BCH; SS is triggered to configure RLC/MAC accordingly; BCCH data on the PDSCH is distinguished by the SI-RNTI PBCH: MIB; PDSCH: scheduling and resource allocation; SIBs |
| PcchConfig | NR_PcchConfig_Type | opt | configuration of PCCH/PCH; SS is triggered to configure RLC/MAC accordingly; PCCH data on the PDSCH is distinguished by the P-RNTI (needed even to modify SI => shall be configured for CELL_BROADCASTING) |
| RachProcedureConfig | NR_RachProcedureConfig_Type | opt | to configure the SS's behaviour for the RACH procedure; may be omitted at initial configuration e.g. in case that the cell shall not have an uplink; NOTE: there is no way to explicitly remove the RACH procedure configuration after it has been configured for a cell |
| DcchDtchConfig | NR_DcchDtchConfig_Type | opt | Parameters related to DCCH/DTCH in UL and DL |
| ServingCellConfig | NR_ServingCellConfig_Type | opt | To be configured at initial configuration of a cell: for non-CA scenarios it shall be either 'SpCell' or 'None' ('None' applies for pure neighbouring cells) |

The associated prose update can be found in R5-215408.