3GPP TSG-RAN5 Meeting #93-e	R5-217302

# Electronic Meeting, 08th – 19th Nov. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0522 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity TP analysis for DC_3A_n78C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The reference sensitivity TP analysis for DC_3A_n78C was missing in 38.905. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the reference sensitivity TP analysis for DC_3A_n78C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference sensitivity TP analysis for DC_3A_n78C would remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.3	Test point analysis per EN-DC configuration

4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 EN-DC test cases in TS 38.521-3 [4] clause 7. The analyses are performed per EN-DC configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration | Single UL | Refsens exception, victim band (NOTE 2) | Requirement coverage (NOTE 2) | Comments |
| --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 | Only single UL requirements defined | 5 | Exception | Added at RAN5#92-e |
| 12 | Only single UL requirements defined | 12 | Exception | Added at RAN5#92-e |
| 38 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 41 | Yes | N/A | NE | Added at RAN5#88-e |
| 48 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 71 | Yes | 71 | Exception | Added at RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_3A_n3A | Yes | 3 | Exception | Added at RAN5#88-e |
| DC_5A_n5A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_7A_n7A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_41A_n41A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_48A_n48A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_66A_n66A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_71A_n71A | Only single UL requirements defined | N/A | NE | Added at RAN5#92-e |
| DC_XA-nYA (NOTE 1) |  |  |  |  |
| DC_1A_n3A | Yes | 1 (IMD3) CBI | NE IMD3 CBI, CBI avoid | Added at RAN5#89-e |
| DC_1A_n78A | No | 1 (IMD4) | NE IMD4 | Added at RAN5#88-e |
| DC_3A_n1A | Yes | 1 (IMD3) CBI | NE IMD3 CBI, CBI avoid | Added at RAN5#89-e |
| DC_3A_n78A | Yes | 3 (IMD2) 3 (IMD4) 3 (HD) n78 (HM) | NE IMD2 PC2&PC3 IMD4 PC2&PC3 HD HM, HM avoid | Added at RAN5#89-e |
| DC_7A_n1A | No | N/A | NE | Added at RAN5#90-e |
| DC_7A_n3A | No | 7 (IMD4¦3*fB3-1*fB7¦) | NE, IMD4 | Added at RAN5#90-e |
| DC_7A_n78A | No | N/A | NE | Added at RAN5#89-e |
| DC_8A_n1A | No | n1 (IMD4 ¦2*fB1-2*fB8¦) | NE, IMD4 | Added at RAN5#90-e |
| DC_8A_n3A | No | 8 (IMD4 ¦3*fB8-1*fB3¦), n3 (IMD5 ¦4*fB8-1*fB3¦) | NE, IMD4, IMD5 | Added at RAN5#90-e |
| DC_8A_n77A | No | 8 (HD4), 8 (IMD4) | NE HD, HD avoid IMD4 | Added at RAN5#92-e |
| DC_11A_n77A | No | N/A | NE | Added at RAN5#91-e |
| DC_11A_n78A | No | N/A | NE | Added at RAN5#91-e |
| DC_11A_n79A | No | 11 (HM) | NE HM, HM avoid | Added at RAN5#92-e |
| DC_19A_n79A | No | 19 (HM ¦5*fB19DL-1*fB79¦), | NE, HM, HM avoid | Added at RAN5#90-e |
| DC_20A_n1A | No | N/A | NE | Added at RAN5#90-e |
| DC_20A_n3A | No | n3 (IMD4), 20 (IMD4) | NE IMD4 | Added at RAN5#89-e |
| DC_20A_n78A | No | 20 (IMD4), n78 (HD) | NE IMD4, HD, HD avoid | Added at RAN5#88-e |
| DC_25A_n41A | No | N/A | NE | Added at RAN5#91-e |
| DC_26A_n41A | No | n41 (HD3), 26 (IMD3) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n77A | No | n77 (HD4), 26 (IMD4) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n78A | No | n78 (HD4), 26 (IMD4) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n79A | No | 26 (HM ¦4*fB26DL-1*fB79¦) | NE HM, HM avoid | Added at RAN5#92-e |
| DC_28A_n77A | No | n77 (HD), n77 (HM), 28 (IMD5) | NE (HD avoid), HD, HM, IMD5 | Added at RAN5#92-e |
| DC_28A_n78A | No | n78 (HD) , 28 (IMD5) | NE (HD avoid), HD, IMD5 | Added at RAN5#92-e |
| DC_40A_n1A | No | N/A | NE | Added at RAN5#89-e |
| DC_40A_n78A | No | 40 (HM) | NE HM, HM avoid | Added at RAN5#89-e |
| DC_41A_n77A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦), 41 (CBI), n77 (CBI) | NE HM CBI | Added at RAN5#92-e |
| DC_41A_n78A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦), 41 (CBI), n78 (CBI) | NE HM CBI | Added at RAN5#92-e |
| DC_42A_n77A | Only single UL requirements defined | N/A | NE | Added at RAN5#91-e |
| DC_48A_n66A | No | 48 (HD2) 66 (IMD5) | NE (anchor agnostic) HD2 IMD5 | Added at RAN5#92-e |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 HD avoid: single carrier requirements apply with aggressor of UL Harmonic Distortion active. HM avoid: single carrier requirements apply with aggressor of RX Harmonic Mixing active. CBI avoid: single carrier requirements apply with aggressor of Cross Band Isolation active. NOTE 3: HM cannot be avoided with aggressor still active since CBI exception always exists. Therefore, no HM avoid test point is added. |  |  |  |  |

Table 4.3.3.1-2: Reference Sensitivity test cases per EN-DC configuration (3CC)

| Band or band configuration | Single UL |  | UL config | Refsens exception, victim band |  | Requirement coverage (NOTE 2) |  | Fallback DC configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |  |  |  |
| 41 | Yes |  | DC_(n)41AA | N/A |  | NE |  | DC_(n)41AA DC_41A_n41A | Added at RAN5#89-e |
| 41 | Yes |  | DC_41A_n41A | N/A |  | NE |  | DC_(n)41AA DC_41A_n41A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined |  | DC_(n)48AA | N/A |  | NE |  | DC_(n)48AA DC_48A_n48A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined |  | DC_48A_n48A | N/A |  | NE |  | DC_(n)48AA DC_48A_n48A | Added at RAN5#89-e |
| DC_XA_nYC |  |  |  |  |  |  |  |  |  |
| DC_3A_n78C | Yes |  | DC_3A_n78A | N/A |  | NE |  |  | Added at RAN5#93-e |
| DC_XA_nY(2A) |  |  |  |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |  |  |  |
| DC_XA_nYA-nZA |  |  |  |  |  |  |  |  |  |
| DC_1A_n28A-n78A | No |  | DC_1A_n28A | n78 (3 band IMD3) |  | NE, IMD3 |  |  | Added at RAN5#92-e |
|  | No |  | DC_1A_n78A | n28 (3 band IMD5) |  | NE, IMD5 |  |  | Added at RAN5#92-e |
| DC_3A_n28A-n78A | No |  | DC_3A_n28A | n78 (3 band IMD5) |  | NE, IMD5 |  |  | Added at RAN5#92-e |
|  | Yes |  | DC_3A_n78A | No 3CC exception |  | No test required |  |  |  |
| DC_7A_n28A-n78A | No |  | DC_7A_n28A | n78 (3 band IMD2) |  | NE, IMD2 |  |  | Added at RAN5#92-e |
|  | No |  | DC_7A_n78A | n28 (3 band IMD2) |  | NE, IMD2 |  |  |  |
| DC_XA-YA_nZA |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n28A | No |  | DC_1A_n28A | 3 (3 band IMD5) |  | IMD5 |  |  | Added at RAN5#92-e |
|  | No |  | DC_3A_n28A | 1 (3 band IMD4) |  | IMD4 |  |  | Added at RAN5#92-e |
| DC_1A-7A_n3A | Yes |  | DC_1A_n3A | No 3CC exception |  | No test required |  |  |  |
|  | No |  | DC_7A_n3A |  |  |  |  |  |  |
| DC_1A-7A_n28A | No |  | DC_1A_n28A | 7 (3 band IMD2) |  | IMD2 |  |  | Added at RAN5#92-e |
|  | No |  | DC_7A_n28A | No 3 band exception |  | No test required |  |  | Added at RAN5#92-e |
| DC_1A-7A_n78A | No |  | DC_1A_n78A | 7 (IMD4 ¦3*fB1-1*fB78¦) |  | IMD4 |  |  |  |
|  | No |  | DC_7A_n78A | 1 (IMD4 ¦2*fB7-2*fB78¦) |  | IMD4 |  |  |  |
| DC_1A-8A_n3A | Yes |  | DC_1A_n3A | No 3CC exception |  | No test required |  |  |  |
|  | No |  | DC_8A_n3A |  |  |  |  |  |  |
| DC_1A-20A_n3A | Yes |  | DC_1A_n3A | No 3CC exception |  | No test required |  |  |  |
|  |  | No |  | DC_20A_n3A |  |  |  |  |  |  |
| DC_1A-20A_n78A | No |  | DC_1A_n78A | 20 (IMD5 ¦ |  | IMD5 |  |  |  |
|  | No |  | DC_20A_n78A | 1 (IMD3 |  | IMD3 |  |  |  |
| DC_1A-28A_n3A | Yes |  | DC_1A_n3A | 1 (3 band IMD4) |  | IMD4 |  | Yes | Added at RAN5#90-e |
|  | No |  | DC_28A_n3A | No 3CC exception |  | - |  | No |  |
| DC_1A-28A_n78A | No |  | DC_1A_n78A | 28 (3 band IMD3) |  | IMD3 |  |  | Added at RAN5#92-e |
|  | No |  | DC_28A_n78A | 1 (3 band IMD5) |  | IMD5 |  | DC_28A_n78A |  |
| DC_3A-7A_n1A | Yes |  | DC_3A_n1A | No 3CC exception |  | No test required |  |  | Added at RAN5#90-e |
|  | No |  | DC_7A_n1A | No 3CC exception |  |  |  |  |  |
| DC_3A-7A_n28A | No |  | DC_3A_n28A | 7 (3 band IMD3) |  | IMD3 |  |  | Added at RAN5#92-e |
|  | No |  | DC_7A_n28A | 3 (3 band IMD2) |  | IMD2 |  |  | Added at RAN5#92-e |
| DC_3A-20A_n1 | Yes |  | DC_3A_n1A | No 3CC exception |  | No test required |  |  |  |
|  | No |  | DC_20A_n1A |  |  |  |  |  |  |
| DC_3A-20A_n28A | No |  | DC_3A_n28A | No 3CC exception |  | No test required |  |  | Added at RAN5#92-e |
|  | No |  | DC_20A_n28A | 3 (3 band IMD4) |  | IMD4 |  |  | Added at RAN5#92-e |
| DC_3A-20A_n78 | Yes |  | DC_3A_n78A | No 3CC exception |  |  |  |  |  |
|  | No |  | DC_20A_n78A | 3 (3 band IMD3) |  | IMD3 |  |  |  |
| DC_3A-28A_n78A | Yes |  | DC_3A_n78A | No 3CC exception |  |  |  | DC_3A_n78A | Added at RAN5#92-e |
|  | No |  | DC_28A_n78A | 3 (3 band IMD3) |  | IMD3 |  | DC_28A_n78A |  |
| DC_3A-40A_n1A | Yes |  | DC_3A_n1A | 40 (3 band IMD5) |  | IMD5 if UE supporting dual UL |  |  |  |
|  | No |  | DC_40A_n1A | No 3CC exception |  |  |  |  |  |
| DC_7A-20A_n1A | No |  | DC_20A_n1A | 20 (3 band IMD5) |  | IMD5 |  |  | Added at RAN5#90-e |
|  | No |  | DC_7A_n1A | No 3CC exception |  | _ |  |  |  |
| DC_7A-20A_n3A | No |  | DC_7A_n3A | 20 (IMD2 ¦1*fB7-1*fB3) |  | NE, IMD2 |  |  |  |
|  | No |  | DC_20A_n3A | 7 (IMD2 ¦1*fB3+1*fB20) |  | NE, IMD2 |  |  |  |
| DC_7A-20A_n28A | No |  | DC_7A_n28A | No 3CC exception |  | No test required |  |  | Added at RAN5#92-e |
|  | No |  | DC_20A_n28A | 7 (3 band IMD5) |  | IMD 5 |  |  |  |
| DC_7A-20A_n78A | No |  | DC_7A_n78A | 20 (3 band IMD2) 20 (3 band IMD5) |  | IMD2, IMD5 |  |  |  |
|  | No |  | 2 DC_0A_n78A | 7 (3 band IMD2) |  | IMD2 |  |  |  |
| DC_7A-28A_n3A | No |  | DC_7A_n3A | 28 (3 band IMD2) |  | IMD2 |  |  | Added at RAN5#90-e |
|  | No |  | DC_28A_n3A | 7 (3 band IMD3) |  | IMD3 |  |  |  |
| DC_7A-28A_n78A | No |  | DC_7A_n78A | 28 (3 band IMD2) 28 (3 band IMD5) |  | IMD2, IMD5 |  |  | Added at RAN5#92-e |
|  | No |  | DC_28A_n78A | 7 (3 band IMD2) |  | IMD2 |  | DC_28A_n78A |  |
| DC_(n)XAA-nYA |  |  |  |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |  |  |  |
| DC_XA_nXA_nYA |  |  |  |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |  |  |  |

Table 4.3.3.1-3: Reference Sensitivity test cases per EN-DC configuration (4CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (NOTEote 2) | Fallback DC configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XDA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | NE | DC_(n)41CA DC_41C_n41A | Added at RAN5#89-e |
| 41 | Yes | DC_41A_n41A | N/A | NE | N/A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | NE | DC_(n)48CA DC_48C_n48A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | NE | N/A | Added at RAN5#89-e |
| DC_XA_nY(2A)-nZA |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| DC_XA_nYA-nZC |  |  |  |  |  |  |
| FFS |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

<End of Changes>