3GPP TSG-RAN5 Meeting #93-e	R5-217590

# Electronic Meeting, 8- 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2158 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for CA_n257x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test frequencies for CA_n257G to CA_n257M have been specified for both adjusted channel spacing as for nominal channel spacing. Adjusted channel spacing was introduced based on the discussion paper in R5-195408 at RAN5#83 to achieve the same value of channel spacing for both SCS=60 and SCS=120kHz based on current discussion in RAN4 for how to calculate nominal channel spacing. One of the options for calculating nominal channel spacing caused different values for SCS=60 kHz and SCS=120 kHz. However the formula for nominal channel spacing and FR2 agreed by RAN4 and included in TS 38.101-2 does not cause differnet values of nominal channel spacing for different SCS. The formula in TS 38.101-2 is: For CA_n257G to CA_n257M the CC(n) CBW = CC(n+1) = 100 MHz the formula for nominal channel spacing willl be Floor(200/0.06*2n+1)/ 0.06*2n SCS=60 kHz => u0 =2 => n=0 => NCS=99.96 MHz SCS=120 kHz => u0 =3 => n=1 => NCS=99.96 MHz Similarly for CC CBW combo 50+100 MHz the nominal channel spacing is 74.4 MHz for both SCS=60 and SCS=120kHz. Thus there is no need to have specific tables with adjusted channel spacing for CA_n257G to CA_n257M. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Marked all tables for case of adjusted channel spacing as Void for CA_n257G to CA_n257M |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not needed test frequency tables remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.3.1.6, 4.3.1.2.3.1.7, 4.3.1.2.3.1.8, 4.3.1.2.3.1.9, , 4.3.1.2.3.1.10, 4.3.1.2.3.1.11 and 4.3.1.2.3.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.6	CA_n257G

Table 4.3.1.2.3.1.6-1: Void

Table 4.3.1.2.3.1.6-2: NR Intra-Band contiguous CA configuration CA_n257G, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  |  | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 |  | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 | 66 | Downlink | Low |  | 26525.04 | 2054583 | 26501.28 | 2054187 | 0 | 120 |  |  | 22388 | 2054683 |  | 4 | 13 | 1 (8) |  | 21 |
|  |  |  |  |  | & | Mid |  | 27949.92 | 2078331 | 27852.72 | 2076711 | 102 |  |  |  | 22470 | 2078299 |  | 4 | 2 | 1 (8) |  | 112 |
|  |  |  |  |  | Uplink | High |  | 29375.52 | 2102091 | 28988.88 | 2095647 | 504 |  |  |  | 22553 | 2102203 |  | 4 | 14 | 1 (8) |  | 526 |
|  | Channel spacing CC1-CC2=74.40 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low |  | 26599.44 | 2055823 | 26551.92 | 2055031 | 0 | 120 |  |  | 22391 | 2055547 |  | 0 | 15 | 1 (8) |  | 23 |
|  |  |  |  |  | & | Mid |  | 28024.32 | 2079571 | 27903.36 | 2077555 | 102 |  |  |  | 22473 | 2079163 |  | 0 | 4 | 1 (8) |  | 114 |
|  |  |  |  |  | Uplink | High |  | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  |  |  | 22555 | 2102779 |  | 0 | 0 | 0 (0) |  | 504 |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink |  | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 |  | 2054683 |  | 8 | 11 |  | 1 (8) | 19 |  |
|  |  |  |  |  | & |  | Mid | 27949.92 | 2078331 | 27828.96 | 2076315 | 102 |  |  | 22469 |  | 2078011 |  | 4 | 11 |  | 1 (8) | 121 |  |
|  |  |  |  |  | Uplink |  | High | 29349.96 | 2101665 | 28939.56 | 2094825 | 504 |  |  | 22550 |  | 2101339 |  | 10 | 10 |  | 1 (8) | 522 |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink |  | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 |  | 2056123 |  | 10 | 0 |  | 0 (0) | 0 |  |
|  |  |  |  |  | & |  | Mid | 28049.88 | 2079997 | 27928.92 | 2077981 | 102 |  |  | 22474 |  | 2079451 |  | 6 | 0 |  | 0 (0) | 102 |  |
|  |  |  |  |  | Uplink |  | High | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  |  | 22555 |  | 2102779 |  | 0 | 0 |  | 0 (0) | 504 |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.6-3: Void

Table 4.3.1.2.3.1.6-4: NR Intra-Band contiguous CA configuration CA_n257G, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] |  |  | carrierBandwidth [PRBs] | Range |  |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] |  | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] |  | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 |  | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 |  | 120 | 32 |  | Downlink | Low |  |  | 26525.04 | 2054583 | 26502 | 2054199 |  | 0 | 120 |  | 22388 |  |  | 2054683 |  | 2 | 6 | 1 (4) |  | 20 |
|  |  |  |  |  |  |  | & | Mid |  |  | 27950.04 | 2078333 | 27780.12 | 2075501 |  | 102 |  |  | 22470 |  |  | 2078299 |  | 7 | 0 | 1 (4) |  | 212 |
|  |  |  |  |  |  |  | Uplink | High |  |  | 29375.52 | 2102091 | 28626.72 | 2089611 |  | 504 |  |  | 22553 |  |  | 2102203 |  | 8 | 6 | 1 (4) |  | 1028 |
|  | Channel spacing CC1-CC2=74.40 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 |  | 120 | 66 |  | Downlink | Low |  |  | 26599.44 | 2055823 | 26551.92 | 2055031 |  | 0 | 120 |  | 22391 |  |  | 2055547 |  | 6 | 7 | 1 (4) |  | 22 |
|  |  |  |  |  |  |  | & | Mid |  |  | 28024.44 | 2079573 | 27830.04 | 2076333 |  | 102 |  |  | 22473 |  |  | 2079163 |  | 11 | 1 | 1 (4) |  | 214 |
|  |  |  |  |  |  |  | Uplink | High |  |  | 29449.92 | 2103331 | 28676.64 | 2090443 |  | 504 |  |  | 22555 |  |  | 2102779 |  | 0 | 0 | 0 (0) |  | 1008 |
| 100+100 | CC1 | 100 | 120 |  |  | 66 | Downlink |  | Low | 26550 |  | 2054999 | 26502.48 |  | 2054207 | 0 | 120 |  |  | 22388 | 2054683 |  | 10 |  | 5 |  | 1 (4) | 18 |  |
|  |  |  |  |  |  |  | & |  | Mid | 27950.04 |  | 2078333 | 27755.64 |  | 2075093 | 102 |  |  |  | 22469 | 2078011 |  | 7 |  | 5 |  | 1 (4) | 222 |  |
|  |  |  |  |  |  |  | Uplink |  | High | 29349.96 |  | 2101665 | 28576.68 |  | 2088777 | 504 |  |  |  | 22550 | 2101339 |  | 5 |  | 5 |  | 1 (4) | 1026 |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 |  |  | 66 | Downlink |  | Low | 26649.96 |  | 2056665 | 26602.44 |  | 2055873 | 0 | 120 |  |  | 22393 | 2056123 |  | 5 |  | 0 |  | 0 (0) | 0 |  |
|  |  |  |  |  |  |  | & |  | Mid | 28050 |  | 2079999 | 27855.6 |  | 2076759 | 102 |  |  |  | 22474 | 2079451 |  | 2 |  | 0 |  | 0 (0) | 204 |  |
|  |  |  |  |  |  |  | Uplink |  | High | 29449.92 |  | 2103331 | 28676.64 |  | 2090443 | 504 |  |  |  | 22555 | 2102779 |  | 0 |  | 0 |  | 0 (0) | 1008 |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.7	CA_n257H

Table 4.3.1.2.3.1.7-1: Void

Table 4.3.1.2.3.1.7-2: NR Intra-Band contiguous CA configuration CA_n257H, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] |  | SCS [kHz] | carrierBandwidth [PRBs] |  | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] |  | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 |  | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 | 50 | 60 |  |  | 66 | Downlink | Low |  | 26525.04 | 2054583 | 26501.28 | 2054187 | 0 | 120 |  | 22388 |  |  | 2054683 | 4 |  |  | 13 | 1 (8) | 21 |
| +100 |  |  |  |  |  |  | & | Mid |  | 27900 | 2077499 | 27802.8 | 2075879 | 102 |  |  | 22467 |  |  | 2077435 | 8 |  |  | 7 | 0 (0) | 109 |
|  |  |  |  |  |  |  | Uplink | High |  | 29275.56 | 2100425 | 28888.92 | 2093981 | 504 |  |  | 22547 |  |  | 2100475 | 2 |  |  | 9 | 1 (8) | 521 |
|  | Channel spacing CC1-CC2=74.40 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 |  |  | 132 | Downlink | Low |  | 26599.44 | 2055823 | 26551.92 | 2055031 | 0 | 120 |  | 22391 |  |  | 2055547 | 0 |  |  | 15 | 1 (8) | 23 |
|  |  |  |  |  |  |  | & | Mid |  | 27974.4 | 2078739 | 27853.44 | 2076723 | 102 |  |  | 22470 |  |  | 2078299 | 4 |  |  | 1 | 1 (8) | 111 |
|  |  |  |  |  |  |  | Uplink | High |  | 29349.96 | 2101665 | 28939.56 | 2094825 | 504 |  |  | 22550 |  |  | 2101339 | 10 |  |  | 10 | 1 (8) | 522 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1l) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 |  |  | 132 | Downlink | Low |  | 26699.4 | 2057489 | 26651.88 | 2056697 | 0 | 120 |  | 22396 |  |  | 2056987 | 2 |  |  | 4 | 0 (0) | 4 |
|  |  |  |  |  |  |  | & | Mid |  | 28074.36 | 2080405 | 27953.4 | 2078389 | 102 |  |  | 22476 |  |  | 2080027 | 6 |  |  | 6 | 1 (8) | 116 |
|  |  |  |  |  |  |  | Uplink | High |  | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  |  | 22555 |  |  | 2102779 | 0 |  |  | 0 | 0 (0) | 504 |
| 100+100 | CC1 | 100 |  | 60 | 132 |  | Downlink | Low | 26550 |  | 2054999 | 26502.48 | 2054207 | 0 | 120 |  |  | 22388 | 2054683 |  | 8 |  | 11 |  | 1 (8) | 19 |
| +100 |  |  |  |  |  |  | & | Mid | 27900 |  | 2077499 | 27779.04 | 2075483 | 102 |  |  |  | 22466 | 2077147 |  | 8 |  | 8 |  | 1 (8) | 118 |
|  |  |  |  |  |  |  | Uplink | High | 29250 |  | 2099999 | 28839.6 | 2093159 | 504 |  |  |  | 22544 | 2099611 |  | 8 |  | 5 |  | 1 (8) | 517 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 |  | 60 | 132 |  | Downlink | Low | 26649.96 |  | 2056665 | 26602.44 | 2055873 | 0 | 120 |  |  | 22393 | 2056123 |  | 10 |  | 0 |  | 0 (0) | 0 |
|  |  |  |  |  |  |  | & | Mid | 27999.96 |  | 2079165 | 27879 | 2077149 | 102 |  |  |  | 22472 | 2078875 |  | 10 |  | 13 |  | 1 (8) | 123 |
|  |  |  |  |  |  |  | Uplink | High | 29349.96 |  | 2101665 | 28939.56 | 2094825 | 504 |  |  |  | 22550 | 2101339 |  | 10 |  | 10 |  | 1 (8) | 522 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 |  | 60 | 132 |  | Downlink | Low | 26749.92 |  | 2058331 | 26702.4 | 2057539 | 0 | 120 |  |  | 22399 | 2057851 |  | 0 |  | 6 |  | 0 (0) | 6 |
|  |  |  |  |  |  |  | & | Mid | 28099.92 |  | 2080831 | 27978.96 | 2078815 | 102 |  |  |  | 22477 | 2080315 |  | 0 |  | 3 |  | 0 (0) | 105 |
|  |  |  |  |  |  |  | Uplink | High | 29449.92 |  | 2103331 | 29039.52 | 2096491 | 504 |  |  |  | 22555 | 2102779 |  | 0 |  | 0 |  | 0 (0) | 504 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.7-3: Void

Table 4.3.1.2.3.1.7-4: NR Intra-Band contiguous CA configuration CA_n257H, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] |  | SCS [kHz] |  | carrierBandwidth [PRBs] |  | Range |  |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  |  | SS block SCS [kHz] |  | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 |  | 50 |  | 120 |  | 32 | Downlink | Low |  |  | 26525.04 | 2054583 | 26502 | 2054199 |  | 0 |  |  | 120 | 22388 |  |  | 2054683 |  | 2 | 6 | 1 (4) |  | 20 |
| +100 |  |  |  |  |  |  |  | & | Mid |  |  | 27900 | 2077499 | 27730.08 | 2074667 |  | 102 |  |  |  | 22467 |  |  | 2077435 |  | 4 | 3 | 0 (0) |  | 210 |
|  |  |  |  |  |  |  |  | Uplink | High |  |  | 29275.56 | 2100425 | 28526.76 | 2087945 |  | 504 |  |  |  | 22547 |  |  | 2100475 |  | 1 | 4 | 1 (4) |  | 1024 |
|  | Channel spacing CC1-CC2=74.40 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  | 100 |  | 120 |  | 66 | Downlink | Low |  |  | 26599.44 | 2055823 | 26551.92 | 2055031 |  | 0 |  |  | 120 | 22391 |  |  | 2055547 |  | 6 | 7 | 1 (4) |  | 22 |
|  |  |  |  |  |  |  |  | & | Mid |  |  | 27974.4 | 2078739 | 27780 | 2075499 |  | 102 |  |  |  | 22470 |  |  | 2078299 |  | 8 | 0 | 1 (4) |  | 212 |
|  |  |  |  |  |  |  |  | Uplink | High |  |  | 29349.96 | 2101665 | 28576.68 | 2088777 |  | 504 |  |  |  | 22550 |  |  | 2101339 |  | 5 | 5 | 1 (4) |  | 1026 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 |  | 100 |  | 120 |  | 66 | Downlink | Low |  |  | 26699.4 | 2057489 | 26651.88 | 2056697 |  | 0 |  |  | 120 | 22396 |  |  | 2056987 |  | 1 | 2 | 0 (0) |  | 4 |
|  |  |  |  |  |  |  |  | & | Mid |  |  | 28074.36 | 2080405 | 27879.96 | 2077165 |  | 102 |  |  |  | 22476 |  |  | 2080027 |  | 3 | 3 | 1 (4) |  | 218 |
|  |  |  |  |  |  |  |  | Uplink | High |  |  | 29449.92 | 2103331 | 28676.64 | 2090443 |  | 504 |  |  |  | 22555 |  |  | 2102779 |  | 0 | 0 | 0 (0) |  | 1008 |
| 100+100 | CC1 | 100 |  | 120 |  | 66 |  | Downlink |  | Low | 26550 |  | 2054999 | 26502.48 | 2054207 | 0 |  | 120 |  |  | 22388 | 2054683 |  |  | 10 |  | 5 | 1 (4) | 18 |  |
| +100 |  |  |  |  |  |  |  | & |  | Mid | 27900 |  | 2077499 | 27705.6 | 2074259 | 102 |  |  |  |  | 22466 | 2077147 |  |  | 4 |  | 4 | 1 (4) | 220 |  |
|  |  |  |  |  |  |  |  | Uplink |  | High | 29250 |  | 2099999 | 28476.72 | 2087111 | 504 |  |  |  |  | 22544 | 2099611 |  |  | 10 |  | 2 | 1 (4) | 1020 |  |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 |  | 120 |  | 66 |  | Downlink |  | Low | 26649.96 |  | 2056665 | 26602.44 | 2055873 | 0 |  | 120 |  |  | 22393 | 2056123 |  |  | 5 |  | 0 | 0 (0) | 0 |  |
|  |  |  |  |  |  |  |  | & |  | Mid | 27999.96 |  | 2079165 | 27805.56 | 2075925 | 102 |  |  |  |  | 22472 | 2078875 |  |  | 11 |  | 6 | 1 (4) | 224 |  |
|  |  |  |  |  |  |  |  | Uplink |  | High | 29349.96 |  | 2101665 | 28576.68 | 2088777 | 504 |  |  |  |  | 22550 | 2101339 |  |  | 5 |  | 5 | 1 (4) | 1026 |  |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 |  | 120 |  | 66 |  | Downlink |  | Low | 26749.92 |  | 2058331 | 26702.4 | 2057539 | 0 |  | 120 |  |  | 22399 | 2057851 |  |  | 0 |  | 3 | 0 (0) | 6 |  |
|  |  |  |  |  |  |  |  | & |  | Mid | 28099.92 |  | 2080831 | 27905.52 | 2077591 | 102 |  |  |  |  | 22477 | 2080315 |  |  | 6 |  | 1 | 0 (0) | 206 |  |
|  |  |  |  |  |  |  |  | Uplink |  | High | 29449.92 |  | 2103331 | 28676.64 | 2090443 | 504 |  |  |  |  | 22555 | 2102779 |  |  | 0 |  | 0 | 0 (0) | 1008 |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.8	CA_n257I

Table 4.3.1.2.3.1.8-1: Void

Table 4.3.1.2.3.1.8-2: NR Intra-Band contiguous CA configuration CA_n257I, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

| CBW combination |  | CC Note 2 | CBW [MHz] | SCS [kHz] |  | carrierBandwidth [PRBs] |  | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  |  | SS block SCS [kHz] |  | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 |  | CORESET#0 Index (Offset [RBs]) Note 4 |  | offsetToPointA (SIB1) [PRBs] Note 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CC1 |  | 50 | 60 | 66 |  | Downlink |  | Low | 26525.04 | 2054583 |  | 26501.28 | 2054187 |  | 0 |  |  | 120 | 22388 |  | 2054683 | 4 |  |  | 13 |  | 1 (8) | 21 |
| +100+100 |  |  |  |  |  |  | & |  | Mid | 27849.96 | 2076665 |  | 27752.76 | 2075045 |  | 102 |  |  |  | 22464 |  | 2076571 | 2 |  |  | 5 |  | 0 (0) | 107 |
|  |  |  |  |  |  |  | Uplink |  | High | 29175.6 | 2098759 |  | 28788.96 | 2092315 |  | 504 |  |  |  | 22541 |  | 2098747 | 0 |  |  | 4 |  | 1 (8) | 516 |
|  | Channel spacing CC1-CC2=74.40 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  | 100 | 60 | 132 |  | Downlink |  | Low | 26599.44 | 2055823 |  | 26551.92 | 2055031 |  | 0 |  |  | 120 | 22391 |  | 2055547 | 0 |  |  | 15 |  | 1 (8) | 23 |
|  |  |  |  |  |  |  | & |  | Mid | 27924.36 | 2077905 |  | 27803.4 | 2075889 |  | 102 |  |  |  | 22467 |  | 2077435 | 10 |  |  | 6 |  | 0 (0) | 108 |
|  |  |  |  |  |  |  | Uplink |  | High | 29250 | 2099999 |  | 28839.6 | 2093159 |  | 504 |  |  |  | 22544 |  | 2099611 | 8 |  |  | 5 |  | 1 (8) | 517 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 |  | 100 | 60 | 132 |  | Downlink |  | Low | 26699.4 | 2057489 |  | 26651.88 | 2056697 |  | 0 |  |  | 120 | 22396 |  | 2056987 | 2 |  |  | 4 |  | 0 (0) | 4 |
|  |  |  |  |  |  |  | & |  | Mid | 28024.32 | 2079571 |  | 27903.36 | 2077555 |  | 102 |  |  |  | 22473 |  | 2079163 | 0 |  |  | 4 |  | 1 (8) | 114 |
|  |  |  |  |  |  |  | Uplink |  | High | 29349.96 | 2101665 |  | 28939.56 | 2094825 |  | 504 |  |  |  | 22550 |  | 2101339 | 10 |  |  | 10 |  | 1 (8) | 522 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 |  | 100 | 60 | 132 |  | Downlink |  | Low | 26799.36 | 2059155 |  | 26751.84 | 2058363 |  | 0 |  |  | 120 | 22402 |  | 2058715 | 4 |  |  | 1 |  | 1 (8) | 9 |
|  |  |  |  |  |  |  | & |  | Mid | 28124.28 | 2081237 |  | 28003.32 | 2079221 |  | 102 |  |  |  | 22479 |  | 2080891 | 2 |  |  | 9 |  | 1 (8) | 119 |
|  |  |  |  |  |  |  | Uplink |  | High | 29449.92 | 2103331 |  | 29039.52 | 2096491 |  | 504 |  |  |  | 22555 |  | 2102779 | 0 |  |  | 0 |  | 0 (0) | 504 |
| 100+100 |  | CC1 | 100 | 60 |  | 132 |  | Downlink | Low | 26550 |  | 2054999 | 26502.48 | 2054207 | 0 |  | 120 |  |  | 22388 | 2054683 |  | 8 |  | 11 |  | 1 (8) |  | 19 |  |
| +100+100 |  |  |  |  |  |  |  | & | Mid | 27849.96 |  | 2076665 | 27729 | 2074649 | 102 |  |  |  |  | 22463 | 2076283 |  | 2 |  | 6 |  | 1 (8) |  | 116 |  |
|  |  |  |  |  |  |  |  | Uplink | High | 29150.04 |  | 2098333 | 28739.64 | 2091493 | 504 |  |  |  |  | 22538 | 2097883 |  | 6 |  | 0 |  | 1 (8) |  | 512 |  |
|  |  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 60 |  | 132 |  | Downlink | Low | 26649.96 |  | 2056665 | 26602.44 | 2055873 | 0 |  | 120 |  |  | 22393 | 2056123 |  | 10 |  | 0 |  | 0 (0) |  | 0 |  |
|  |  |  |  |  |  |  |  | & | Mid | 27949.92 |  | 2078331 | 27828.96 | 2076315 | 102 |  |  |  |  | 22469 | 2078011 |  | 4 |  | 11 |  | 1 (8) |  | 121 |  |
|  |  |  |  |  |  |  |  | Uplink | High | 29250 |  | 2099999 | 28839.6 | 2093159 | 504 |  |  |  |  | 22544 | 2099611 |  | 8 |  | 5 |  | 1 (8) |  | 517 |  |
|  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 60 |  | 132 |  | Downlink | Low | 26749.92 |  | 2058331 | 26702.4 | 2057539 | 0 |  | 120 |  |  | 22399 | 2057851 |  | 0 |  | 6 |  | 0 (0) |  | 6 |  |
|  |  |  |  |  |  |  |  | & | Mid | 28049.88 |  | 2079997 | 27928.92 | 2077981 | 102 |  |  |  |  | 22474 | 2079451 |  | 6 |  | 0 |  | 0 (0) |  | 102 |  |
|  |  |  |  |  |  |  |  | Uplink | High | 29349.96 |  | 2101665 | 28939.56 | 2094825 | 504 |  |  |  |  | 22550 | 2101339 |  | 10 |  | 10 |  | 1 (8) |  | 522 |  |
|  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 60 |  | 132 |  | Downlink | Low | 26849.88 |  | 2059997 | 26802.36 | 2059205 | 0 |  | 120 |  |  | 22405 | 2059579 |  | 2 |  | 3 |  | 1 (8) |  | 11 |  |
|  |  |  |  |  |  |  |  | & | Mid | 28149.84 |  | 2081663 | 28028.88 | 2079647 | 102 |  |  |  |  | 22480 | 2081179 |  | 8 |  | 5 |  | 0 (0) |  | 107 |  |
|  |  |  |  |  |  |  |  | Uplink | High | 29449.92 |  | 2103331 | 29039.52 | 2096491 | 504 |  |  |  |  | 22555 | 2102779 |  | 0 |  | 0 |  | 0 (0) |  | 504 |  |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.8-3: Void

Table 4.3.1.2.3.1.8-4: NR Intra-Band contiguous CA configuration CA_n257I, SCS=120 kHz, ΔFRaster 120 kHz nominal channel spacing

| CBW combination | CC Note 2 |  | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  |  |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] |  | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 |  | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 |  | CC1 | 50 | 120 | 32 |  | Downlink |  | Low | 26525.04 | 2054583 |  |  | 26502 | 2054199 |  | 0 | 120 |  | 22388 |  | 2054683 |  | 2 |  | 6 | 1 (4) | 20 |
| +100+100 |  |  |  |  |  |  | & |  | Mid | 27849.96 | 2076665 |  |  | 27680.04 | 2073833 |  | 102 |  |  | 22464 |  | 2076571 |  | 1 |  | 2 | 0 (0) | 208 |
|  |  |  |  |  |  |  | Uplink |  | High | 29175.6 | 2098759 |  |  | 28426.8 | 2086279 |  | 504 |  |  | 22541 |  | 2098747 |  | 6 |  | 1 | 1 (4) | 1018 |
|  |  | Channel spacing CC1-CC2=74.40 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 120 | 66 |  | Downlink |  | Low | 26599.44 | 2055823 |  |  | 26551.92 | 2055031 |  | 0 | 120 |  | 22391 |  | 2055547 |  | 6 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  |  |  | & |  | Mid | 27924.36 | 2077905 |  |  | 27729.96 | 2074665 |  | 102 |  |  | 22467 |  | 2077435 |  | 5 |  | 3 | 0 (0) | 210 |
|  |  |  |  |  |  |  | Uplink |  | High | 29250 | 2099999 |  |  | 28476.72 | 2087111 |  | 504 |  |  | 22544 |  | 2099611 |  | 10 |  | 2 | 1 (4) | 1020 |
|  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 120 | 66 |  | Downlink |  | Low | 26699.4 | 2057489 |  |  | 26651.88 | 2056697 |  | 0 | 120 |  | 22396 |  | 2056987 |  | 1 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  |  |  | & |  | Mid | 28024.32 | 2079571 |  |  | 27829.92 | 2076331 |  | 102 |  |  | 22473 |  | 2079163 |  | 0 |  | 2 | 1 (4) | 216 |
|  |  |  |  |  |  |  | Uplink |  | High | 29349.96 | 2101665 |  |  | 28576.68 | 2088777 |  | 504 |  |  | 22550 |  | 2101339 |  | 5 |  | 5 | 1 (4) | 1026 |
|  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 120 | 66 |  | Downlink |  | Low | 26799.36 | 2059155 |  |  | 26751.84 | 2058363 |  | 0 | 120 |  | 22402 |  | 2058715 |  | 8 |  | 0 | 1 (4) | 8 |
|  |  |  |  |  |  |  | & |  | Mid | 28124.28 | 2081237 |  |  | 27929.88 | 2077997 |  | 102 |  |  | 22479 |  | 2080891 |  | 7 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  |  |  | Uplink |  | High | 29449.92 | 2103331 |  |  | 28676.64 | 2090443 |  | 504 |  |  | 22555 |  | 2102779 |  | 0 |  | 0 | 0 (0) | 1008 |
| 100+100 | CC1 |  | 100 | 120 | 66 | Downlink |  | Low |  | 26550 |  | 2054999 | 26502.48 |  | 2054207 | 0 |  | 120 |  | 22388 | 2054683 |  | 10 |  | 5 |  | 1 (4) | 18 |
| +100+100 |  |  |  |  |  | & |  | Mid |  | 27849.96 |  | 2076665 | 27655.56 |  | 2073425 | 102 |  |  |  | 22463 | 2076283 |  | 1 |  | 3 |  | 1 (4) | 218 |
|  |  |  |  |  |  | Uplink |  | High |  | 29150.04 |  | 2098333 | 28376.76 |  | 2085445 | 504 |  |  |  | 22538 | 2097883 |  | 3 |  | 0 |  | 1 (4) | 1016 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 |  | 100 | 120 | 66 | Downlink |  | Low |  | 26649.96 |  | 2056665 | 26602.44 |  | 2055873 | 0 |  | 120 |  | 22393 | 2056123 |  | 5 |  | 0 |  | 0 (0) | 0 |
|  |  |  |  |  |  | & |  | Mid |  | 27949.92 |  | 2078331 | 27755.52 |  | 2075091 | 102 |  |  |  | 22469 | 2078011 |  | 8 |  | 5 |  | 1 (4) | 222 |
|  |  |  |  |  |  | Uplink |  | High |  | 29250 |  | 2099999 | 28476.72 |  | 2087111 | 504 |  |  |  | 22544 | 2099611 |  | 10 |  | 2 |  | 1 (4) | 1020 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 |  | 100 | 120 | 66 | Downlink |  | Low |  | 26749.92 |  | 2058331 | 26702.4 |  | 2057539 | 0 |  | 120 |  | 22399 | 2057851 |  | 0 |  | 3 |  | 0 (0) | 6 |
|  |  |  |  |  |  | & |  | Mid |  | 28049.88 |  | 2079997 | 27855.48 |  | 2076757 | 102 |  |  |  | 22474 | 2079451 |  | 3 |  | 0 |  | 0 (0) | 204 |
|  |  |  |  |  |  | Uplink |  | High |  | 29349.96 |  | 2101665 | 28576.68 |  | 2088777 | 504 |  |  |  | 22550 | 2101339 |  | 5 |  | 5 |  | 1 (4) | 1026 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 |  | 100 | 120 | 66 | Downlink |  | Low |  | 26849.88 |  | 2059997 | 26802.36 |  | 2059205 | 0 |  | 120 |  | 22405 | 2059579 |  | 7 |  | 1 |  | 1 (4) | 10 |
|  |  |  |  |  |  | & |  | Mid |  | 28149.84 |  | 2081663 | 27955.44 |  | 2078423 | 102 |  |  |  | 22480 | 2081179 |  | 10 |  | 2 |  | 0 (0) | 208 |
|  |  |  |  |  |  | Uplink |  | High |  | 29449.92 |  | 2103331 | 28676.64 |  | 2090443 | 504 |  |  |  | 22555 | 2102779 |  | 0 |  | 0 |  | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.9	CA_n257J

Table 4.3.1.2.3.1.9-1: Void

Table 4.3.1.2.3.1.9-2: NR Intra-Band contiguous CA configuration CA_n257J, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 8 |  | 11 | 1 (8) | 19 |
| +100+100 |  |  |  |  | & | Mid | 27799.92 | 2075831 | 27678.96 | 2073815 | 102 |  |  | 22460 | 2075419 |  | 8 |  | 3 | 1 (8) | 113 |
| +100 |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28639.68 | 2089827 | 504 |  |  | 22532 | 2096155 |  | 4 |  | 3 | 0 (0) | 507 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 10 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27899.88 | 2077497 | 27778.92 | 2075481 | 102 |  |  | 22466 | 2077147 |  | 10 |  | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28739.64 | 2091493 | 504 |  |  | 22538 | 2097883 |  | 6 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27999.84 | 2079163 | 27878.88 | 2077147 | 102 |  |  | 22472 | 2078875 |  | 0 |  | 14 | 1 (8) | 124 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28839.6 | 2093159 | 504 |  |  | 22544 | 2099611 |  | 8 |  | 5 | 1 (8) | 517 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 2 |  | 3 | 1 (8) | 11 |
|  |  |  |  |  | & | Mid | 28099.8 | 2080829 | 27978.84 | 2078813 | 102 |  |  | 22477 | 2080315 |  | 2 |  | 3 | 0 (0) | 105 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28939.56 | 2094825 | 504 |  |  | 22550 | 2101339 |  | 10 |  | 10 | 1 (8) | 522 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 60 | 132 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 4 |  | 8 | 1 (8) | 16 |
|  |  |  |  |  | & | Mid | 28199.76 | 2082495 | 28078.8 | 2080479 | 102 |  |  | 22483 | 2082043 |  | 4 |  | 0 | 1 (8) | 110 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 504 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.9-3: Void

Table 4.3.1.2.3.1.9-4: NR Intra-Band contiguous CA configuration CA_n257J, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 10 |  | 5 | 1 (4) | 18 |
| +100+100 |  |  |  |  | & | Mid | 27800.04 | 2075833 | 27605.64 | 2072593 | 102 |  |  | 22460 | 2075419 |  | 9 |  | 1 | 1 (4) | 214 |
| +100 |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28276.8 | 2083779 | 504 |  |  | 22532 | 2096155 |  | 8 |  | 1 | 0 (0) | 1010 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 5 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27900 | 2077499 | 27705.6 | 2074259 | 102 |  |  | 22466 | 2077147 |  | 4 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28376.76 | 2085445 | 504 |  |  | 22538 | 2097883 |  | 3 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 3 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27999.96 | 2079165 | 27805.56 | 2075925 | 102 |  |  | 22472 | 2078875 |  | 11 |  | 6 | 1 (4) | 224 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28476.72 | 2087111 | 504 |  |  | 22544 | 2099611 |  | 10 |  | 2 | 1 (4) | 1020 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 7 |  | 1 | 1 (4) | 10 |
|  |  |  |  |  | & | Mid | 28099.92 | 2080831 | 27905.52 | 2077591 | 102 |  |  | 22477 | 2080315 |  | 6 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28576.68 | 2088777 | 504 |  |  | 22550 | 2101339 |  | 5 |  | 5 | 1 (4) | 1026 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 120 | 66 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 2 |  | 4 | 1 (4) | 16 |
|  |  |  |  |  | & | Mid | 28199.88 | 2082497 | 28005.48 | 2079257 | 102 |  |  | 22483 | 2082043 |  | 1 |  | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 28676.64 | 2090443 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.10	CA_n257K

Table 4.3.1.2.3.1.10-1: Void

Table 4.3.1.2.3.1.10-2: NR Intra-Band contiguous CA configuration CA_n257K, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 8 |  | 11 | 1 (8) | 19 |
| +100+100 |  |  |  |  | & | Mid | 27750 | 2074999 | 27629.04 | 2072983 | 102 |  |  | 22457 | 2074555 |  | 0 |  | 1 | 1 (8) | 111 |
| +100+100 |  |  |  |  | Uplink | High | 28950.12 | 2095001 | 28539.72 | 2088161 | 504 |  |  | 22527 | 2094715 |  | 2 |  | 14 | 1 (8) | 526 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 10 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27849.96 | 2076665 | 27729 | 2074649 | 102 |  |  | 22463 | 2076283 |  | 2 |  | 6 | 1 (8) | 116 |
|  |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28639.68 | 2089827 | 504 |  |  | 22532 | 2096155 |  | 4 |  | 3 | 0 (0) | 507 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27949.92 | 2078331 | 27828.96 | 2076315 | 102 |  |  | 22469 | 2078011 |  | 4 |  | 11 | 1 (8) | 121 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28739.64 | 2091493 | 504 |  |  | 22538 | 2097883 |  | 6 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 2 |  | 3 | 1 (8) | 11 |
|  |  |  |  |  | & | Mid | 28049.88 | 2079997 | 27928.92 | 2077981 | 102 |  |  | 22474 | 2079451 |  | 6 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28839.6 | 2093159 | 504 |  |  | 22544 | 2099611 |  | 8 |  | 5 | 1 (8) | 517 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 60 | 132 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 4 |  | 8 | 1 (8) | 16 |
|  |  |  |  |  | & | Mid | 28149.84 | 2081663 | 28028.88 | 2079647 | 102 |  |  | 22480 | 2081179 |  | 8 |  | 5 | 0 (0) | 107 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28939.56 | 2094825 | 504 |  |  | 22550 | 2101339 |  | 10 |  | 10 | 1 (8) | 522 |
|  | Channel spacing CC5-CC6=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC6 | 100 | 60 | 132 | Downlink | Low | 27049.8 | 2063329 | 27002.28 | 2062537 | 0 | 120 |  | 22417 | 2063035 |  | 6 |  | 13 | 1 (8) | 21 |
|  |  |  |  |  | & | Mid | 28249.8 | 2083329 | 28128.84 | 2081313 | 102 |  |  | 22486 | 2082907 |  | 10 |  | 2 | 1 (8) | 112 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 504 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.10-3: Void

Table 4.3.1.2.3.1.10-4: NR Intra-Band contiguous CA configuration CA_n257K, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 10 |  | 5 | 1 (4) | 18 |
| +100+100 |  |  |  |  | & | Mid | 27750 | 2074999 | 27555.6 | 2071759 | 102 |  |  | 22457 | 2074555 |  | 6 |  | 0 | 1 (4) | 212 |
| +100+100 |  |  |  |  | Uplink | High | 28950.12 | 2095001 | 28176.84 | 2082113 | 504 |  |  | 22527 | 2094715 |  | 1 |  | 7 | 1 (4) | 1030 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 5 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27849.96 | 2076665 | 27655.56 | 2073425 | 102 |  |  | 22463 | 2076283 |  | 1 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28276.8 | 2083779 | 504 |  |  | 22532 | 2096155 |  | 8 |  | 1 | 0 (0) | 1010 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 3 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27949.92 | 2078331 | 27755.52 | 2075091 | 102 |  |  | 22469 | 2078011 |  | 8 |  | 5 | 1 (4) | 222 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28376.76 | 2085445 | 504 |  |  | 22538 | 2097883 |  | 3 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 7 |  | 1 | 1 (4) | 10 |
|  |  |  |  |  | & | Mid | 28049.88 | 2079997 | 27855.48 | 2076757 | 102 |  |  | 22474 | 2079451 |  | 3 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28476.72 | 2087111 | 504 |  |  | 22544 | 2099611 |  | 10 |  | 2 | 1 (4) | 1020 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 120 | 66 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 2 |  | 4 | 1 (4) | 16 |
|  |  |  |  |  | & | Mid | 28149.84 | 2081663 | 27955.44 | 2078423 | 102 |  |  | 22480 | 2081179 |  | 10 |  | 2 | 0 (0) | 208 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28576.68 | 2088777 | 504 |  |  | 22550 | 2101339 |  | 5 |  | 5 | 1 (4) | 1026 |
|  | Channel spacing CC5-CC6=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC6 | 100 | 120 | 66 | Downlink | Low | 27049.8 | 2063329 | 27002.28 | 2062537 | 0 | 120 |  | 22417 | 2063035 |  | 9 |  | 6 | 1 (4) | 20 |
|  |  |  |  |  | & | Mid | 28249.8 | 2083329 | 28055.4 | 2080089 | 102 |  |  | 22486 | 2082907 |  | 5 |  | 1 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 28676.64 | 2090443 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.11	CA_n257L

Table 4.3.1.2.3.1.11-1: Void

Table 4.3.1.2.3.1.11-2: NR Intra-Band contiguous CA configuration CA_n257L, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 8 |  | 11 | 1 (8) | 19 |
| +100+100 |  |  |  |  | & | Mid | 27699.96 | 2074165 | 27579 | 2072149 | 102 |  |  | 22454 | 2073691 |  | 6 |  | 6 | 0 (0) | 108 |
| +100+100 |  |  |  |  | Uplink | High | 28850.16 | 2093335 | 28439.76 | 2086495 | 504 |  |  | 22521 | 2092987 |  | 0 |  | 9 | 1 (8) | 521 |
| +100 | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 10 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27799.92 | 2075831 | 27678.96 | 2073815 | 102 |  |  | 22460 | 2075419 |  | 8 |  | 3 | 1 (8) | 113 |
|  |  |  |  |  | Uplink | High | 28950.12 | 2095001 | 28539.72 | 2088161 | 504 |  |  | 22527 | 2094715 |  | 2 |  | 14 | 1 (8) | 526 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27899.88 | 2077497 | 27778.92 | 2075481 | 102 |  |  | 22466 | 2077147 |  | 10 |  | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28639.68 | 2089827 | 504 |  |  | 22532 | 2096155 |  | 4 |  | 3 | 0 (0) | 507 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 2 |  | 3 | 1 (8) | 11 |
|  |  |  |  |  | & | Mid | 27999.84 | 2079163 | 27878.88 | 2077147 | 102 |  |  | 22472 | 2078875 |  | 0 |  | 14 | 1 (8) | 124 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28739.64 | 2091493 | 504 |  |  | 22538 | 2097883 |  | 6 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 60 | 132 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 4 |  | 8 | 1 (8) | 16 |
|  |  |  |  |  | & | Mid | 28099.8 | 2080829 | 27978.84 | 2078813 | 102 |  |  | 22477 | 2080315 |  | 2 |  | 3 | 0 (0) | 105 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28839.6 | 2093159 | 504 |  |  | 22544 | 2099611 |  | 8 |  | 5 | 1 (8) | 517 |
|  | Channel spacing CC5-CC6=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC6 | 100 | 60 | 132 | Downlink | Low | 27049.8 | 2063329 | 27002.28 | 2062537 | 0 | 120 |  | 22417 | 2063035 |  | 6 |  | 13 | 1 (8) | 21 |
|  |  |  |  |  | & | Mid | 28199.76 | 2082495 | 28078.8 | 2080479 | 102 |  |  | 22483 | 2082043 |  | 4 |  | 0 | 1 (8) | 110 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28939.56 | 2094825 | 504 |  |  | 22550 | 2101339 |  | 10 |  | 10 | 1 (8) | 522 |
|  | Channel spacing CC6-CC7=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC7 | 100 | 60 | 132 | Downlink | Low | 27149.76 | 2064995 | 27102.24 | 2064203 | 0 | 120 |  | 22422 | 2064475 |  | 8 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 28299.72 | 2084161 | 28178.76 | 2082145 | 102 |  |  | 22489 | 2083771 |  | 6 |  | 5 | 1 (8) | 115 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 504 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.11-3: Void

Table 4.3.1.2.3.1.11-4: NR Intra-Band contiguous CA configuration CA_n257L, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 10 |  | 5 | 1 (4) | 18 |
| +100+100 |  |  |  |  | & | Mid | 27699.96 | 2074165 | 27505.56 | 2070925 | 102 |  |  | 22454 | 2073691 |  | 3 |  | 3 | 0 (0) | 210 |
| +100+100 |  |  |  |  | Uplink | High | 28850.16 | 2093335 | 28076.88 | 2080447 | 504 |  |  | 22521 | 2092987 |  | 6 |  | 4 | 1 (4) | 1024 |
| +100 | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 5 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27799.92 | 2075831 | 27605.52 | 2072591 | 102 |  |  | 22460 | 2075419 |  | 10 |  | 1 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 28950.12 | 2095001 | 28176.84 | 2082113 | 504 |  |  | 22527 | 2094715 |  | 1 |  | 7 | 1 (4) | 1030 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 3 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27899.88 | 2077497 | 27705.48 | 2074257 | 102 |  |  | 22466 | 2077147 |  | 5 |  | 4 | 1 (4) | 220 |
|  |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28276.8 | 2083779 | 504 |  |  | 22532 | 2096155 |  | 8 |  | 1 | 0 (0) | 1010 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 7 |  | 1 | 1 (4) | 10 |
|  |  |  |  |  | & | Mid | 27999.84 | 2079163 | 27805.44 | 2075923 | 102 |  |  | 22472 | 2078875 |  | 0 |  | 7 | 1 (4) | 226 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28376.76 | 2085445 | 504 |  |  | 22538 | 2097883 |  | 3 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 120 | 66 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 2 |  | 4 | 1 (4) | 16 |
|  |  |  |  |  | & | Mid | 28099.8 | 2080829 | 27905.4 | 2077589 | 102 |  |  | 22477 | 2080315 |  | 7 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28476.72 | 2087111 | 504 |  |  | 22544 | 2099611 |  | 10 |  | 2 | 1 (4) | 1020 |
|  | Channel spacing CC5-CC6=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC6 | 100 | 120 | 66 | Downlink | Low | 27049.8 | 2063329 | 27002.28 | 2062537 | 0 | 120 |  | 22417 | 2063035 |  | 9 |  | 6 | 1 (4) | 20 |
|  |  |  |  |  | & | Mid | 28199.76 | 2082495 | 28005.36 | 2079255 | 102 |  |  | 22483 | 2082043 |  | 2 |  | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28576.68 | 2088777 | 504 |  |  | 22550 | 2101339 |  | 5 |  | 5 | 1 (4) | 1026 |
|  | Channel spacing CC6-CC7=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC7 | 100 | 120 | 66 | Downlink | Low | 27149.76 | 2064995 | 27102.24 | 2064203 | 0 | 120 |  | 22422 | 2064475 |  | 4 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 28299.72 | 2084161 | 28105.32 | 2080921 | 102 |  |  | 22489 | 2083771 |  | 9 |  | 2 | 1 (4) | 216 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 28676.64 | 2090443 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.12	CA_n257M

Table 4.3.1.2.3.1.12-1: Void

Table 4.3.1.2.3.1.12-2: NR Intra-Band contiguous CA configuration CA_n257M, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 60 | 132 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 8 |  | 11 | 1 (8) | 19 |
| +100+100 |  |  |  |  | & | Mid | 27649.92 | 2073331 | 27528.96 | 2071315 | 102 |  |  | 22451 | 2072827 |  | 0 |  | 4 | 0 (0) | 106 |
| +100+100 |  |  |  |  | Uplink | High | 28750.2 | 2091669 | 28339.8 | 2084829 | 504 |  |  | 22515 | 2091259 |  | 10 |  | 3 | 1 (8) | 515 |
| +100+100 | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 60 | 132 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 10 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27749.88 | 2074997 | 27628.92 | 2072981 | 102 |  |  | 22457 | 2074555 |  | 2 |  | 1 | 1 (8) | 111 |
|  |  |  |  |  | Uplink | High | 28850.16 | 2093335 | 28439.76 | 2086495 | 504 |  |  | 22521 | 2092987 |  | 0 |  | 9 | 1 (8) | 521 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 60 | 132 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 27849.84 | 2076663 | 27728.88 | 2074647 | 102 |  |  | 22463 | 2076283 |  | 4 |  | 6 | 1 (8) | 116 |
|  |  |  |  |  | Uplink | High | 28950.12 | 2095001 | 28539.72 | 2088161 | 504 |  |  | 22527 | 2094715 |  | 2 |  | 14 | 1 (8) | 526 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 60 | 132 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 2 |  | 3 | 1 (8) | 11 |
|  |  |  |  |  | & | Mid | 27949.8 | 2078329 | 27828.84 | 2076313 | 102 |  |  | 22469 | 2078011 |  | 6 |  | 11 | 1 (8) | 121 |
|  |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28639.68 | 2089827 | 504 |  |  | 22532 | 2096155 |  | 4 |  | 3 | 0 (0) | 507 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 60 | 132 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 4 |  | 8 | 1 (8) | 16 |
|  |  |  |  |  | & | Mid | 28049.76 | 2079995 | 27928.8 | 2077979 | 102 |  |  | 22474 | 2079451 |  | 8 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28739.64 | 2091493 | 504 |  |  | 22538 | 2097883 |  | 6 |  | 0 | 1 (8) | 512 |
|  | Channel spacing CC5-CC6=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC6 | 100 | 60 | 132 | Downlink | Low | 27049.8 | 2063329 | 27002.28 | 2062537 | 0 | 120 |  | 22417 | 2063035 |  | 6 |  | 13 | 1 (8) | 21 |
|  |  |  |  |  | & | Mid | 28149.72 | 2081661 | 28028.76 | 2079645 | 102 |  |  | 22480 | 2081179 |  | 10 |  | 5 | 0 (0) | 107 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28839.6 | 2093159 | 504 |  |  | 22544 | 2099611 |  | 8 |  | 5 | 1 (8) | 517 |
|  | Channel spacing CC6-CC7=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC7 | 100 | 60 | 132 | Downlink | Low | 27149.76 | 2064995 | 27102.24 | 2064203 | 0 | 120 |  | 22422 | 2064475 |  | 8 |  | 2 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 28249.68 | 2083327 | 28128.72 | 2081311 | 102 |  |  | 22486 | 2082907 |  | 0 |  | 3 | 1 (8) | 113 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28939.56 | 2094825 | 504 |  |  | 22550 | 2101339 |  | 10 |  | 10 | 1 (8) | 522 |
|  | Channel spacing CC7-CC8=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC8 | 100 | 60 | 132 | Downlink | Low | 27249.72 | 2066661 | 27202.2 | 2065869 | 0 | 120 |  | 22428 | 2066203 |  | 10 |  | 7 | 0 (0) | 7 |
|  |  |  |  |  | & | Mid | 28349.64 | 2084993 | 28228.68 | 2082977 | 102 |  |  | 22492 | 2084635 |  | 2 |  | 8 | 1 (8) | 118 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 504 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.3.1.12-3: Void

Table 4.3.1.2.3.1.12-4: NR Intra-Band contiguous CA configuration CA_n257M, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | CORESET#1 Offset from [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100 | CC1 | 100 | 120 | 66 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 10 |  | 6 | 1 (4) | 18 |
| +100+100 |  |  |  |  | & | Mid | 27999.6 | 2079159 | 27805.2 | 2075919 | 102 |  |  | 22472 | 2078875 |  | 2 |  | 12 | 1 (4) | 226 |
| +100+100 |  |  |  |  | Uplink | High | 28750.2 | 2091669 | 27976.92 | 2078781 | 504 |  |  | 22515 | 2091259 |  | 11 |  | 6 | 1 (4) | 1018 |
| +100+100 | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 120 | 66 | Downlink | Low | 26649.96 | 2056665 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 5 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 28099.56 | 2080825 | 27905.16 | 2077585 | 102 |  |  | 22477 | 2080315 |  | 9 |  | 6 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 28850.16 | 2093335 | 28076.88 | 2080447 | 504 |  |  | 22521 | 2092987 |  | 6 |  | 6 | 1 (4) | 1024 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 120 | 66 | Downlink | Low | 26749.92 | 2058331 | 26702.4 | 2057539 | 0 | 120 |  | 22399 | 2057851 |  | 0 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 28199.52 | 2082491 | 28005.12 | 2079251 | 102 |  |  | 22483 | 2082043 |  | 4 |  | 0 | 1 (4) | 212 |
|  |  |  |  |  | Uplink | High | 28950.12 | 2095001 | 28176.84 | 2082113 | 504 |  |  | 22527 | 2094715 |  | 1 |  | 12 | 1 (4) | 1030 |
|  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC4 | 100 | 120 | 66 | Downlink | Low | 26849.88 | 2059997 | 26802.36 | 2059205 | 0 | 120 |  | 22405 | 2059579 |  | 7 |  | 6 | 1 (4) | 10 |
|  |  |  |  |  | & | Mid | 28299.48 | 2084157 | 28105.08 | 2080917 | 102 |  |  | 22489 | 2083771 |  | 11 |  | 6 | 1 (4) | 216 |
|  |  |  |  |  | Uplink | High | 29050.08 | 2096667 | 28276.8 | 2083779 | 504 |  |  | 22532 | 2096155 |  | 8 |  | 6 | 0 (0) | 1010 |
|  | Channel spacing CC4-CC5=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC5 | 100 | 120 | 66 | Downlink | Low | 26949.84 | 2061663 | 26902.32 | 2060871 | 0 | 120 |  | 22411 | 2061307 |  | 2 |  | 6 | 1 (4) | 16 |
|  |  |  |  |  | & | Mid | 28399.44 | 2085823 | 28205.04 | 2082583 | 102 |  |  | 22495 | 2085499 |  | 6 |  | 6 | 1 (4) | 222 |
|  |  |  |  |  | Uplink | High | 29150.04 | 2098333 | 28376.76 | 2085445 | 504 |  |  | 22538 | 2097883 |  | 3 |  | 0 | 1 (4) | 1016 |
|  | Channel spacing CC5-CC6=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC6 | 100 | 120 | 66 | Downlink | Low | 27049.8 | 2063329 | 27002.28 | 2062537 | 0 | 120 |  | 22417 | 2063035 |  | 9 |  | 6 | 1 (4) | 20 |
|  |  |  |  |  | & | Mid | 28499.4 | 2087489 | 28305 | 2084249 | 102 |  |  | 22500 | 2086939 |  | 1 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 29250 | 2099999 | 28476.72 | 2087111 | 504 |  |  | 22544 | 2099611 |  | 10 |  | 6 | 1 (4) | 1020 |
|  | Channel spacing CC6-CC7=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC7 | 100 | 120 | 66 | Downlink | Low | 27149.76 | 2064995 | 27102.24 | 2064203 | 0 | 120 |  | 22422 | 2064475 |  | 4 |  | 6 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 28599.36 | 2089155 | 28404.96 | 2085915 | 102 |  |  | 22506 | 2088667 |  | 8 |  | 6 | 0 (0) | 208 |
|  |  |  |  |  | Uplink | High | 29349.96 | 2101665 | 28576.68 | 2088777 | 504 |  |  | 22550 | 2101339 |  | 5 |  | 6 | 1 (4) | 1026 |
|  | Channel spacing CC7-CC8=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC8 | 100 | 120 | 66 | Downlink | Low | 27249.72 | 2066661 | 27202.2 | 2065869 | 0 | 120 |  | 22428 | 2066203 |  | 11 |  | 6 | 0 (0) | 6 |
|  |  |  |  |  | & | Mid | 28699.32 | 2090821 | 28504.92 | 2087581 | 102 |  |  | 22512 | 2090395 |  | 3 |  | 6 | 1 (4) | 214 |
|  |  |  |  |  | Uplink | High | 29449.92 | 2103331 | 28676.64 | 2090443 | 504 |  |  | 22555 | 2102779 |  | 0 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the channel bandwidths of the two respective NR component carriers. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |