Title:	Minutes of RAN5 TTCN Sidebar RAN5#93-e (12/11/2021)

Source:	MCC TF160

# Attendees

Olivier Genoud, TF160

Ram Thiruvathirai, R&S

Parikshit Bhise, R&S, TF160

Virginie Bardaux, Anritsu, TF160

Wolfgang Seka, Anritsu, TF160

Shaun Harry, Keysight

Mohit Kanchan, Keysight

Hellen Saunders, Keysight, TF160

Sheila Vazquez Millan, R&S, TF160

Lidia Salmeron, TF160

Mohammed Abdul Rasheed, Motorola Mobility, TF160

Narendra Kalahasti, Anritsu

Zhaobing Yang, HiSilicon, TF160

Eniko Sokondar, MediaTek

Fidel Liberal, UPV/EHU

Ingbert Sigovich, ETSI

# 1. Agenda

The meeting was opened on Friday 12th November 10:00 CET.

# Action 55.1 – Assumption 2.2.1-4 in R5w210310

Meeting agreed to wait for a response.  No need to create an LS at the moment.

# Action 55.6 – Introduction of new PIXIT

R&S requested to discuss this separately.

# Action 55.7 – 5G V2X ASPs

These ASPs will be included in the next delivery, but not yet sure if we will be able to include a test case.

# R5w210310 – MCDATA ASP Type def correction

TF160 has made a small revision to the document presented in the workshop.

# NR MAC TC 7.1.1.9.1 (Keysight)

The prose specifies to transmits a DL PDU at step 1, then to use the same NDI to send the PDU at step 6.  It’s not clear if the TTCN is enforcing to use the same NDI at step 6.

TF160 asked if it was expected that the TTCN should handle this NDI.  We have a field in the ASP to toggle the NDI, but this field is not currently being used in this test case.

There is an RRC reconfiguration between steps 1 and 6, so this will also be sent on the NR PTC if it’s run in NR5GC.

SS vendors and TF160 will investigate further.

# Issue with EPS Fallback test cases (R&S)

EPS Fallback test cases 11.1.2, 11.5, 11.1.6 all test behaviour on a network without the N26 interface.

In 11.1.2 this issue is at step 8 the UE can send either (a) an Attach Request or (b) a TAU Request.

The 3GPP core specs specify that an Attach Req is sent using LTE security, but the TAU Req will use 5G security.

If the UE sends a TAU Req, we send a TAU Reject (as specified in TS 23.502).  The UE will then go into Deregistered state, then perform the attach procedure, but for a fresh registration - so the fallback will fail.

There may be some discrepancy in the core specs.

SS vendors, TF160 and other interested parties will investigate further.

# 2. Closure of the Meeting

The meeting was ended on Friday 20th August 10:40 CET.