3GPP TSG-RAN5 Meeting #94-e	R5-220687

# Electronic Meeting, 21th Feb– 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2236 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default RRC IEs for RRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - CSI-IM resources are configured In RRM test cases which involving CSI reporting. However, the REs occupied by CSI-IM resources are not excluded during TBS determination. So a conformant UE may fail the test due to wrong TBS calculation. 38.214, cl.5.1.4.2 - within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) for aperiodic, semi-persistent and periodic time-domain behaviours (higher layer parameters aperiodic-ZP-CSI-RS-ResourceSetsToAddModList, sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet respectively comprised in PDSCH-Config), with each ZP-CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for PDSCH. The REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList and aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are declared as not available for PDSCH when their triggering and activation are applied, respectively. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration: 38.211, cl 7.3.1.5 - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].. To solve this issue, we suggest to reuse the same way used in Demod test——configure ZP CSI-RS resource which overlaps with CSI IM resource. - NrofPort in CSI-RS RMC CSI-RS.3.1 TDD is changed from 2 to 1 in R4-2006436. In RAN4 #100-e we had submitted several CRs (R4-2115266/R4-2115268) to add reportQuantity = cri-RI-CQI in FR2 SCell activation/PSCell addition test cases, which are also based on the assumption of 1-port CSI-RS (since 1 port CSI-RS can't support PMI calculation). However, In RAN4 #101-e nrofPort in CSI-RS RMC CSI-RS.3.1 TDD was changed to 2 in R4-2117732. From our point of view, we agree that 2-ports CSI-RS is a better choice. The consideration is that csi-ReportWithoutPMI is an optional UE capability according to 38.306. Using 1 port CSI-RS will narrow down the applicability of FR2 RRM TCs involving CSI-reporting. So we suggest change reportQuantity = cri-RI-PMI-CQI for FR2 test cases. |
| Summary of change: |  | - Configuration of ZP CSI-RS resource is added in Table 7.3.1-18 - Change reportQuantity = cri-RI-PMI-CQI in CSI-ReportingConfig for FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	CSI-RS-ResourceMapping for CSI

Table 7.3.1-8A: CSI-RS-ResourceMapping for CSI

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000001 |  |  |
| } |  |  |  |
| nrofPorts | p2 |  |  |
| firstOFDMSymbolInTimeDomain | 5 |  | NOT SCS15 |
|  | 4 |  | SCS15 |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

–	CSI-RS-ResourceMapping for BM

Table 7.3.1-8B: CSI-RS-ResourceMapping for BM (Id)

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

–	CSI-RS-ResourceMapping for ZP-CSI-RS

Table 7.3.1-8B: CSI-RS-ResourceMapping-ZP-CSI-RS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row4 | 100 | k0 = 8 |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 4 |  |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| } |  |  |  |

–	CSI-ResourcePeriodicityAndOffset for TRS

<End of modified section 1>

–	CSI-ReportConfig for CSI

Table 7.3.1-12F: CSI-ReportConfig for CSI

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-CSI |  |  |
|  | CSI-ReportConfigId-CSI with condition SCELL_CSI_ON_SPCELL |  | SCELL_CSI_ON_SPCELL |
| carrier | Not present | indicates the same serving cell as this report configuration |  |
|  | ServCellIndex of the SCell |  | SCELL_CSI_ON_SPCELL |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-CSI |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId-CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig CHOICE { |  |  |  |
| slots5 | 2 |  | SCS15 |
| slots10 | 4 |  | SCS30 |
| slots40 | 4 |  | SCS120 |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { | 1 entry |  |  |
| PUCCH-CSI-Resource [1] SEQUENCE { |  | entry 1 |  |
| uplinkBandwidthPartId | BWP-Id of active UL BWP |  |  |
| pucch-Resource | 8 | The first format 2 PUCCH resource configured in Table 4.6.3-112 is used |  |
|  | 9 | The second format 2 PUCCH resource configured in Table 4.6.3-112 is used | SCELL_CSI_ON_SPCELL |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | null |  |  |
| } |  |  |  |
| codebookConfig | CodebookConfig-CSI |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCELL_CSI_ON_SPCELL | For SCell CSI reporting on NR SpCell |

<Start of modified section 2>

–	PDSCH-Config

Table 7.3.1-18: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1+n1+n2+n3 entries | n1 = 1 if SSB configuration used in test case contains two SSBs in a burst, n1 = 0 otherwise n2 = 1 if TRS is configured in test case, n1 = 0 otherwise n3 = 1 if two resource sets of TRS are configured in test case, n3 = 0 otherwise |  |
| TCI-State[1] | TCI-State(0) | entry 1 QCLed to SSB index #0 |  |
| TCI-State[k, k=2..1+n1] | TCI-State(1) | entry ... QCLed to SSB index #1 | SECOND_SSB |
| TCI-State[k, k=2+n1..1+n1+n2] | TCI-State(2) | entry ... QCLed to TRS resource #4 in the first resource set | TRS |
| TCI-State[k, k=2+n1+n2..1+n1+n2+n3] | TCI-State(3) | entry ... QCLed to TRS resource #4 in the second resource set | SECOND_SET |
| } |  |  |  |
| zp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource { | 1 entry |  |  |
| ZP-CSI-RS-Resource[1] | ZP-CSI-RS-Resource-RRM | entry 1 |  |
| } |  |  |  |
| p-ZP-CSI-RS-ResourceSet CHOICE { |  |  | CSI |
| setup | ZP-CSI-RS-ResourceSet-RRM |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SSB | SSB configuration used in test case contain two SSBs in a burst |
| TRS | One resource set for TRS is configured in test case |
| SECOND_SET | Two resource sets for TRS are configured in test case, only applies to FR2. |
| CSI | CSI-RS for CSI reporting are configured in test case |

<End of modified section 3>

–	CodebookConfig-CSI

Table 7.3.1-32: CodebookConfig-CSI

| Derivation Path: Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| two SEQUENCE { |  |  |  |
| twoTX-CodebookSubsetRestriction | 111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

–	ZP-CSI-RS-Resource-RRM

Table 7.3.1-33: ZP-CSI-RS-Resource-RRM

| Derivation Path: Table 4.6.3-204 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| zp-CSI-RS-ResourceId | ZP-CSI-RS-ResourceId-RRM |  |  |
| resourceMapping | CSI-RS-ResourceMapping-ZP-CSI-RS |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| } |  |  |  |

Table 7.3.1-34: ZP-CSI-RS-ResourceId-RRM

| Derivation Path: Table 4.6.3-35 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-ResourceId | n | n is the first ZP-CSI-RS-ResourceId allocated for ZP CSI-RS resource. Value of n is left to internal implementation |  |

–	ZP-CSI-RS-ResourceSet-RRM

Table 7.3.1-35: ZP-CSI-RS-ResourceSet-RRM

| Derivation Path: Table 4.6.3-205 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| zp-CSI-RS-ResourceSetId | ZP-CSI-RS-ResourceSetId-RRM |  |  |
| zp-CSI-RS-ResourceIdList SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId { | 1 entry |  |  |
| ZP-CSI-RS-ResourceId[1] | ZP-CSI-RS-ResourceId-RRM | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

–	ZP-CSI-RS-ResourceSetId-RRM

Table 7.3.1-36: ZP-CSI-RS-ResourceSetId-RRM

| Derivation Path: Table 4.6.3-206 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-ResourceSetId | n | n is the first ZP-CSI-RS-ResourceSetId allocated for ZP-CSI-RS resource set. Value of n is left to internal implementation |  |