3GPP TSG-RAN5 Meeting #95-e	R5-222130

# Electronic Meeting, 9th - 20th May 2022

# Source:	Rohde & Schwarz

# Title:	Continuing and hopefully finalizing the discussion on PICS for NG.114

# Document for:	Endorsement

## 1.	Introduction

Earlier email discussions, having started in November 2020, and discussion paper R5-220241 had elaborated on the issues involved in supporting GSMA PRD NG.114 in RAN5 conformance testing. While this discussion during RAN5#94-e made some progress, RAN5 could not conclude. This submission intends to conclude the discussion.

## 2.	Discussion

R5-220241 had proposed the following.

- Version NG.114 related PICS

- Use specific feature level PICS instead of generic PRD level PICS

Please check R5-220241 for rationale and the details what A. and B. mean implementation wise.

During discussions at RAN5#94-e, there seemed to be support for A. For B, more discussion seemed needed. Therefore, Rohde & Schwarz was tasked to continue discussion via email and reach consensus on highl level level principles. Such discussion was conducted now and the following summarizes this offline discussion:

- Existing PICS like pc_IMS_Session_Timer which date back to requirements in IR.92/94 and now have corresponding requirements in NG.114 should be used also for IMS5GS tests (the alternative would be to introduce new PICS pc_NG114_v1_0_session_timer and continue to use pc_IMS_Session_Timer solely for the LTE tests. Such alternative was said to be too heavyweight).

- On the question how fine grain PICS should get there seems agreement that this is a case-by-case decision.

Example 1 of a fine granular PICS: NG.114 has a requirement that the UE includes in all INVITE requests an option tag “199”. This option tag indicate UE’s capability to act accordingly in certain forking scenarios. The discussion resulted in the proposal to add a PICS tentatively named pc_NG114_v1_0_Forking_199_OptionTag which would lead to two follow-up actions: first, generic INVITE procedure whould be changed such that all initial INVITE requests would have to carry the tag when pc_NG114_v1_0_Forking_199_OptionTag was set to true. Second, test cases where the UE actually receives a 199 Early Dialog Terminated response would list this capability in their applicability statement. 
Note that we do not have above mentioned PICS yet, but we have more generic PICS pc_NG114_v1_0_forking which currently subsumes all forking behavior described in NG.114 v1.0.

Example 2 of fine granular PICS: TS 34.229-2 already has pc_NG114_v1_0_EVS_B0, pc_NG114_v1_0_EVS_B1, pc_NG114_v1_0_EVS_B2, and pc_NG114_v1_0_EVS_A1 for the four non-default EVS configurations where the default configuration is described by pc_NG114_v1_0_default_config_evs_br and pc_NG114_v1_0_default_config_evs_bw. These PICS will likely be used in corresponding applicability statements (usage in test procedures unlikely).

A general understanding seems to be to keep the number of such fine granular PICS in check in order to not overly grow applicability statements.

- Currently we have the broader PRD-level PICS pc_NG114_v1_0 and pc_NG114_v2_0. With the current test coverage in TS 34.229-1 and the other PICS we currently have they seem to be useless. This blends with the observation that NG.114 is so broad in applicability that it seems unlikely that UEs ever will implement all of it. If we would simply state that a UE putting a PICS like pc_NG114_v1_0 to true would to be conformant to NG.114 v1.0, we would always face discussions if that means really all of NG114 or only a subset of NG.114 of interest of RAN5 – and that subset being a moving target. Such everlasting discussion need to be avoided by making it clear what parts of NG114 a UE has to support for our test cases and what not.

## 3.	Proposals

Above proposals A and B from R5-220241 are refined as follows:

- Version NG.114 related PICS (with details of such versioning explained in R5-220241). 
Note: this is already implemented to a wide degree.

- Void the generic PRD level PICS pc_NG114_v1_0 and pc_NG114_v2_0

- Add more feature specific PICS for NG114 only as needed

- Investigate existing feature specific PICS for NG114 if they are really needed (candidates are pc_NG114_v1_0_initial_SDP_offer and similar PICS)

- Implement these proposals, as far as possible, in a revision of R5-222075