3GPP TSG-RAN5 Meeting #95-e	R5-223139

# Electronic Meeting, 9th May – 20th May. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1742 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.2.3 A-MPR PC2 NS_04 test requirements for band n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 6.2.3 A-MPR Power Class 2 test requirements (NS_04) for band n41 (Step 3) is incorrect and need to be modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified Table 6.2.3.5-2 in Test requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some test points cannot be correctly executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 26 | 3.5 | 3.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 9 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 10 | 26 | 0.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 28+TT | 12.5-TT |
| 11 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 12 | 26 | 0.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 13 | 26 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 28+TT | 12.0-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 16 | 26 | 1 | 6 | 0 | 17 | 5.0 | 3 | 28+TT | 12.0-TT |
| 17 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 18 | 26 | 1 | 0 | 0 | 22 | 2.0 | 3 | 28+TT | 20.0-TT |
| 19 | 26 | 2 | 6 | 0 | 17 | 5.0 | 3 | 28+TT | 12.0-TT |
| 20 | 26 | 3.5 | 5 | 0 | 18 | 4.0 | 3 | 28+TT | 14.0-TT |
| 21 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 22 | 26 | 2 | 6 | 0 | 17 | 5.0 | 3 | 28+TT | 12.0-TT |
| 23 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 24 | 26 | 2 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 19.0-TT |
| 25 | 26 | 3.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 26 | 26 | 3.5 | 5 | 0 | 18 | 4.0 | 3 | 28+TT | 14.0-TT |
| 27 | 26 | 2.5 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 19.0-TT |
| 28 | 26 | 2.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 29 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 30 | 26 | 2.5 | 0 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 31 | 26 | 4.5 | 8 | 0 | 15 | 5.0 | 3 | 28+TT | 10.0-TT |
| 32 | 26 | 4.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 33 | 26 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 34 | 26 | 4.5 | 8 | 0 | 15 | 5.0 | 3 | 28+TT | 10.0-TT |
| 35 | 26 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 36 | 26 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 37 | 26 | 3.5 | 7.5 | 0 | 15.5 | 5.0 | 3 | 28+TT | 10.5-TT |
| 38 | 26 | 3.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 39 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 40 | 26 | 3 | 7.5 | 0 | 15.5 | 5.0 | 3 | 28+TT | 10.5-TT |
| 41 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 42 | 26 | 3 | 0 | 0 | 20 | 2.5 | 3 | 28+TT | 17.5-TT |
| 43 | 26 | 3.5 | 7.5 | 0 | 15.5 | 5.0 | 3 | 28+TT | 10.5-TT |
| 44 | 26 | 3.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 45 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 46 | 26 | 3 | 7.5 | 0 | 15.5 | 5.0 | 3 | 28+TT | 10.5-TT |
| 47 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 48 | 26 | 3 | 0 | 0 | 20 | 2.5 | 3 | 28+TT | 17.5-TT |
| 49 | 26 | 3.5 | 7.5 | 0 | 15.5 | 5.0 | 3 | 28+TT | 10.5-TT |
| 50 | 26 | 3.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 51 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 52 | 26 | 3.5 | 7.5 | 0 | 15.5 | 5.0 | 3 | 28+TT | 10.5-TT |
| 53 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 54 | 26 | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16.0-TT |
| 55 | 26 | 6.5 | 10 | 0 | 13 | 5.0 | 3 | 28+TT | 8.0-TT |
| 56 | 26 | 6.5 | 7.5 | 0 | 15.5 | 5.0 | 3 | 28+TT | 10.5-TT |
| 57 | 26 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 58 | 26 | 6.5 | 10 | 0 | 13 | 5.0 | 3 | 28+TT | 8.0-TT |
| 59 | 26 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 60 | 26 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>