3GPP TSG-RAN WG5 Meeting # 95-e	R5-223237

# Electronic meeting, 9 – 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0346 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to A.4.3.9 for Additional capabilities for UE declared capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The available band information for UE declared multi-band peak EIRP relaxation factors for FR2 power class 3 in Table A.4.3.9-2 is incorrect and need to update for supported FR2 band set “n260, n261”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial corrections to the available bands in Table A.4.3.9-2 for supported FR2 band set “n260, n261”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE declared multi-band peak EIRP relaxation factors for supported FR2 bands set of “n260, n261” will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |

Table A.4.3.9-2: UE declared multi-band peak EIRP relaxation factors for FR2 power class 3

| Item | Supported FR2 bands set | Ref. | Release | peak EIRP relaxation factor per band, MBp (dB) (Note 1) |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A | N/A | 1.3 |  |
| 2 | n257, n260 |  |  |  | N/A |  | N/A | 1.0 |  |
| 3 | n258, n260 |  |  | N/A |  |  | N/A | 1.0 |  |
| 4 | n258, n261 |  |  | N/A |  | N/A |  | 1.0 |  |
| 5 | n260, n261 |  |  | N/A | N/A |  |  | 0.0 | No relaxation factor allowed |
| 6 | n257, n258, n260 |  |  |  |  |  | N/A | 1.7 |  |
| 7 | n257, n258, n261 |  |  |  |  | N/A |  | 1.7 |  |
| 8 | n257, n260, n261 |  |  |  | N/A |  |  | 0.5 |  |
| 9 | n258, n260, n261 |  |  | N/A |  |  |  | 1.5 |  |
| 10 | n257, n258, n260, n261 |  |  |  |  |  |  | 1.7 |  |
| 11 | n257, n261 |  |  |  | N/A | N/A |  | 0.0 | No relaxation factor allowed |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3. Note 2: Max allowed sum of MBp over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-3: UE declared multi-band peak EIRP Spherical coverage relaxation factors for FR2 power class 3

| Item | Supported FR2 bands set | Ref. | Release | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A | N/A | 1.25 |  |
| 2 | n257, n260 |  |  |  | N/A |  | N/A | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 3 | n258, n260 |  |  | N/A |  |  | N/A | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 4 | n258, n261 |  |  | N/A |  | N/A |  | 1.25 |  |
| 5 | n260, n261 |  |  | N/A | N/A |  |  | 0.75 | No relaxation allowed for n260 |
| 6 | n257, n258, n260 |  |  |  |  |  | N/A | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 7 | n257, n258, n261 |  |  |  |  | N/A |  | 1.75 |  |
| 8 | n257, n260, n261 |  |  |  | N/A |  |  | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 9 | n258, n260, n261 |  |  | N/A |  |  |  | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 10 | n257, n258, n260, n261 |  |  |  |  |  |  | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 11 | n257, n261 |  |  |  | N/A | N/A |  | 0.0 | No relaxation factor allowed |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3 Note 2: Max allowed sum of MBs over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 | Rel-15 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 | Rel-15 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 7 | FDD Band n7 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |
| 28 | FDD Band n28 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is targeted for FWA form factor |
| ... |  |  |  |  |
| 30 | FDD Band n30 | 38.101-1, 7.3.2 | Rel-16 |  |
| ... |  |  |  |  |
| 66 | FDD Band n66 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 70 | FDD Band n70 | 38.101-1, 7.3.2 | Rel-15 |  |
| 71 | FDD Band n71 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is targeted for FWA form factor |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/2. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |

Table A.4.3.9-4b: TDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 34 | TDD Band n34 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |
| 38 | TDD Band n38 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 39 | TDD Band n39 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |
| 40 | TDD Band n40 | 38.101-1, 7.3.2 | Rel-15 |  |
| 41 | TDD Band n41 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |
| 48 | TDD Band n48 | 38.101-1, 7.3.2 | Rel-16 | NOTE 2 |
| … |  |  |  |  |
| 77 | TDD Band n77 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 78 | TDD Band n78 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 79 | TDD Band n79 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/3. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |

Table A.4.3.9-4c: 2 Rx antenna ports Capabilities

| Item | Band | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 |  |
| 4 | FDD Band n5 | 38.101-1, 7.3.2 |  |
| 5 | FDD Band n7 | 38.101-1, 7.3.2 | NOTE 2 |
| 6 | FDD Band n8 | 38.101-1, 7.3.2 |  |
| 7 | FDD Band n12 | 38.101-1, 7.3.2 |  |
| 7b | FDD Band n14 | 38.101-1, 7.3.2 |  |
| 8 | FDD Band n20 | 38.101-1, 7.3.2 |  |
| 8d | FDD Band n24 | 38.101-1, 7.3.2 |  |
| 9 | FDD Band n25 | 38.101-1, 7.3.2 |  |
| 9a | FDD Band n26 | 38.101-1, 7.3.2 |  |
| 10 | FDD Band n28 | 38.101-1, 7.3.2 |  |
| 10a | FDD Band n29 | 38.101-1, 7.3A.2.4 |  |
| 10b | FDD Band n30 | 38.101-1, 7.3.2 |  |
| 11 | TDD Band n34 | 38.101-1, 7.3.2 |  |
| 12 | TDD Band n38 | 38.101-1, 7.3.2 | NOTE 2 |
| 13 | TDD Band n39 | 38.101-1, 7.3.2 |  |
| 14 | TDD Band n40 | 38.101-1, 7.3.2 |  |
| 15 | TDD Band n41 | 38.101-1, 7.3.2 | NOTE 2 |
| 16 | TDD Band n48 | 38.101-1, 7.3.2 |  |
| 17 | TDD Band n50 | 38.101-1, 7.3.2 |  |
| 18 | TDD Band n51 | 38.101-1, 7.3.2 |  |
| 18a | Reserved |  |  |
| 18b | TDD Band n53 | 38.101-1, 7.3.2 |  |
| 19 | FDD Band n65 | 38.101-1, 7.3.2 |  |
| 20 | FDD Band n66 | 38.101-1, 7.3.2 |  |
| 21 | FDD Band n70 | 38.101-1, 7.3.2 |  |
| 22 | FDD Band n71 | 38.101-1, 7.3.2 |  |
| 23 | FDD Band n74 | 38.101-1, 7.3.2 |  |
| 24 | TDD Band n77 | 38.101-1, 7.3.2 | NOTE 2 |
| 25 | TDD Band n78 | 38.101-1, 7.3.2 | NOTE 2 |
| 26 | TDD Band n79 | 38.101-1, 7.3.2 | NOTE 2 |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/1. NOTE 2: Support of 2 Rx for this band is allowed only for vehicular UEs i.e. if support has been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |

Table A.4.3.9-4d: Enhanced transient capabilities

| Item | Band | Ref | Release | enhanced transient capability per band |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2us | 4us | 7us |  |
| 1 | FDD Band n1 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 2 | FDD Band n2 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 3 | FDD Band n3 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 4 | FDD Band n5 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 5 | FDD Band n7 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 6 | FDD Band n8 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 7 | FDD Band n12 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 8 | FDD Band n14 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 9 | FDD Band n20 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 10 | FDD Band n24 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 11 | FDD Band n25 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 12 | FDD Band n26 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 13 | FDD Band n28 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 14 | FDD Band n30 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 15 | TDD Band n34 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 16 | TDD Band n38 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 17 | TDD Band n39 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 18 | TDD Band n40 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 19 | TDD Band n41 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 20 | TDD Band n46 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 21 | TDD Band n48 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 22 | TDD Band n50 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 23 | TDD Band n51 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 24 | TDD Band n53 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 25 | TDD Band n65 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 26 | TDD Band n66 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 27 | FDD Band n70 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 28 | FDD Band n71 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 29 | FDD Band n74 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 30 | TDD Band n77 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 31 | TDD Band n78 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 32 | TDD Band n79 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 33 | TDD Band n96 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported with enhanced transient capability of 2us, 4us or 7us if UE has indicated support of the capability defined in Table A.4.3.2-1/79. NOTE 2: Indicate transient capability for each band by ticking the cell corresponding to the smallest enhanced transient capability that the UE supports for that band. |  |  |  |  |  |  |  |

Table A.4.3.9-5: Beam Peak Search Vendor Declarations with respect to test frequency range for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1. |
| 3 | n260 | n260 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 5 | n261 | n261 single CC beam peak is leveraged from n257 single CC mid channel to n261 low, mid and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 2 |
| NOTE 1: The beam peak searches shall be performed for every test frequency range by default unless the device manufacturer explicitly declares that the beam peak at the mid test frequency range is applicable for the remaining (low, high) test frequency ranges. NOTE 2: Beam peak search results can be re-used from bands that completely contain the target bands if explicitly declared by the manufacturer. |  |  |  |  |  |

Table A.4.3.9-6: Beam Peak Search Vendor Declarations with respect to test frequency range for different CA BW classes

| Item | Bands | NR CA bandwidth class | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258, n260, n261 | A, B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | The beam peak is leveraged from a reference (frequency band, CBW) or (frequency band combination, CA BW class) to a group of other intra-band contiguous combinations and CA BW classes | 38.521-2, K.1.1 & K.1.2 | Rel-15 | A beam peak search shall be performed for every intra-band contiguous combination and CA BW class by default unless the device manufacturer explicitly declares that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes. |

Table A.4.3.9-7: Beam Peak Search Vendor Declarations with respect to modulation for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every modulation by default unless the device manufacturer explicitly declares that the beam peak at the QPSK modulation is applicable for the remaining 16QAM and 64QAM modulations. |  |  |  |  |  |

Table A.4.3.9-8: Beam Peak Search Vendor Declarations with respect to waveform for single CC

| Item | Band | Intent | Reference Waveform | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every waveform by default unless the device manufacturer explicitly declares that the beam peak from one waveform is applicable for the other waveform. |  |  |  |  |  |  |

Table A.4.3.9-9: Reference Point Vendor Declaration for grey-box test approach

| Item | Band | Positioning Reference Point: Offset (x/y/z) from geometric centre of DUT [cm] | Minimum QZ required to contain all active antennas within the quiet zone | Ref. | Release |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 2 | n258 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 3 | n260 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 4 | n261 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| Note: The available QZ sizes are defined in TS 38.508-1, Clause B.2.2.2, i.e., 20cm, 30cm, 40cm, and 55cm |  |  |  |  |  |

Table A.4.3.9-10: Vendor Declarations with respect to PC3 antenna configuration

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| NOTE 1: The fine PC3 measurement grids based on the 8x2 worst case configuration shall be applied by default unless the device manufacturer explicitly declares that all antenna arrays with M x N (M ≥ N) comply with M ≤ 4 and N ≤ 2 for each band. |  |  |  |  |  |

Table A.4.3.9-11: Antenna Aperture Vendor Declaration

| Item | Band | Antenna Aperture Declaration | Ref. | Release |
| --- | --- | --- | --- | --- |
| 1 | n257 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 2 | n258 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 3 | n260 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 4 | n261 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |

==============================================================

<< End of changes >>