3GPP TSG-RAN5 Meeting #95-e	R5-222207r1

# Electronic Meeting, 9th May– 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1624 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Moving additional tolerance in 6.2A.3.1.5 and 6.2D.3.5 to end of the section |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additional tolerance in 6.2A.3.1.5 and 6.2D.3.5 are in the middle of the section which will confuse the spec reader. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional tolerance in 6.2A.3.1.5 and 6.2D.3.5 were moved to the end of the section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec reader will be confused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1.5, 6.2D.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: NR-DC requirements were added combining the corresponding changes in R5-222326 to resolve the overlapping issue. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance (Additional Maximum Power Reduction (A-MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78 and CA_n8-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20+TT |
| 2 | 23 | 6.5 | 6.5 | 7 | 0 | 0 | 0 | 3 | 9.8 | 7 | 25+TT | 2.8+TT |
| 3 | 23 | 6.5 | 0 | 7 | 0 | 0 | 0 | 3 | 20 | 6 | 25+TT | 14+TT |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

6.2A.4	Configured output power for CA

<< Unchanged sections omitted >>

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 2 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 3 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 4 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 6 | 26 | 1.5 | 0 | 0 | 24.5 | 3 | 3 | 28+TT | 21.3-TT |
| 7 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 8 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 9 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 10 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.0-TT |
| 14 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 15 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 16 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 17 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 18 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 19 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 20 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 21 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 22 | 26 | 0 | 7.5 | 0 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 23 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 24 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 2 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 6 | 23 | 1.5 | 0 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
| 7 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 8 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 12 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 13 | 23 | 2.0 | 0 | 0 | 21.0 | 5 | 3 | 25+TT | 16.0-TT |
| 14 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 15 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 21 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 22 | 23 | 0 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 2 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 4 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 5 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 7 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 8 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 9 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 10 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 11 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 12 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-4: UE Power Class 3 test requirements (NS_05) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-5: UE Power Class 3 test requirements (NS_05U) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-6: UE Power Class 3 test requirements (NS_48) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 11 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 16 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 17 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 18 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 21 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 22 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 27 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 29 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 30 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-7: UE Power Class 3 test requirements (NS_49) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 5 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 11 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 16 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 17 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 30 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 31 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 32 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 35 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 36 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 38 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 39 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 40 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 41 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 42 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4, 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7, 8 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4, 5 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7, 8 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 1.5 | 15 | 5 | 2 | 25+TT | 10-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 1.5 | 14 | 5 | 2 | 25+TT | 9-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-12: UE Power Class 3 test requirements (NS_46) for band n7

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 2 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 3 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 4 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 10 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 11 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 12 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-13: UE Power Class 3 test requirements (NS_21) for band n30

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 4,5 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 6,7 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 5 | 23 | 3 | 0 | 0 |  | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9,10 | 10 | 23 | 3 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 11,12 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 14,15 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 16,17 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 18 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19,20 | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-14: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 2 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 3 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 4 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 5 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 6 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 7 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 8 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 10 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 11 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 14 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 17 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 21 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 22 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-15: UE Power Class 3 test requirements (NS_27) for band n48

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 7 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 9 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 12 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 14 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 15 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 17 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 18 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 19 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 21 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 22 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 23 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 25 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 32 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 33 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 34 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 35 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 36 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 37 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 39 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 40 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 41 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 43 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 45 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 49 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 52 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 53 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 57 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 58 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 59 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 60 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 61 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 62 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 63 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 64 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 65 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 66 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 67 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 68 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 70 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 71 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 72 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 73 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 74 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 75 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 76 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 77 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 78 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 79 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 80 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 81 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 82 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 83 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 84 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 85 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 86 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 87 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 88 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 89 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 90 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 91 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 92 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 93 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 94 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 95 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 96 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 97 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 98 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 102 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 103 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 104 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 105 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 106 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 107 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 108 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 109 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 110 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 |  | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

6.2D.4	Configured transmitted power for UL MIMO

<< End of changes >>