Title:	Minutes of RAN5 TTCN Sidebar RAN5#95-e (13/05/2022)

Source:	MCC TF160

# Attendees

Olivier Genoud, TF160

Ram Thiruvathirai, R&S

Virginie Bardaux, Anritsu, TF160

Wolfgang Seka, Anritsu, TF160

Shaun Harry, Keysight

Mohit Kanchan, Keysight

Hellen Saunders, Keysight, TF160

Marija Buis, Adare, TF160

Lidia Salmeron, TF160

Francisca Rodriguez, Adare, TF160

Mohammed Abdul Rasheed, Motorola Mobility, TF160

Narendra Kalahasti, Anritsu

Zhaobing Yang, Hisilicon, TF160

Carlos Arroyo-Narvaez, Adare, TF160

Eniko Sokondar, MediaTek

Fidel Liberal, UPV/EHU

Sheila Vazquez Milan, R&S, TF160

# 1. Agenda

The meeting was opened on Friday 13th May 11:00 CET.

# R5w220102 – NR IIoT Test Model and ASP updates, PDCP CA duplication (Sheila)

There aren’t yet any signalling frequencies available in 38-508-1 and we don’t have details for the inter-band test cases.  These TCs cannot therefore be made verifiable.

R&S asked how the TTCN will inform the SS that the MAC logical channel is linked to the RLC when telling the SS there’s 3 RLC entities.  TF160 replied that the logical channel is defined for SCell 1 or SCell 2, but doesn’t say if it’s SCell or PCell.  The intention is that it appears to be a DRB on a PCell.

The duplication is only for UL.

# R5-222385

This is also required for NR-DC, therefore it’s also included in clause 5.2.2.2.5.

R5-222816

There are submissions from Starpoint and Huawei to remove the PUCCH configuration on the SCell, but does not reduce the number of SCells.

The PUCCH is optional in the SCell for the UE, this prose CR is setting this to omit.  The meeting agreed that this prose CR can be accepted, but there should be at least 1 TC testing PUCCH config.

R&S asked if there a possibility to send the grant for the SCell in the PCell?  TF160 replied that there’s no TC testing this.

R&S asked if it was still necessary to configure the PUCCH in the SS?  TF160 replied that SIB1 still includes the PUCCH, therefore it needs to be included in the cell array.  It would complicate the TTCN initialization for just one TC if we changed this.

# R5w220108 – 5G V2X: Test Model and ASP updates (Virginie)

# R5-222378r2

No comments received

# R5s220680- Correction for 5G NAS Type definitions (Keysight)

TF160 accept the change and will check if any templates are also impacted.

TTCN Delivery, (Olivier)

Due to the decision to make RAN5#95 virtual, our next delivery will be in wk37 on 16th September.

# 2. Closure of the Meeting

The meeting was ended on Friday 13th May 11:30 CET.