3GPP TSG-RAN5 Meeting #96-e	R5-224633

# Online,  , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0559 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core alignment and editorial corrections for FR1 Demodulation CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 5.1.1,5, Table 5.2A-1, and tables in 5.2A.3.1.0 are not aligned with TS 38.101-4. Incorrect TE diagram number for CA test cases. Overlapping descriptions between 5.5A.1.1.3, and 5.5A.1.1.1 and 5.5A.1.1.3.1. Editorial errors exist in chapter 2, 5, and Annex F. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.1.1,5, Table 5.2A-1, and tables in 5.2A.3.1.0 were corrected according to TS 38.101-4. TE diagram number for CA test cases was corrected. Overlapping descriptions in 5.5A.1.1.3 were removed. Editorial errors for reference number to other documents, section numbers, table numbers were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA devices may fail the Test Cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.1.5, 5.2A, 5.5A, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[6]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment"

[7]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone"

[8]	3GPP TS 38.521-2: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone"

[9]	3GPP TS 38.211: "NR; Physical channels and modulation".

[10]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[11]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[12]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[13]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity", Stage 2.

[14]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[15]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[16]	3GPP TS 36.521-1: "E-UTRA; User Equipment (UE) conformance specification; Radio transmission and reception; Part1: conformance testing"

[17]	3GPP TS 36.211: "Physical Channels and Modulation".

[18]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[19]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[20]	3GPP TS 38.331: “Radio Resource Control (RRC) protocol specification”.

[21]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[22]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)"

[23]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services"

[24]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[25]	3GPP TS 38.133: “NR; Requirements for support of radio resource management”.

3	Definition of terms, symbols and abbreviations

<<Unchanged sections skipped>>

5.1.1.5	Applicability of CA requirements

5.1.1.5.1	Definition of CA capability

The definition with respect to CA capabilities is given as in Table 5.1.1.5.1-1.

Table 5.1.1.5.1-1: Definition of CA capability

| CA Capability | CA Capability Description |
| --- | --- |
| CA_C | Intra-band contiguous CA |
| CA_N | Intra-band non-contiguous CA |
| CA_AX | Inter-band CA (X bands) |
| NOTE 1: CA_C corresponds to NR CA configurations and bandwidth combination sets defined in Section 5.5A.1 of TS 38.101-1[2]. CA_N corresponds to NR CA configurations and bandwidth combination sets defined in Section 5.5A.2 of TS 38.101-1[2]. CA_AX corresponds to NR CA configurations and bandwidth combination sets defined in Section 5.5A.3 of TS 38.101-1[2]. |  |

5.1.1.5.2	Applicability and test rules for different CA configurations and bandwidth combination sets

The performance requirement for CA UE demodulation tests in Section 5.2A are defined independent of CA configurations and bandwidth combination sets specified in Section 5.5A of TS 38.101-1[2]. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined in Table 5.1.1.5.2-1 and Table 5.1.1.5.2-2. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 5.1.1.5.2-1: Applicability and test rules for CA UE demodulation tests

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test 1 in Section 5.2A.2.1 and 5.2A.3.1 | CA_C, CA_N, CA_AX | Table 5.1.1.5.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| Test 2 in Section 5.2A.2.1 and 5.2A.3.1 | CA_C, CA_N, CA_AX | Table 5.1.1.5.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| Test 3 in Section 5.2A.2.1 and 5.2A.3.1 | CA_AX | Table 5.1.1.5.2-2 | Largest aggregated CA bandwidth combination | TDD CC if supported, otherwise FDD CC |
| Test 4 in Section 5.2A.2.1 and 5.2A.3.1 (NOTE 2) | CA_AX | Table 5.1.1.5.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| Test 5 in Section 5.2A.2.1 and 5.2A.3.1 (NOTE 3) | CA_AX | Table 5.1.1.5.2-2 | Largest aggregated CA bandwidth combination | 15 kHz CC if supported, otherwise 30 kHz CC |
| NOTE 1: In case CA_AX with different number of X is supported then one or two CA configurations are selected based on procedure from Table 5.1.1.5.2-2. NOTE 2: These scenarios are only tested for UEs which are not verified with Test 3 in Section 5.2A.2.1 and 5.2A.3.1 NOTE 3: These scenarios are only tested for UEs which are not verified with Test 4 in Section 5.2A.2.1 and 5.2A.3.1 |  |  |  |  |

Table 5.1.1.5.2-2: Selection of CA configurations

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C or CA_N | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |
| NOTE 1: For CA_AX capability, if CA configuration from step 2 is CA configuration with the largest number of bands then Step 3 and Step 4 are skipped. Otherwise, the two CA configurations selected from Step 2 and Step 4 are used for testing. NOTE 2: Maximum supported data rate for Step 2 and Step 4 is calculated based clause 4.1.2 of TS 38.306 [14]. NOTE 3: Tested data rate for Step 2 and Step 4 is calculated based on the equation DataRate=10-3j=1JTBSj2μj and FRCs used in the test. |  |  |  |  |

5.1.1.5.3	Applicability rule and antenna connection for CA tests with 4 RX

Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, 2 out of the 4Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied.

Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, all 4Rx should be connected with data source from system simulator. Requirements from Clause 5.2A.3.1 are applied.

For 4Rx capable UEs, the 2Rx supported RF bands and 4Rx supported RF bands are up to UE’s declaration.

5.1.1.6	Applicability and test rules for PDSCH performance requirements with power imbalance for intra-band contiguous CA

For UE passing the FDD and TDD CA power imbalance performance requirements with 2 DL CCs as defined in sections 5.2A.2.2 and 5.2A.3.2, the test coverage can be considered fulfilled with FDD or TDD intra-band contiguous CA with 3 or more DL CCs supported by the UE. During the test, UE is required to test the supported intra-band contiguous CA configurations with 2 DL CCs covering the lowest and highest operating bands.

The channel bandwidth combination for testing is determined by following procedure:

-	First select the bandwidth combinations with the same bandwidth in each carrier.

-	If there is no such bandwidth combination, select the bandwidth combinations with smallest bandwidth difference between the two carriers, and the carrier with smaller bandwidth will be used for test.

-	Among the bandwidth combinations selected, select the CA combination with largest aggregated bandwidth combination.

<<Unchanged sections skipped>>

5.2A	PDSCH demodulation requirements for CA

The parameters specified in Table 5.2-1 for PDSCH single carrier tests are reused for PDSCH CA tests unless otherwise stated.

Table 5.2A-1: Common test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12 TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases with no more chan 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |

Table 5.2A-2: Test parameters for number of HARQ processes

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | 8 | 8 |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | 4 | 4 |
|  | TDD PCell | 8 | 8 |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | 8 | 12 |
|  | 30kHz PCell | 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Table 5.2A-3: Test parameters for K1 values

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11,9} |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {4,3,2,6} | {4,3,2,6,5} |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | {4,3,2,6} | {4,4,3,3,2,2,6,6} |
|  | 30kHz PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | {8,7,6,5,5,4,3,2} | N/A |

5.2A.1	1RX requirements (Void)

5.2A.2	2RX requirements

5.2A.2.1	Requirements for 2RX normal PDSCH

5.2A.2.1.0	Minimum conformance requirements for 2RX normal PDSCH

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.2.1.0-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.2.1.0-1 to Table 5.2A.2.1.0-3, with the parameters in Table 5.2A-1 to Table 5.2A-3 and the downlink physical channel setup according to Annex C.2.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.2.1.0-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.4 |

Table 5.2A.2.1.0-2: Single carrier performance for TDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-2.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 10 | R.PDSCH.1-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 15 | R.PDSCH.1-2.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 20 | R.PDSCH.1-2.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.9 |
| 25 | R.PDSCH.1-2.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 30 | R.PDSCH.1-3.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.9 |
| 40 | R.PDSCH.1-3.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.2 |
| 50 | R.PDSCH.1-3.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 |

Table 5.2A.2.1.0-3: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-13.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 10 | R.PDSCH.2-13.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 15 | R.PDSCH.2-13.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 20 | R.PDSCH.2-13.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.7 |
| 25 | R.PDSCH.2-13.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.7 |
| 30 | R.PDSCH.2-14.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.7 |
| 40 | R.PDSCH.2-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.9 |
| 50 | R.PDSCH.2-14.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.1 |
| 60 | R.PDSCH.2-14.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.0 |
| 80 | R.PDSCH.2-14.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 |
| 90 | R.PDSCH.2-14.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.3 |
| 100 | R.PDSCH.2-15.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.7 |

Table 5.2A.2.1.0-4: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.2.1.0-1 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1.0-3 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1.0-1 and Table 5.2A.2.1.0-3 per CC |
| 4 | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.2.1.0-1 and Table 5.2A.2.1.0-2 per CC |
| 5 | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1.0-2 and Table 5.2A.2.1.0-3 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.5. |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2A.2.1.

5.2A.2.1.1	2Rx Normal PDSCH Demodulation Performance for CA (2DL CA)

5.2A.2.1.1.1	Test Purpose

To verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions for multiple CA configurations and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput per CC.

5.2A.2.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA.

5.2A.2.1.1.3	Test description

5.2A.2.1.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

Channel BW to be tested: largest aggregated bandwidth combination as per Table 5.1.1.5.2-2.

CA capability to be tested: As per table 5.1.1.5.2-1

Table 5.2A.2.1.1.3.1-1: Test point selection table

| Test number | CA duplex mode | Configuration |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.2.1.0-1 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1.0-3 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1.0-1 and Table 5.2A.2.1.0-3 per CC |
| 4 (note 2) | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.2.1.0-1 and Table 5.2A.2.1.0-2 per CC |
| 5 (note 3) | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1.0-2 and Table 5.2A.2.1.0-3 per CC |
| Note 1: For each test point, select any one of the CA configurations which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate based on the equation DataRate=10-3j=1JTBSj2μj Note 2: Test point 4 can be skipped if test point 3 is verified. Note 3: Test point 5 can be skipped if test point 3 or 4 is verified. |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.6 for TE diagram and clause A.3.2.6 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2A-1 to Table 5.2A-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2A.2.1.1.3.3.

5.2A.2.1.1.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2 A.2.1.1.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.2.1.0-1 to 5.2A.2.1.0-4 as appropriate on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2A.2.1.0-1 to 5.2A.2.1.0-4 as appropriate on both PCC and SCC.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.2.1.0-4 as appropriate.

5.2A.2.1.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2A.2.1.1.4	Test Requirement

Tables 5.2A.2.1.1.4-1, 5.2A.2.1.1.4-2 and 5.2A.2.1.1.4-3 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 and A.3.2.2 for each component carrier for throughput test point combination shall meet or exceed the specified value in Table 5.2A.2.1.1.4-1, 5.2A.2.1.1.4-2 and 5.2A.2.1.1.4-3 for the specified SNR including test tolerances for the test points listed in Table 5.2A.2.1.1.3.1-1.

Table 5.2A.2.1.1.4-1: Test requirements for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.4 |

Table 5.2A.2.1.1.4-2: Test requirements for TDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-2.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 10 | R.PDSCH.1-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 15 | R.PDSCH.1-2.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 20 | R.PDSCH.1-2.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.9 |
| 25 | R.PDSCH.1-2.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 30 | R.PDSCH.1-3.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.9 |
| 40 | R.PDSCH.1-3.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.2 |
| 50 | R.PDSCH.1-3.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.5 |

Table 5.2A.2.1.1.4-3: Test requirements for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-13.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 10 | R.PDSCH.2-13.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 15 | R.PDSCH.2-13.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 20 | R.PDSCH.2-13.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.7 |
| 25 | R.PDSCH.2-13.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.7 |
| 30 | R.PDSCH.2-14.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.7 |
| 40 | R.PDSCH.2-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.9 |
| 50 | R.PDSCH.2-14.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.1 |
| 60 | R.PDSCH.2-14.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 80 | R.PDSCH.2-14.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.5 |
| 90 | R.PDSCH.2-14.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.3 |
| 100 | R.PDSCH.2-15.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.7 |

5.2A.2.1.2	2Rx Normal PDSCH Demodulation Performance for CA (3DL CA)

5.2A.2.1.2.1	Test Purpose

Same as 5.2A.2.1.1

5.2A.2.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 3DL CA.

5.2A.2.1.2.3	Test description

5.2A.2.1.2.3.1	Initial conditions

Same as 5.2A.2.1.1.3.1

5.2A.2.1.2.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2 A.2.1.2.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.2.1.0-1 to 5.2A.2.1.0-4 as appropriate on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2A.2.1.0-1 to 5.2A.2.1.0-4 as appropriate on both PCC and SCCs.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.2.1.0-4 as appropriate.

5.2A.2.1.2.3.3	Message contents

Same as 5.2A.2.1.1.3.3.

5.2A.2.1.2.4	Test Requirement

Same as 5.2A.2.1.1.4 evaluated per component carrier.

5.2A.2.1.3	2Rx Normal PDSCH Demodulation Performance for CA (4DL CA)

5.2A.2.1.3.1	Test Purpose

Same as 5.2A.2.1.1

5.2A.2.1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 4DL CA.

5.2A.2.1.3.3	Test description

5.2A.2.1.3.3.1	Initial conditions

Same as 5.2A.2.1.1.3.1

5.2A.2.1.3.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2 A.2.1.3.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.2.1.0-1 to 5.2A.2.1.0-4 as appropriate on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2A.2.1.0-1 to 5.2A.2.1.0-4 as appropriate on both PCC and SCCs.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.2.1.0-4 as appropriate.

5.2A.2.1.3.3.3	Message contents

Same as 5.2A.2.1.1.3.3.

5.2A.2.1.3.4	Test Requirement

Same as 5.2A.2.1.1.4 evaluated per component carrier.

5.2A.2.2	Requirements for 2RX PDSCH carrier aggregation with power imbalance

5.2A.2.2.0	Minimum conformance requirements for 2RX PDSCH CA with power imbalance

The performance requirements are specified in Table 5.2A.2.2.0-3 and Table 5.2A.2.2.0-4, with the addition of test parameters in Table 5.2A.2.2.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2A.2.2.0-1.

Table 5.2A.2.2.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the ability of an intra-band adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only |  |

Table 5.2A.2.2.0-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration |  |  | 1x2 |
| PDSCH configuration | Length (L) |  | FDD: 12TDD: 12 for DL slot, 4 for special slot |
|  | PRB bundling size |  | WB |
| Modulation and code rate |  |  | 64QAM, MCS 26 |
| Number of HARQ Processes |  |  | FDD: 4 TDD: 8 |
| Maximum number of HARQ transmission |  |  | 1 |
| Redundancy version coding sequence |  |  | {0} |
| TDD UL-DL pattern |  |  | 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table A.1.2-2 for FR1.30-1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 |
| Overhead for TBS determination |  |  | 0 |
| SSB transmission |  |  | Slot#0 with periodicity 20ms |
| RB assignment |  |  | Full applicable test bandwidth as defined in Table 5.3.5-1 of TS 38.101-1 [2] |

Table 5.2A.2.2.0-3: Minimum performance for FDD CA with 15 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

Table 5.2A.2.2.0-4: Minimum performance for TDD CA with 30 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2A.2.2.

5.2A.2.2.1	2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA)

5.2A.2.2.1.1	Test Purpose

To verify the ability of an intra-band adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only.

5.2A.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL intra-band contiguous CA.

5.2A.2.2.1.3	Test description

5.2A.2.2.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

Band selection: Supported intra-band contiguous CA configurations covering the lowest and highest operating bands,

Channel BW combination to be tested: Select bandwidth combination with same bandwidth in each carrier. If not supported, select bandwidth combination with smallest bandwidth difference between the two carriers and the carrier with smaller bandwidth will be used for the test.

CA capability to be tested: Either FDD or TDD intra-band contiguous CA

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2A for TE diagram and clause A.3.2.3 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2A-1 to Table 5.2A-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2A.2.2.1.3.3.

5.2A.2.2.1.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2A.2.2.1.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the MCS according to Table 5.2A.2.2.0-2 on PCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix according to Tables 5.2A.2.2.0-2 to 5.2A.2.2.0-4 as appropriate on both PCC and SCC.

6.	Measure the average throughput on PCC for at least 300 frames.. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail based on measured throughput > 85% of the maximum scheduled throughput..

5.2A.2.2.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2A.2.2.1.4	Test Requirement

Table 5.2A.2.2.0-2, Table 5.2A.2.2.1.4-1 and Table 5.2A.2.2.1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the MCS scheduled as specified in Table 5.2A.2.2.0-2 for the throughput test shall meet or exceed the specified value in Table 5.2A.2.2.1.4-1 or Table 5.2A.2.2.1.4-2 as applicable for the specified cell power levels.

Table 5.2A.2.2.1.4-1: Test Requirements for FDD CA with 15 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

Table 5.2A.2.2.1.4-2: Test Requirements for TDD CA with 30 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

5.2A.2.2.2	2Rx PDSCH Demodulation Performance for CA with power imbalance (3DL CA)

FFS

5.2A.2.2.3	2Rx PDSCH Demodulation Performance for CA with power imbalance (4DL CA)

FFS

5.2A.3	4RX requirements

5.2A.3.1	Requirements for 4RX normal PDSCH

5.2A.3.1.0	Minimum conformance requirements for 4RX normal PDSCH

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.3.1.0-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.3.1.0-1 to Table 5.2A.3.1.0-3, with the parameters in Table 5.2A-1 to Table 5.2A-3 and the downlink physical channel setup according to Annex C.2.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.3.1.0-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.9 |

Table 5.2A.3.1.0-2: Single carrier performance for TDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-2.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 10 | R.PDSCH.1-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 15 | R.PDSCH.1-2.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 20 | R.PDSCH.1-2.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 25 | R.PDSCH.1-2.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.8 |
| 30 | R.PDSCH.1-3.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 40 | R.PDSCH.1-3.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.8 |
| 50 | R.PDSCH.1-3.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.0 |

Table 5.2A.3.1.0-3: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-13.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 10 | R.PDSCH.2-13.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 15 | R.PDSCH.2-13.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 20 | R.PDSCH.2-13.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 25 | R.PDSCH.2-13.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 30 | R.PDSCH.2-14.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.6 |
| 40 | R.PDSCH.2-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.7 |
| 50 | R.PDSCH.2-14.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.9 |
| 60 | R.PDSCH.2-14.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.8 |
| 80 | R.PDSCH.2-14.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.1 |
| 90 | R.PDSCH.2-14.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.0 |
| 100 | R.PDSCH.2-15.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.3 |

Table 5.2A.3.1.0-4: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.1.0-1 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-3 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-3 per CC |
| 4 | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-2 per CC |
| 5 | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-2 and Table 5.2A.3.1.0-3 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.5. |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2A.3.1.

5.2A.3.1.1	4Rx Normal PDSCH Demodulation Performance for CA (2DL CA)

5.2A.3.1.1.1	Test Purpose

To verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions for multiple CA configurations and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput per CC.

5.2A.3.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 2DL CA and 4Rx antenna ports on each CC.

5.2A.3.1.1.3	Test description

5.2A.3.1.1.3.1	Initial conditions

Same initial conditions as specified in clause 5.2A.2.1.1.3.1 with the following exception

Channel BW to be tested: largest aggregated bandwidth combination as per Table 5.1.1.5.2-2.

CA capability to be tested: As per table 5.1.1.5.2-1

Table 5.2A.3.1.1.3.1-1: Test point selection table

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.1.0-1 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-3 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-3 per CC |
| 4 (note 2) | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-2 per CC |
| 5 (note 3) | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1.0-2 and Table 5.2A.3.1.0-3 per CC |
| Note 1: For each test point, select any one of the CA configurations which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate based on the equation DataRate=10-3j=1JTBSj2μj Note 2: Test point 4 can be skipped if test point 3 is verified. Note 3: Test point 5 can be skipped if test point 3 or 4 is verified. |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.7 for TE diagram and clause A.3.2.6 for UE diagram.

5.2A.3.1.1.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2 A.3.1.1.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.3.1.0-1 to 5.2A.3.1.0-4 as appropriate on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2A.3.1.0-1 to 5.2A.3.1.0-4 as appropriate on both PCC and SCCs.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.3.1.0-4 as appropriate.

5.2A.3.1.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2A.3.1.1.4	Test Requirement

Tables 5.2A.3.1.1.4-1, 5.2A.3.1.1.4-2 and 5.2A.3.1.1.4-3 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 and A.3.2.2 for each component carrier for throughput test point combination shall meet or exceed the specified value in Table 5.2A.3.1.1.4-1, 5.2A.3.1.1.4-2 and 5.2A.3.1.1.4-3 for the specified SNR including test tolerances for the combination selected following the test rules outlined in 5.1.1.5.2-2.

Table 5.2A.3.1.1.4-1: Test requirements for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.8 |

Table 5.2A.3.1.1.4-2: Test requirements for TDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-2.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 10 | R.PDSCH.1-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 15 | R.PDSCH.1-2.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 20 | R.PDSCH.1-2.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 25 | R.PDSCH.1-2.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.7 |
| 30 | R.PDSCH.1-3.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 40 | R.PDSCH.1-3.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.7 |
| 50 | R.PDSCH.1-3.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.9 |

Table 5.2A.3.1.1.4-3: Test requirements for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-13.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 10 | R.PDSCH.2-13.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 15 | R.PDSCH.2-13.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 20 | R.PDSCH.2-13.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 25 | R.PDSCH.2-13.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 30 | R.PDSCH.2-14.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 40 | R.PDSCH.2-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 50 | R.PDSCH.2-14.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.8 |
| 60 | R.PDSCH.2-14.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.7 |
| 80 | R.PDSCH.2-14.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 10.0 |
| 90 | R.PDSCH.2-14.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.9 |
| 100 | R.PDSCH.2-15.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 10.2 |

5.2A.3.1.2	4Rx Normal PDSCH Demodulation Performance for CA (3DL CA)

5.2A.3.1.2.1	Test Purpose

Same as 5.2A.3.1.1.1

5.2A.3.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 3DL CA and 4Rx antenna ports on each CC.

5.2A.3.1.2.3	Test description

5.2A.3.1.2.3.1	Initial conditions

Same as 5.2A.3.1.1.3.1

5.2A.3.1.2.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2 A.3.1.2.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.3.1.0-1 to 5.2A.3.1.0-4 as appropriate on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2A.3.1.0-1 to 5.2A.3.1.0-4 as appropriate on both PCC and SCCs.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.3.1.0-4 as appropriate.

5.2A.3.1.2.3.3	Message contents

Same as 5.2A.3.1.1.3.3

5.2A.3.1.2.4	Test Requirement

Same as 5.2A.3.1.1.4 evaluated per component carrier.

5.2A.3.1.3	4Rx Normal PDSCH Demodulation Performance for CA (4DL CA)

5.2A.3.1.3.1	Test Purpose

Same as 5.2A.3.1.1.1

5.2A.3.1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4DL CA and 4Rx antenna ports on each CC.

5.2A.3.1.3.3	Test description

5.2A.3.1.3.3.1	Initial conditions

Same as 5.2A.3.1.1.3.1

5.2A.3.1.3.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2 A.3.1.3.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2A.3.1.0-1 to 5.2A.3.1.0-4 as appropriate on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2A.3.1.0-1 to 5.2A.3.1.0-4 as appropriate on both PCC and SCCs.

6.	Measure the average throughput per each component carrier for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

7.	Repeat steps from 1 to 6 for each test points in Table 5.2A.3.1.0-4 as appropriate.

5.2A.3.1.3.3.3	Message contents

Same as 5.2A.3.1.1.3.3

5.2A.3.1.3.4	Test Requirement

Same as 5.2A.3.1.1.4 evaluated per component carrier.

5.2A.3.2	Requirements for 4RX PDSCH carrier aggregation with power imbalance

5.2A.3.2.0	Minimum requirements for carrier aggregation with power imbalance

The performance requirements are specified in Table 5.2A.3.2.0-3 and Table 5.2A.3.2.0-4, with the addition of test parameters in Table 5.2A.3.2.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2A.3.2.0-1.

Table 5.2A.3.2.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the ability of an intra-band adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only |  |

Table 5.2A.3.2.0-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration |  |  | 1x4 |
| PDSCH configuration | Length (L) |  | FDD: 12TDD: 12 for DL slot, 4 for special slot |
|  | PRB bundling size |  | WB |
| Modulation and code rate |  |  | 64QAM, MCS 27 |
| Number of HARQ Processes |  |  | FDD: 4 TDD: 8 |
| Maximum number of HARQ transmission |  |  | 1 |
| Redundancy version coding sequence |  |  | {0} |
| TDD UL-DL pattern |  |  | 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table A.1.2-2 for FR1.30-1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 |
| Overhead for TBS determination |  |  | 0 |
| SSB transmission |  |  | Slot#0 with periodicity 20ms |
| RB assignment |  |  | Full applicable test bandwidth as defined in Table 5.3.5-1 of TS 38.101-1 [2] |

Table 5.2A.3.2.0-3: Minimum performance for FDD CA with 15 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

Table 5.2A.3.2.0-4: Minimum performance for TDD CA with 30 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2A.3.2.

5.2A.3.2.1	4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA)

5.2A.3.2.1.1	Test Purpose

To verify the ability of an intra-band adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only.

5.2A.3.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL intra-band contiguous CA and 4Rx antenna ports.

5.2A.3.2.1.3	Test description

5.2A.3.2.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

Band selection: Supported intra-band contiguous CA configurations covering the lowest and highest operating bands,

Channel BW combination to be tested: Select bandwidth combination with same bandwidth in each carrier. If not supported, select bandwidth combination with smallest bandwidth difference between the two carriers and the carrier with smaller bandwidth will be used for the test.

CA capability to be tested: Either FDD or TDD intra-band contiguous CA

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2A for TE diagram and clause A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2A-1 to Table 5.2A-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2A.3.2.1.3.3.

5.2A.3.2.1.3.2	Test procedure

Same test procedure as specified in clause 5.2A.2.2.1.3.2 with the following exception.

Instead of Table 5.2A.2.2.1.3.3  5.2A.3.2.1.3.3

Instead of Table 5.2A.2.2.0-2  5.2A.3.2.0-2

Instead of Table 5.2A.2.2.0-4  5.2A.3.2.0-4

5.2A.3.2.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2A.3.2.1.4	Test Requirement

Table 5.2A.3.2.0-2, Table 5.2A.3.2.1.4-1 and Table 5.2A.3.2.1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the MCS scheduled as specified in Table 5.2A.3.2.0-2 for the throughput test shall meet or exceed the specified value in Table 5.2A.3.2.1.4-1 or Table 5.2A.3.2.1.4-2 as applicable for the specified cell power levels.

Table 5.2A.3.2.1.4-1: Test Requirements for FDD CA with 15 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

Table 5.2A.3.2.1.4-2: Test Requirements for TDD CA with 30 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

5.2A.3A	2Rx-4RX requirements

5.2A.3A.1	Requirements for 2Rx-4RX normal PDSCH

5.2A.3A.1.0	Minimum conformance requirements for 2Rx-4RX normal PDSCH

For UE supporting a combination of 2Rx and 4Rx antenna port per component carrier,

The 2Rx requirements are defined in Table 5.2A.2.1.0-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.2.1.0-1 to Table 5.2A.2.1.0-3.

The 4Rx requirements are defined in Table 5.2A.3.1.0-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.3.1.0-1 to Table 5.2A.3.1.0-3.

Parameters are specified in Table 5.2A-1 to Table 5.2A-3 and the downlink physical channel setup according to Annex C.2.1.

5.2A.3A.1.1	2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA)

5.2A.3A.1.1.1	Test Purpose

To verify the PDSCH mapping Type A normal performance under combination of 2 and 4 receive antenna conditions for multiple CA configurations and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput per CC.

5.2A.3A.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 2DL CA and 4Rx antenna ports on some of the CC.

5.2A.3A.1.1.3	Test description

5.2A.3A.1.1.3.1	Initial conditions

Same initial conditions as specified in clause 5.2A.2.1.1.3.1 with the following exception

Channel BW to be tested: largest aggregated bandwidth combination as per Table 5.1.1.5.2-2.

CA capability to be tested: As per table 5.1.1.5.2-1

Table 5.2A.3A.1.1.3.1-1: Test point selection table

| Test number | CA duplex mode | Configuration |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | For 2Rx CC, As defined in Table 5.2A.2.1.0-1 For 4Rx CC, as defined in Table 5.2A.3.1.0-1 |
| 2 | TDD 30 kHz + TDD 30 kHz | For 2Rx CC, As defined in Table 5.2A.2.1.0-3 For 4Rx CC, as defined in Table 5.2A.3.1.0-3 |
| 3 | FDD 15 kHz + TDD 30 kHz | For 2Rx CC, As defined in Table 5.2A.2.1.0-1 and Table 5.2A.2.1.0-3 per CC For 4Rx CC, as defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-3 per CC |
| 4 (note 2) | FDD 15 kHz + TDD 15 kHz | For 2Rx CC, As defined in Table 5.2A.2.1.0-1 and Table 5.2A.2.1.0-2 per CC For 4Rx CC, as defined in Table 5.2A.3.1.0-1 and Table 5.2A.3.1.0-2 per CC |
| 5 (note 3) | TDD 15 kHz + TDD 30 kHz | For 2Rx CC, As defined in Table 5.2A.2.1.0-2 and Table 5.2A.2.1.0-3 per CC For 4Rx CC, as defined in Table 5.2A.3.1.0-2 and Table 5.2A.3.1.0-3 per CC |
| Note 1: For each test point, select any one of the CA configurations which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate based on the equation DataRate=10-3j=1JTBSj2μj Note 2: Test point 4 can be skipped if test point 3 is verified. Note 3: Test point 5 can be skipped if test point 3 or 4 is verified. |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.6 and A.3.1.7.7 for TE diagram for 2Rx and 4Rx CC respectively and clause A.3.2.6 for UE diagram

5.2A.3A.1.1.3.2	Test procedure

Same test procedure as specified in clause 5.2A.2.1.1.3.2 for 2Rx CC and with the following exception for 4Rx CC

Instead of Table 5.2A.2.1.0-1  5.2A.3.1.0-1 for 4Rx CC

Instead of Table 5.2A.2.1.0-4  5.2A.3.1.0-4 for 4Rx CC

Instead of Table 5.2A.2.1.1.3.3  5.2A.3.1.1.3.3 for 4Rx CC

5.2A.3A.1.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2A.3A.1.1.4	Test Requirement

Tables 5.2A.3A.1.1.4-1, 5.2A.3A.1.1.4-2, 5.2A.3A.1.1.4-3, 5.2A.3A.1.1.4-4, 5.2A.3A.1.1.4-5, 5.2A.3A.1.1.4-6  define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 and A.3.2.2 for each component carrier for throughput test point combination shall meet or exceed the specified value in Table 5.2A.3A.1.1.4-1, 5.2A.3A.1.1.4-2, 5.2A.3A.1.1.4-3, 5.2A.3A.1.1.4-4, 5.2A.3A.1.1.4-5, 5.2A.3A.1.1.4-6 for the specified SNR including test tolerances for the combination selected following the test rules outlined in 5.1.1.5.2-2.

Table 5.2A.3A.1.1.4-1: Test requirements for FDD 15 kHz SCS for CA configurations (2Rx CC)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.4 |

Table 5.2A.3A.1.1.4-2: Test requirements for TDD 15 kHz SCS for CA configurations (2Rx CC)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-2.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 10 | R.PDSCH.1-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 15 | R.PDSCH.1-2.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.8 |
| 20 | R.PDSCH.1-2.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.9 |
| 25 | R.PDSCH.1-2.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 30 | R.PDSCH.1-3.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.9 |
| 40 | R.PDSCH.1-3.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.2 |
| 50 | R.PDSCH.1-3.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.5 |

Table 5.2A.3A.1.1.4-3: Test requirements for TDD 30 kHz SCS for CA configurations (2Rx CC)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-13.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 10 | R.PDSCH.2-13.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 15 | R.PDSCH.2-13.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.6 |
| 20 | R.PDSCH.2-13.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.7 |
| 25 | R.PDSCH.2-13.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.7 |
| 30 | R.PDSCH.2-14.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.7 |
| 40 | R.PDSCH.2-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 14.9 |
| 50 | R.PDSCH.2-14.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.1 |
| 60 | R.PDSCH.2-14.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.0 |
| 80 | R.PDSCH.2-14.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.5 |
| 90 | R.PDSCH.2-14.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.3 |
| 100 | R.PDSCH.2-15.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 15.7 |

Table 5.2A.3A.1.1.4-4: Test requirements for FDD 15 kHz SCS for CA configurations (4Rx CC)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 [ |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 15 | R.PDSCH.1-9.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 20 | R.PDSCH.1-9.3 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 25 | R.PDSCH.1-9.4 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 30 | R.PDSCH.1-9.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 40 | R.PDSCH.1-10.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 50 | R.PDSCH.1-10.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.8 |

Table 5.2A.3A.1.1.4-5: Test requirements for TDD 15 kHz SCS for CA configurations (4Rx CC)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-2.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 10 | R.PDSCH.1-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 15 | R.PDSCH.1-2.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 20 | R.PDSCH.1-2.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 25 | R.PDSCH.1-2.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.7 |
| 30 | R.PDSCH.1-3.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 40 | R.PDSCH.1-3.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.7 |
| 50 | R.PDSCH.1-3.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.9 |

Table 5.2A.3A.1.1.4-6: Test requirements for TDD 30 kHz SCS for CA configurations (4Rx CC)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-13.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 10 | R.PDSCH.2-13.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 15 | R.PDSCH.2-13.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.4 |
| 20 | R.PDSCH.2-13.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 25 | R.PDSCH.2-13.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 30 | R.PDSCH.2-14.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.5 |
| 40 | R.PDSCH.2-2.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.6 |
| 50 | R.PDSCH.2-14.2 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.8 |
| 60 | R.PDSCH.2-14.3 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.7 |
| 80 | R.PDSCH.2-14.4 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 10.0 |
| 90 | R.PDSCH.2-14.5 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.9 |
| 100 | R.PDSCH.2-15.1 TDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 10.2 |

5.2A.3A.1.2	2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA)

5.2A.3A.1.2.1	Test Purpose

Same as 5.2A.3A.1.1.1

5.2A.3A.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 3DL CA and 4Rx antenna ports on some of the CC.

5.2A.3A.1.2.3	Test description

5.2A.3A.1.2.3.1	Initial conditions

Same as 5.2A.3A.1.1.3.1

5.2A.3A.1.2.3.2	Test procedure

Same as 5.2A.3A.1.1.3.2

5.2A.3A.1.2.3.3	Message contents

Same as 5.2A.3A.1.1.3.3

5.2A.3A.1.2.4	Test Requirement

Same as 5.2A.3A.1.1.4 evaluated per component carrier.

5.2A.3A.1.3	2Rx-4Rx Normal PDSCH Demodulation Performance for CA (4DL CA)

5.2A.3A.1.3.1	Test Purpose

Same as 5.2A.3A.1.1.1

5.2A.3A.1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4DL CA and 4Rx antenna ports on some of the CC.

5.2A.3A.1.3.3	Test description

5.2A.3A.1.3.3.1	Initial conditions

Same as 5.2A.3A.1.1.3.1

5.2A.3A.1.3.3.2	Test procedure

Same as 5.2A.3A.1.1.3.2

5.2A.3A.1.3.3.3	Message contents

Same as 5.2A.3A.1.1.3.3

5.2A.3A.1.3.4	Test Requirement

Same as 5.2A.3A.1.1.4 evaluated per component carrier.

<<Unchanged sections skipped>>

5.5A.1	FR1 Sustained downlink data rate performance for carrier aggregation

5.5A.1.1	FR1 SDR performance for CA (2DLCA)

5.5A.1.1.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement.

5.5A.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA

5.5A.1.1.3	Minimum conformance requirements

The Sustained Data Rate (SDR) requirements in this clause are applicable to the FR1 CA.

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the selected CA bandwidth combination and with the downlink physical channel setup according to Annex C.2.1.

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters are specified in Table 5.5A.1.1.3-1. The parameters specified in Table 5.5A.1.1.3-2 are applicable for tests on FDD CCs and parameters specified in Table 5.5A.1.1.3-3 are applicable for tests on TDD CCs.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz.

Table 5.5A.1.1.3-1: Common test parameters for FDD and TDD component carriers

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5A-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL 1 for 30 kHz / 5 MHz 1/AL4 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 1/AL 8 for other combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For 2Tx: Single Panel Type I, Random precoder chosen from precoder index 0 and 2, selection updated per slot For 4Tx: Single Panel Type I, Random precoder chosen from precoders with i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2}, selection updated per slot |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | wideband |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

Table 5.5A.1.1.3-2: Additional test parameters for FDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 4 |
| K1 value |  |  | 2 |

Table 5.5A.1.1.3-3: Additional test parameters for TDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 |
| K1 value |  |  | Specific to each UL-DL pattern |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| Note 1: PDSCH is scheduled only on full DL slots |  |  |  |

Table 5.5A.1.1.3-4: Number of PRBs in CORESET

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 48 | 78 | 102 | 132 | 156 | 186 | 216 | 240 | 270 | N/A | N/A | N/A |
| 30 | 6 | 24 | 36 | 48 | 60 | 78 | 90 | 102 | 114 | 132 | 162 | 216 | 270 |

Table 5.5A.1.1.3-5: MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 8 | 1 | 26 |
| 1 | 8 | 0.8 | 21 |
| 1 | 8 | 0.75 | 20 |
| 1 | 8 | 0.4 | 11 |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 8 | 1 | 26 |
| 2 | 8 | 0.8 | 21 |
| 2 | 8 | 0.75 | 20 |
| 2 | 8 | 0.4 | 11 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| 2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |
| 4 | 8 | 1 | 26 |
| 4 | 8 | 0.8 | 23 |
| 4 | 8 | 0.75 | 22 |
| 4 | 8 | 0.4 | 12 |
| 4 | 6 | 1 | 27 |
| 4 | 6 | 0.8 | 24 |
| 4 | 6 | 0.75 | 23 |
| 4 | 6 | 0.4 | 14 |
| 4 | 4 | 1 | 16 |
| 4 | 4 | 0.8 | 16 |
| 4 | 4 | 0.75 | 16 |
| 4 | 4 | 0.4 | 11 |
| 4 | 2 | 1 | 9 |
| 4 | 2 | 0.8 | 9 |
| 4 | 2 | 0.75 | 9 |
| 4 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

5.5A.1.1.3.1	Procedure for test parameter selection

The test parameters are determined by the following procedure:

-	Select one CA bandwidth combination among all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate in accordance with clause 4.1.2 of TS 38.306 [14].

-	Set of per CC UE capabilities includes channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor in accordance with clause 4.1.2 of TS 38.306 [14].

-	When there are multiple sets of CA bandwidth combinations and UE capabilities (channel bandwidth, subcarrier spacing, number of MIMO layer, modulation format, scaling factor) with same largest data rate, select one among sets with the smallest aggregated channel bandwidth.

-	For each CC in CA bandwidth combination, use Table 5.5A.1.1.3-5 to determine MCS based on test parameters and indicated UE capabilities.

The normative reference for this requirement is TS 38.101-4 [5], clause 5.5A.1.

5.5A.1.1.4	Test description

5.5A.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of TS 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

1.	Connect the SS  to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2A, A.3.1.7.6, and A.3.1.7.7 for TE diagram (without fader and AWGN) for 2Rx and 4Rx CC respectively and clause A.3.2.6 for UE diagram.

2.	The parameter settings for the NR cell are initially set up according to Table 5.5A.1.1.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR with Connected without release On, Test Loop Function On with UE Test Loop Mode A with UL_PDCP_SDU_SIZE = 0 according to TS 38.508-1 [6] clause 4.5.4. Message content are defined in clause 5.5A.1.1.4.3.

6.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

7.	The SS shall configure SCC as per TS 38.508-1 [6] clause 5.5.1.

8.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

9.	SS shall transmit UECapabilityEnquiry message.

10.	The UE shall transmit UECapabilityInformation message.

11.	Using the UE capabilities advertised in the UE-CapabilityRAT-Container of the type UE-NR-Capability, and the procedure outlined in 5.5A.1.1.3.1 determine one set of parameters that would provide the largest data rate.

12.	Setup up the NR cells using these parameters for the test.

13.	Configure the TBsize, DL RMC, UL RMC, PDCP size from Annex A.3.2_1 and Annex A.2.2 for UL as appropriate.

5.5A.1.1.4.2	Test procedure

1.	SS configures T-reordering timer to be infinity.

2.	SS sends a PDCP reestablishment via RRC Reconfiguration message requesting for PDCP Status Report.

3.	SS sets the counters NDL_newtx NDL_retx to 0.

4.	For each new DL HARQ transmission the SS generates sufficient PDCP SDUs (max PDCP SDU size and minimum  number of consecutive PDCP SDUs) to fill up the TB on both PCC and SCC in accordance with Annex A.3.2_1. The SS ciphers the PDCP SDUs, concatenates the resultant PDCP PDUs to form an RLC PDU and then a MAC PDU. The SS transmits the MAC PDU. The SS increments then NDL_newtx by one

5.	If PHY requests a DL HARQ retransmission, the SS performs a HARQ retransmission and increments NDL_retx by one.

6.	Steps 5 to 6 are repeated at every TTI for at least 300 frames and the SS waits for 300ms to let any HARQ retransmissions and RLC retransmissions to finish.

7.	SS sends a PDCP reestablishment via RRC Reconfiguration message requesting for PDCP Status Report.

8.	The SS calculates the TB success rate as A = 100%* NDL_correct_rx / (NDL_newtx + NDL_retx).

9.	SS computes the PDCP SDU loss by looking into the FMC and Bitmap field in the PDCP Status Report. PDCP SDU loss B = COUNT reported in the Bitmap field of PDCP Stata Report.

10.	The UE passes the test if A ≥  85% TB success rate on both PCC and SCC and B = 0.

Note 1:	In case of RLC PDU retransmission, the number of new required PDCP SDUs is as many as to fill the rest of TB.

5.5A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 5.4.2 with the following exceptions

Table 5.5A.1.1.4.3-1: CLOSE UE TEST LOOP (in the preamble)

| Derivation Path: TS 38.509 [9] clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 0 0 0 |  |  |
| UE test loop mode |  | 0 0 0 0 0 0 0 0 | UE test loop mode A |  |
| UE test loop mode A LB setup |  |  |  |  |
| Length of UE test loop mode A LB setup list in bytes |  | 0 0 0 0 0 0 1 1 | Length of one LB setup DRB (3 bytes) |  |
| LB setup DRB |  | 0 0 0 0 0 0 0 0, 0 0 0 0 0 0 0 0, 0 0 Q5 Q4 Q3 Q2 Q1 Q0 | UL PDCP SDU size = 0 Q5 = 1 (for NR Data Radio Bearers) Q4..Q0 = Data Radio Bearer identity number -1 for the radio bearer. See 38.509 clause 6.3.1 |  |
| UE test loop mode B LB setup |  | Not present |  |  |

Table 5.5A.1.1.4.3-2: RadioBearerConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  | DRB1 |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB1 |  |  |
| reestablishPDCP | true |  | DRB1 AND Re-establish_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |

Table 5.5A.1.1.4.3-3: PDCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| headerCompression CHOICE { |  |  |  |
| notUsed | Null |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true |  |  |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| t-Reordering | Not present |  |  |
| } |  |  |  |

5.5A.1.1.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.

<<Unchanged sections skipped>>

F.1.1.2	Measurement of Demod Performance requirements

This clause defines the maximum test system uncertainty for Demod Performance requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.2-1.

Table F.1.1.2-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | ±2.0 | Same as in LTE |
| Signal to noise ratio uncertainty | dB | ±0.3 | Same as in LTE |
| Signal to noise ratio variation | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 1Tx | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 2Tx | dB | ±0.7 | Same as in LTE |
| Fading profile power uncertainty for 4Tx | dB | ±0.7 | Same as in LTE |

The maximum test system uncertainty for test cases defined in section 5 is defined in Table F.1.1.2-2.

Table F.1.1.2-2: Maximum test system uncertainty for FR1 demodulation performance test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler ± 0.6 dB for test 1-6 ± 0.9 dB for test 1-7 | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) + SNR uncertainty due to finite test time2 Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB For test point 1-6, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For test point 1-7, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.2_1 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.3_1 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | [± 0.6 dB] | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | [0.7dB] | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness 3. SNR uncertainty due to finite test time Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±[0.4] dB for 1% residual BLER |
| 5.2.2.1.7_1 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.8_1 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.9_1 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | ± 0.6 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty, ±0.3 dB 2. Effect of AWGN flatness and signal flatness, ±2.0 dB 3. SNR uncertainty due to finite test time, ±0.0 dB for >10Hz Doppler. Items 1, 2, 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB |
| 5.2.2.1.10_1 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | ± 0.6 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty, ±0.3 dB 2. Effect of AWGN flatness and signal flatness, ±2.0 dB 3. SNR uncertainty due to finite test time, ±0.0 dB for >10Hz Doppler. Items 1, 2, 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB |
| 5.2.2.1.11_1 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.12_1 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.13_1 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.1.14_1 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.6_1 | Same as 5.2.2.1.6_1 |
| 5.2.2.2.1_1 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | ± 0.9 dB for test 1-10 ± 0.6 dB for test 1-11 For other TPs, same as 5.2.2.1.1_1 | For test point 1-10, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB For test point 1-11, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For other TPs, same as 5.2.2.1.1_1 |
| 5.2.2.2.1_2 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.2_1 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.3_1 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.5_1 2Rx TDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.5_1 | Same as 5.2.2.1.5_1 |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.6_1 | Same as 5.2.2.1.6_1 |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.8_1 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.9_1 | Same as 5.2.2.1.9_1 |
| 5.2.2.2.10_1 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.10_1 | Same as 5.2.2.1.10_1 |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.12_1 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.13_1 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2.2.2.14_1 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.2.1.6_1 | Same as 5.2.2.1.6_1 |
| 5.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | ± 0.9 dB for > 10Hz doppler ± 1.0 dB for 10Hz doppler | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB |
| 5.2.3.1.1_2 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.1_4 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.2_1 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.3_1 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.5_1 | Same as 5.2.2.1.5_1 |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.6_1 | Same as 5.2.2.1.6_1 |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.8_1 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.9_1 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.9_1 | Same as 5.2.2.1.9_1 |
| 5.2.3.1.10_1 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.10_1 | Same as 5.2.2.1.10_1 |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.12_1 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.13_1 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.1.14_1 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA | Same as 5.2.3.1.6_1 | Same as 5.2.3.1.6_1 |
| 5.2.3.2.1_1 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.1_2 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.1_4 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.2_1 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.4_1 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.5_1 | Same as 5.2.2.1.5_1 |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.2.1.6_1 | Same as 5.2.2.1.6_1 |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.9_1 | Same as 5.2.3.1.9_1 |
| 5.2.3.2.10_1 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | Same as 5.2.3.1.10_1 | 5.2.3.1.10_1 |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.12_1 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.13_1 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2.3.2.14_1 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Same as 5.2.3.1.6_1 | Same as 5.2.3.1.6_1 |
| 5.2A.2.1.1 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2A.2.1.2 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2A.2.1.3 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | Same as 5.2.2.1.1_1 | Same as 5.2.2.1.1_1 |
| 5.2A.2.2.1 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 6% | 6% EVM is equivalent to a Test system downlink SNR of 24.4dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 5.2A.3.1.1 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2A.3.1.2 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2A.3.1.3 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | Same as 5.2.3.1.1_1 | Same as 5.2.3.1.1_1 |
| 5.2A.3A.1.1 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |
| 5.2A.3A.1.2 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |
| 5.2A.3A.1.3 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |
| 5.3.2.1.1 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | ± 0.9 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for 1Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB |
| 5.3.2.1.2 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | ± 1.0 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB |
| 5.3.2.1.3 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.2.2.1 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.2.2.2 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.2 | Same as 5.3.2.1.2 |
| 5.3.2.2.3 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.3.1.1 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.2 | Same as 5.3.2.1.2 |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Same as 5.3.2.1.2 | Same as 5.3.2.1.2 |
| 5.3.3.2.3 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Same as 5.3.2.1.1 | Same as 5.3.2.1.1 |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 | E-UTRA CC: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz NR CC: Same as 5.5.1 | Same as 5.5.1 |
| 11.1.2 PSSCH demodulation requirements | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 11.1.3 PSCCH demodulation requirements | Same as 11.1.2 | Same as 11.1.2 |
| 11.1.5 PSFCH demodulation requirements | Same as 11.1.2 | Same as 11.1.2 |
| 11.1.6 Power imbalance performance with two links | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1, and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 11.1.7 HARQ buffer soft combining test | Same as 11.1.6 | Same as 11.1.6 |
| 11.1.8 PSCCH decoding capability | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 11.1.9 PSFCH decoding capability | Same as 11.1.8 | Same as 11.1.8 |

<<Unchanged sections skipped>>

F.1.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 5 is defined in Table F.1.3.2-1.

Table F.1.3.2-1: Derivation of Test Requirements (FR1 demodulation performance tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.6 dB for test 1-6 0.9 dB for test 1-7 | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.2_1 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.3_1 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | [0.7] | Formula: SNR + TT |
| 5.2.2.1.7 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.8_1 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.9_1 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.10_1 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.11_1 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.12_1 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.13_1 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.14_1 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | [0.7] | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.1_1 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.9 dB for test 1-10 0.6 dB for test 1-11 | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.1_2 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.2_1 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.3_1 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | [0.6 dB] | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | [0.6 dB] | Formula: SNR + TT |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.8_1 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.10_1 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.12_1 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.13_1 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.14_1 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.1_2 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.1_4 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | SNRs as specified | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.2_1 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.3_1 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | [0.6 dB] | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.8_1 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.9_1 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.10_1 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.12_1 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.13_1 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.14_1 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.2_1 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.4_1 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | [0.6 dB] | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.8_1 4Rx TDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.10_1, 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.12_1 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.13_1 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.14_1 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT T-put limit unchanged |
| 5.2A.2.1.1 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2A.2.1.2 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2A.2.1.3 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2A.2.2.1 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | Power level as specified | No TT added | T-put limit unchanged |
| 5.2A.3.1.1 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2A.3.1.2 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2A.3.1.3 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2A.3A.1.1 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | SNRs as specified | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 5.2A.3A.1.2 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 5.2A.3A.1.3 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 11.1.2 PSSCH demodulation requirements | SNR as specified | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 11.1.3 PSCCH demodulation requirements | SNR as specified | 0.8 dB | Formula: SNR + TT missing detection probability limit unchanged |
| 11.1.5 PSFCH demodulation requirements | SNR as specified | 0.8 dB | Formula: SNR + TT missing detection probability limit unchanged |
| 11.1.6 Power imbalance performance with two links | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 11.1.7 HARQ buffer soft combining test | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 11.1.8 PSCCH decoding capability | sidelink power | 0 dB | sidelink power unchanged missing detection probability limit unchanged |
| 11.1.9 PSFCH decoding capability | sidelink power | 0 dB | sidelink power unchanged missing detection probability limit unchanged |
| 5.3.2.1.1 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.2.1.2 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.2.1.3 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.2.2.1 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.2.2.2 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.2.2.3 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.3.1.1 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance power saving | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |

<<End of change>>