3GPP TSG-RAN5 Meeting #96-e	R5-224730

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3164 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR TC 7.1.2.3.7 to test RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, RedCap UE use 12bit SN for AM RLC, so the RLC header is 2 bytes. 2. the length of RLC Status Report with NACK_SN for 12bit SN and 18 bit SN is different. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, The size RLC SDU during the test will be added 1byte for RedCap UE. 2, change UL Grant for Status Report with NACK_SN for RedCap UE. 3. Update the Note for TBSIZE calculation for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.2.3.7	AM RLC / Receiver status triggers

7.1.2.3.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Reception failure of an RLC data PDU is detected and t-Reassembly expires }

then { UE initiates Status Reporting }

}

(2)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Status Reporting is triggered and t-StatusProhibit is running }

then { UE wait until t-StatusProhibit has expired to send Status Report}

}

(3)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Polling from peer AM RLC entity is detected and the sequence number ‘x’ of the PDU that carries the Poll satisfies x < RX_Highest_Status or x >= RX_Next + AM_Window_Size }

then { UE initiates Status Reporting }

}

(4)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Polling from peer AM RLC entity is detected and the sequence number ‘x’ of the PDU that carries the Poll does not satisfies x < RX_Highest_Status or x >= RX_Next + AM_Window_Size }

then { UE waits until ‘x < RX_Highest_Status or x >= RX_Next + AM_Window_Size’ before initiating Status Reporting}

}

(5)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the UE needs to send a Status Report and the UL grant is not large enough to accommodate the whole report }

then { UE includes as many NACK_SNs in the Status Report as allowed by the UL grant }

}

(6)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the UE needs to send a Status Report and continuous sequence of RLC SDUs that have not been received yet }

then { UE includes NACK_SN with NACK range }

}

7.1.2.3.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clause 5.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.4]

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in subclause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

7.1.2.3.7.3	Test description

7.1.2.3.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.7.3.1-1.

Table 7.1.2.3.7.3.1-1: RLC parameters

| t-Reassembly | ms150 |
| --- | --- |
| t-StatusProhibit | ms300 |
| t-PollRetransmit | ms500 |

7.1.2.3.7.3.2	Test procedure sequence

Table 7.1.2.3.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 1 | The SS transmits 4 AMD PDUs with SN=0, 1, 2, and 4. The SS sets the P field of all the AMD PDUs to 0. A time spacing of 20 ms is applied. Record time TA when the AMD PDU with SN=4 is sent.(Note 9) | <-- | AMD PDU (SN=0, P=0) AMD PDU (SN=1, P=0) AMD PDU (SN=2, P=0) AMD PDU (SN=4, P=0) | - | - |
| 2 | The SS waits for 70 ms after the transmission of the first AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 3 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits (UL Grant Allocation type 2). (Note 1) (Note 6) | <-- | (UL grants, 848 bits) | - | - |
| 3 | The UE transmits RLC SDU#1. | --> | (RLC SDU#1) | - | - |
| 4 | The UE transmits RLC SDU#2. | --> | (RLC SDU#2) | - | - |
| 5 | The UE transmits RLC SDU#3. | --> | (RLC SDU#3) | - | - |
| 6 | 60 ms after step 5, the SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 7 | 80 ms after step 5, the SS starts the UL default grant transmission. | - | - | - | - |
| 8 | Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? Record time TB (Note 5) Check 2: (TB – TA) = t-Reassembly? | --> | STATUS PDU | 1 | P |
| 9 | 100 ms after the Status Report is received at Step 8, the SS transmits 4 AMD PDUs with SN=5, 6, 8 and 9. The SS sets the P field of all the AMD PDUs to 0. A time spacing of 20 ms is applied. (Note 9) | <-- | AMD PDU (SN=5, P=0) AMD PDU (SN=6, P=0) AMD PDU (SN=8, P=0) AMD PDU (SN=9, P=0) | - | - |
| 10 | Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=7? Record time TC Check 2: (TC – TB) = t-StatusProhibit? | --> | STATUS PDU | 2 | P |
| 11 | The SS ignores scheduling requests unless otherwise specified and does not allocate any uplink grant. | - | - | - | - |
| 12 | 200 ms after step 10, the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. (Note 9) | <-- | AMD PDU (SN=3, P=0) AMD PDU (SN=7, P=1) | - | - |
| 13 | The SS waits for 100 ms after the transmission of the last AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | (UL grant, 88 bits) | - | - |
| 14 | Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN = 10? | --> | STATUS PDU | 3 | P |
| 15 | In the second frame following the one scheduled in step 13 the SS assigns 7 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits. (Note 1) (Note 6) | <-- | (UL grant, 848 bits) | - | - |
| 16 | The UE transmits RLC SDU#4. | --> | (RLC SDU#4) | - | - |
| 17 | The UE transmits RLC SDU#5. | --> | (RLC SDU#5) | - | - |
| 18 | The UE transmits RLC SDU#6. | --> | (RLC SDU#6) | - | - |
| 19 | The UE transmits RLC SDU#7. | --> | (RLC SDU#7) | - | - |
| 20 | The UE transmits RLC SDU#8. | --> | (RLC SDU#8) | - | - |
| 21 | The UE transmits RLC SDU#9. | --> | (RLC SDU#9) | - | - |
| 22 | The UE transmits RLC SDU#10. | --> | (RLC SDU#10) | - | - |
| 23 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 24 | After 300 ms the SS transmits an AMD PDU with SN=11 and P=0, and an AMD PDU with SN=12 and P=1.A time spacing of 20 ms is applied. (Note 9) | <-- | AMD PDU (SN=11, P=0) AMD PDU (SN=12, P=1) | - | - |
| 25 | Check: Does the UE transmit a scheduling request within t-Reassembly/ 2 ms after the transmission of the first AMD PDU of Step 24? | --> | (SR) | 4 | F |
| 26 | At t-Reassembly/ 2 ms after the transmission of the second AMD PDU of Step 24, the SS transmits an AMD PDU with SN=10 and P=0. (Note 9) | <-- | AMD PDU (SN=10, P=0) | - | - |
| 27 | The SS waits for 60 ms to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | (UL grant, 88 bits) | - | - |
| 28 | Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN=13? | --> | STATUS PDU | 4 | P |
| 29 | The SS assigns 3 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits. (Note 1) | <-- | (UL grant, 848 bits) | - | - |
| 30 | The UE transmits RLC SDU#11. | --> | (RLC SDU#11) | - | - |
| 31 | The UE transmits RLC SDU#12. | --> | (RLC SDU#12) | - | - |
| 32 | The UE transmits RLC SDU#13. | --> | (RLC SDU#13) | - | - |
| 33 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| 34 | After 300 ms the SS transmits an AMD PDU with SN=17 and P=0, and an AMD PDU with SN=19 and P=1. A time spacing of 20 ms is applied. (Note 9) | <-- | AMD PDU (SN=17, P=0) AMD PDU (SN=19, P=1) | - | - |
| 35 | The SS waits for t-Reassembly ms to ensure expiry. | - | - | - | - |
| 36 | 60 ms after step 35 the SS assigns an UL grant (UL grant allocation type 3) of size 88 bits. (Note 3) (Note 7) | <-- | (UL Grant, 88 bits) | - | - |
| 37 | Check: Does the UE transmit a Status Report with ACK_SN=18 and NACK_SN: 13 including NACK Range 4 (SN 13, 14, 15, 16)? | --> | STATUS PDU | 5,6 | P |
| 38 | After 300 ms the SS transmits an AMD PDU with SN=16 and P=1. (Note 9) | <-- | AMD PDU (SN=16, P=1) | - | - |
| 39 | 30 ms after step 38 the SS assigns an UL grant (UL grant allocation type 3) of size 112 bits. (Note 4) (Note 8) | <-- | (UL Grant, size 112) | - | - |
| 40 | Check: Does the UE transmit a Status Report with ACK_SN=20 and NACK_SN: 13 including NACK Range 3 (SN 13, 14, 15) and NACK_SN=18 without NACK Range? | --> | STATUS PDU | 5,6 | P |
| 41 | 60 ms after step 38 the SS transmits 4 AMD PDUs with SN=13, 14, 15 and 18. A time spacing of 20 ms is applied. (Note 9) | <-- | AMD PDU (SN=13, P=0) AMD PDU (SN=14, P=0) AMD PDU (SN=15, P=0) AMD PDU (SN=18, P=0) | - | - |
| 42 | 130 ms after the transmission of the first AMD PDU the SS assigns 7 UL grant (UL grant allocation type 3 with a time spacing of 20 ms of size 848 bits. (Note 1) (Note6) | <-- | (UL grant, 848 bits) | - | - |
| 43 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#14) | - | - |
| 44 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#15) | - | - |
| 45 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#16) | - | - |
| 46 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#17) | - | - |
| 47 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#18) | - | - |
| 48 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#19) | - | - |
| 49 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#20) | - | - |
| 50 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 848 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time ( 99 bytes RLC SDU + 3 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). Note 2: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit a Status Report with ACK_SN(3 byte) + 2 byte MAC PDU subheader and (2 byte short BSR). 4 Bytes additional space provided to confirm UE does not include NACK_SN and conformant UE instead will include MAC Padding. Note 3: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes)and 1 NACK_SNs with NACK Range(4 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte). Note 4: UL grant of 112 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes)and 1 NACK_SNs with NACK Range(4 Bytes) +NACK SN (3 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte). Note 5: Time TB should be recorded by factoring in the SR-Periodicity as configured in the SchedulingRequestResourceConfig->periodicityAndOffset Note 6: If pc_RedCap_r17=True, UL grant of 848 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time ( 100 bytes RLC SDU + 2 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). Note 7: If pc_RedCap_r17=True, UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range (3 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Bytes) + Padding (1 Bytes). Note 8: If pc_RedCap_r17=True, UL grant of 112 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range (3 Bytes) +NACK SN (2 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Bytes) + Padding (2 Bytes). Note 9: Every DL AMD PDU contains 1 RLC SDU size of 99 bytes for pc_RedCap_r17=False and 100 bytes for pc_RedCap_r17=True. |  |  |  |  |  |

7.1.2.3.7.3.3	Specific message contents

None

<End of modified section 1>