3GPP TSG-RAN5 Meeting #96-e	R5-224740

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2495 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Default configuration for DCI format 4_1 scheduling MBS Multicast test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, There is no default configuration for DCI format 4_1 scheduling MBS |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add default configuration for DCI format 4_1 scheduling MBS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCI format 4_1 is not configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6.1.X.2	Physical layer parameters for DCI format 4_1

DCI format 4_1 is used for the scheduling of PDSCH for multicast in DL cell.

Default physical layer parameters for DCI format 4_1 are specified in Table 4.3.6.1.X.2-1.

Table 4.3.6.1.X.2-1: Physical layer parameters for DCI format 4_1

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Frequency domain resource assignment | Dependent on test parameters | - |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |
| VRB-to-PRB mapping | Non-interleaved | “0” |
| Modulation and coding scheme | Dependent on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |
| Redundancy version | Dependent on test parameters | - |
| HARQ process number | Dependent on test parameters | - |
| Downlink assignment index | Dependent on test parameters | - |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | “110” |
| PDSCH-to-HARQ_feedback timing indicator | K1 slots, dl-DataToUL-ACK-MulticastDciFormat4-1 is not provided, as specified in 9.2.3 in TS 38.213 |  |
|  | µ=0 (SCS=15kHz): K1=7 | '110'B |
|  | µ=1 (SCS=30kHz): K1=5 | '100'B |
|  | µ=3 (SCS=120kHz): K1=8 | '111'B |
| Reserved bits | Reserved 3 bits | - |

<End of modified section 1>