3GPP TSG-RAN5 Meeting #96-e	R5-225679

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0779 | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of FR2 5 to 8UL CA Test Cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FR2 5 to 8UL CA ACLR & Gen Spurious emissions Test cases are incomplete with MU/TT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of FR2 5 to 8UL CA Test Cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 5 to 8UL CA for ACLR & Gen Spurious emissions test cases won’t be accepted to test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Clause 6.5A.3.1.1.4.1 step 1, “section A.3.4.1.1” is changed as “Figure A.3.4.1.1”. - The statement of “Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz” is removed for 5UL ~ 8UL CA in F.1.2 & F.3.2. - The WIC of “5GS_NR_LTE-UEConTest” is included. R2: - Editor’s note change of Test cases are removed and only keep the update in F.1.2 & F.3.2 - “Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz” is added as there has not been any discussion on FR2 MU for an aggregated bandwidth > 400 MHz - “Inter-band” is removed from F.1.2 & F.3.2 This is the final outcome of R5-224370 and the result of 1 revision |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| Many columns are skipped |  |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.4 Transmitter Spurious emissions for CA (5UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |
| 6.5A.3.1.5 Transmitter Spurious emissions for CA (6UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |
| 6.5A.3.1.6 Transmitter Spurious emissions for CA (7UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |
| 6.5A.3.1.7 Transmitter Spurious emissions for CA (8UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD |  |

## << Unchanged sections omitted >>

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.87 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) Max EIRP 0 dB | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |
| Many columns are skipped |  |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.5A.3.1.4 Transmitter Spurious emissions for CA (5UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |
| 6.5A.3.1.5 Transmitter Spurious emissions for CA (6UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |
| 6.5A.3.1.6 Transmitter Spurious emissions for CA (7UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |
| 6.5A.3.1.7 Transmitter Spurious emissions for CA (8UL CA) | Intra-band contiguous CA 400 MHz < aggregated BW ≤ TBD MHz Intra-band non-contiguous CA TBD | TBD |

## << End of changes >>