# Initial proposal 3rd July

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type            p_NR_TargetCellId,

NextHopChainingCount      p_NextHopChainingCount := 0,

template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit,

template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *

) runs on NR_BASE_PTC

{ // @sic R5-206369 R5-213516: added p_MacBearerRoutingCA R5s220372  sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_TargetCellId);

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO (v_NR_PhysicalParameters, v_RNTI_Value);

// 3. Void

//@sic R5-221467 sic@

// 4. void

// @sic R5-217830 sic@

// 5. Target Cell: Activate Security

f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true);

// 5a.	Target Cell: Reconfigure RACH procedure with contention-based with C-RNTI based contention resolution

// R5w220210

f_NR_SS_RachProcedureConfig(p_NR_TargetCellId, v_RachProcedureConfig);

// 6. Target : Send RRCReestablishment

SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA));

// 7. Target Cell:  Receive RRCReestablishmentComplete

SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) );

}

# Updated proposal correcting DCI to DCI 0_1 ->  0_0

Changes compared to TTCN delivery are shown with revision marks.

Changes compared to the proposal on 3rd July (cl 1) are shown in blue

template (value) NR_RachProcedureConfig_Type cs_NR_RachProcedureConfig_CRNTI_HO(NR_PhysicalParameters_Type p_PhysicalParameters,

RNTI_Value_Type p_TempC_RNTI_Value := tsc_C_RNTI_Value3,

template (value) NR_DciFormat_0_X_SpecificInfo_Type p_DciFormatAndParams := cs_NR_DciFormat_0_1_Params

) :=

{ /* @status    APPROVED (ENDC, NR5GC, NR5GC_IRAT) */

/* Acc to 38.523-3 cl 7.3.5.3.3  */

RachProcedureList := {{

RandomAccessResponse := cs_NR_RandomAccessResponseConfig(p_PhysicalParameters, cs_NR_RAR_SubPdu_RapIdAndPayload(-,

tsc_NR_RACH_TA_Def,

f_NR_RAR_UplinkGrant(p_PhysicalParameters.UL_BWPs[0], tsc_NR_ENDC_Msg3Grant_Def), // Table 7.1.2.3.3-1: Default grants

cs_NR_TempC_RNTI_Explicit(p_TempC_RNTI_Value))),

ContentionResolution := cs_NR_ContentionResolutionCtrl_CrntiBased(p_PhysicalParameters, f_NR_ResourceAllocation_UplinkGrant(tsc_NR_Msg4GrantHO_Def, p_PhysicalParameters.UL_BWPs[0]),-,-,p_DciFormatAndParams) // Table 7.1.2.3.3-1: Default grants @sic R5s191200, R5s200076 sic@

}}

};

//==============================================================================================================

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type            p_NR_TargetCellId,

NextHopChainingCount      p_NextHopChainingCount := 0,

template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit,

template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *

) runs on NR_BASE_PTC

{ // @sic R5-206369 R5-213516: added p_MacBearerRoutingCA R5s220372  sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_TargetCellId);

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, v_RNTI_Value, cs_NR_DciFormat_0_0_Params);

// 3. Void

//@sic R5-221467 sic@

// 4. void

// @sic R5-217830 sic@

// 5. Target Cell: Activate Security

f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true);

// 5a.   Target Cell: Reconfigure RACH procedure with contention-based with C-RNTI based contention resolution

// R5w220210

f_NR_SS_RachProcedureConfig(p_NR_TargetCellId, v_RachProcedureConfig);

// 6. Target : Send RRCReestablishment

SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA));

// 7. Target Cell:  Receive RRCReestablishmentComplete

SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) );

}