ASP changes to add the symbol in the Timing Info

In the PUSCH Type B repetition test case, the actual repetitions can happen in the same slot but start from different symbols, it is proposed to add the symbol in SubFrameTiming_Type. SS will report the MAC PDUs in separate ASPs with the corresponding start symbol value. The start symbol is also needed in the CRC error ASP.

It is proposed to handle the field Symbol in a similar way as done for Slot.

The changes having a widespread impact, it is proposed to do the implementation in the Rel-17 baseline upgrade delivery iWD 22wk43.

#### D.10.4.1.2	Timing_Info

Timing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SystemFrameNumber_Type | integer (0..1023) |  |
| SubFrameNumber_Type | integer (0..9) |  |
| HyperSystemFrameNumberInfo_Type | SystemFrameNumberInfo_Type |  |
| SymbolNumber_Type | integer (0..13) |  |

SubFrameInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SubFrameInfo_Type |  |
| Comment |  |  |
| Number | SubFrameNumber_Type |  |
| Any | Null_Type | no specific sub-frame, in which case the SS may choose the next available subframe (valid for REQ ASPs only) |

SystemFrameNumberInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemFrameNumberInfo_Type |  |
| Comment |  |  |
| Number | SystemFrameNumber_Type |  |
| Any | Null_Type | no specific frame number, in which case the SS may choose the next available SFN (valid for REQ ASPs only) |

SlotOffset_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotOffset_Type |  |
| Comment | slots per subframe according to TS 38.211 Table 4.3.2-1 |  |
| Numerology0 | Null_Type | mu=0; only one slot per subframe |
| Numerology1 | integer (0..1) | mu=1; 2 slots per subframe |
| Numerology2 | integer (0..3) | mu=2; 4 slots per subframe |
| Numerology3 | integer (0..7) | mu=3; 8 slots per subframe |
| Numerology4 | integer (0..15) | mu=4; 16 slots per subframe |

SlotTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SlotTimingInfo to "FirstSlot" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SlotTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the slot number is not relevant, in which case the SS may choose the next available slot of the given subframe IND ASPs: Any:=true only if there is no slot information available for the particular kind of indication |  |
| SlotOffset | SlotOffset_Type | to address a particular slot in a subframe |
| FirstSlot | Null_Type | to address the first slot independent from the numerology (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if slot number is not relevant or not available |

SymbolTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SymbolTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the symbol number is not relevant, in which case the SS may choose the next available symbol of the given slot IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication |  |
| SymbolOffset | SymbolNumber_Type | to address a particular symbol in a slot |
| FirstSymbol | Null_Type | to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available |

SubFrameTiming_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SubFrameTiming_Type |  |  |
| Comment |  |  |  |
| SFN | SystemFrameNumberInfo_Type |  |  |
| Subframe | SubFrameInfo_Type |  |  |
| HSFN | HyperSystemFrameNumberInfo_Type |  |  |
| Slot | SlotTimingInfo_Type |  |  |
| Symbol | SymbolTimingInfo_Type |  |  |

TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TimingInfo_Type |  |
| Comment |  |  |
| SubFrame | SubFrameTiming_Type |  |
| Now | Null_Type | to be used in REQ ASPs when there is no 'activation time' |
| None | Null_Type | only to be used in SYSTEM_CTRL_CNF or NR_SYSTEM_CTRL_CNF but not for EnquireTiming |