3GPP TSG-RAN5 Meeting #97	R5-226841

# Toulouse, France, 14th November 2022 – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0832 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test case addition: 6.5.2.1_1 Spectrum Emission Mask with Power Boost |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no test defined to cover Spectrum Emission Mask under mpr-PowerBoost conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a new test case to enable testing of Rel-16 FR2 RF Enhancements Spectrum Emission Mask with Power Boost as defined in R5-226836. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spectrum Emission Mask could be higher than core requirements under mpr-PowerBoost conditions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1_1 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document depends on R5-226836. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.5.2.1	Spectrum Emission Mask

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

6.5.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.5.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.2.1.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than FOOB as specified in Table 6.5.2.1.3-1 the spurious requirements in clause 6.5.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.1.3-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 6.5.2.1.3-1: General NR spectrum emission mask for Range 2.

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.5.2.1.

6.5.2.1.4	Test description

6.5.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 4 |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 5 |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: All test points in this table must also exist in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-3 (MPR) for PC1 or Table 6.2.2.4.1-7, Table 6.2.2.4.1-8, Table 6.2.2.4.1-9 (MPR) for PC2, PC3 and PC4. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and clause A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.2.1.4.3

6.5.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure the TRP of the transmitted signal with a measurement filter of bandwidths according to Table 6.5.2.1.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. TRP shall be recorded for each step. The measurement period shall capture the active time slots. Total radiated power is measured according to TRP measurement procedure defined in Annex K. The measurement grid used for TRP measurement defined in Annex M. TRP is calculated considering both polarizations, theta and phi.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in Table 6.5.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE 2:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.5.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.5.2.1.5	Test requirement

The measured TRP of any UE emission derived in step 5, shall fulfil requirements in Table.6.5.2.1.5-1.

Table 6.5.2.1.5-1: General NR spectrum emission mask for Range 2

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 + TT | -5 + TT | -5 + TT | -5 + TT | 1 MHz |
| 5-10 | -13 + TT | -5 + TT | -5 + TT | -5 + TT | 1 MHz |
| 10-20 | -13 + TT | -13 + TT | -5 + TT | -5 + TT | 1 MHz |
| 20-40 | -13 + TT | -13 + TT | -13 + TT | -5 + TT | 1 MHz |
| 40-100 | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 100-200 |  | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 200-400 |  |  | -13 + TT | -13 +TT | 1 MHz |
| 400-800 |  |  |  | -13 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.1.5-1a NOTE 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. NOTE 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. |  |  |  |  |  |

Table 6.5.2.1.5-1a: Test Tolerance (Spectrum emission mask)

| Test Metric | 23.45GHz ≤ f ≤ 32.125GHz | 32.125GHz < f ≤ 40.8GHz |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | 3.21 dB | 3.46 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.1_1	Spectrum Emission Mask with Power Boost

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

6.5.2.1_1.1	Test purpose

Same as clause 6.5.2.1.1.

6.5.2.1_1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability.

6.5.2.1_1.3	Minimum conformance requirements

Same as clause 6.5.2.1.3.

6.5.2.1_1.4	Test description

6.5.2.1_1.4.1	Initial conditions

Same as clause 6.2.4_1.4.

6.5.2.1_1.4.2	Test procedure

Same as clause 6.5.2.1.4.2 with following exceptions:

-	Instead of Table 6.5.2.1.4.1-1 use Table 6.2.1.1.4.1-1 in normal environmental conditions only.

6.5.2.1_1.4.3	Message contents

Same as clause 6.2.4_1.4.3.

6.5.2.1_1.5	Test requirement

Same as clause 6.5.2.1.5.

## <<< END OF CHANGES >>>