3GPP TSG RAN WG5 Meeting #97 	R5-227881

Toulouse, France, 14th – 18th November 2022

Source:		Rohde & Schwarz

Title:	MU contributors and workplan for NR FR1 TRP-TRS (Rel.17)

Agenda Item:		5.3.41.2

Document for:	Discussion and Endorsement

# Introduction

As informed in the LS received this meeting [1], the work item led by RAN4 for NR FR1 TRP-TRS [2] was officially closed during September 2022 plenary meeting, which included a secondary responsibility for RAN5 to develop the Measurement Uncertainty (MU) assessment. This RAN5 task was fulfilled with the endorsed Text Proposals in [3][4].

Following closure of the RAN4 work item, a new RAN5 work item [5] was approved dealing with the NR FR1 TRP-TRS test specification TS 38.561. The aim of this contribution is to trigger the MU assessment work for TS 38.561 based on the preliminary assessment included in TS 38.834 [6].

# MU contributors in TR 38.834

Annex B of TR 38.884 [6] contains the MU calculation principles, the list of MU contributors, the description for each MU contribution and preliminary uncertainty budgets for hand only (browsing mode), updated from TS 37.544 [7] based on RAN5 work done during past year [3][4].

Given the limited timeline to finalize the MU work for TR 38.884, the effort was focused on most critical updates to MU descriptions and overall clean-up, but there was not a detailed analysis of each of the values used to derive the example expanded uncertainty.

In order to set the MU assessment framework for TS 38.561, Table 2-1 and Table 2-2 present the list of uncertainty contributors, for TRP and TRS respectively, extracted from TR 38.884 [6] with an additional status column to reflect the items that:

- The description was updated from TS 37.544.

- The example uncertainty value was updated from TS 37.544.

- The uncertainty value need to or could be revised.

Table 2-1: Uncertainty contributions in TRP measurement for anechoic chamber method (Table B.3-1 of TR 38.884 [6])

| UID | Description of uncertainty contribution | Details in clause | Status |
| --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |
| 1 | Mismatch of receiver chain | B.2.1 | b). The value was updated to show the right probability distribution. c). The assumptions to derive the value (i.e. 0dB cable attenuation) does not seem realistic. |
| 2 | Insertion loss of receiver chain | B.2.2 | - |
| 3 | Influence of the measurement antenna cable | B.2.3 | - |
| 4 | Measurement Receiver: uncertainty of the absolute level | B.2.4 | c). The value could be updated to reflect the performance of state-of-the-art equipment |
| 5 | Measurement distance | B.2.7 | a) and b) to reflect the impact of the change in minimum range length definition |
| 6 | Quality of quiet zone | B.2.8 | a). c). The value could be updated to reflect realistic performance from labs with the revised definition. |
| 7 | DUT Tx-power drift | B.2.9 | c). The definition and value were not updated from TS 37.544, which refer to studies performed for 3G UEs. |
| 8 | Uncertainty related to the use of phantoms | B.2.11 | c). The definition and value could be updated to cover new hand phantoms (i.e. wide grip hand) |
| 9 | Coarse sampling grid | B.2.12 | c). The definition and value were not updated from TS 37.544, which refer to studies performed for 3G UEs. |
| 10 | Random uncertainty | B.2.13 | c). The definition and value were not updated from TS 37.544, which refer to old studies. |
| 11 | Frequency Response | B.2.14 | a). This term was new compared to TS 37.544 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |
| 12 | Uncertainty of network analyzer | B.2.15 | c). The value could be updated to reflect the performance of state-of-the-art equipment |
| 13 | Mismatch of receiver chain | B.2.1 | - |
| 14 | Insertion loss of receiver chain | B.2.2 | - |
| 15 | Mismatch in the connection of calibration antenna | B.2.1 | - |
| 16 | Influence of the calibration antenna feed cable | B.2.3 | c). The value could be updated to reflect realistic performance from labs. |
| 17 | Influence of the measurement antenna cable | B.2.3 | - |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | B.2.16 | c). The value could be updated to reflect the performance of state-of-the-art equipment |
| 19 | Measurement distance | B.2.7 | a) and b) to reflect the impact of the change in minimum range length definition |
| 20 | Quality of the Quiet Zone | B.2.8 | a). c). The value could be updated to reflect realistic performance from labs with the revised definition. |

Table 2-2: Uncertainty contributions in TRS measurement for anechoic chamber method (Table B.4-1 of TR 38.884 [6])

| UID | Description of uncertainty contribution | Details in clause | Status |
| --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |
| 1 | Mismatch of transmitter chain | B.2.1 | b). The value was updated to show the right probability distribution. c). The assumptions to derive the value (i.e. 0dB cable attenuation) does not seem realistic. |
| 2 | Insertion loss of transmitter chain | B.2.2 | - |
| 3 | Influence of the measurement antenna cable | B.2.3 | - |
| 4 | Communication Tester: uncertainty of the absolute output level | B.2.5 | c). The value could be updated to reflect the performance of state-of-the-art equipment |
| 5 | Sensitivity measurement: output level step resolution | B.2.6 | b) to reflect the minimum step used for sensitivity search. |
| 6 | Measurement distance | B.2.7 | a) and b) to reflect the impact of the change in minimum range length definition |
| 7 | Quality of quiet zone | B.2.8 | a). c). The value could be updated to reflect realistic performance from labs with the revised definition. |
| 8 | DUT sensitivity drift | B.2.10 | c). The definition and value were not updated from TS 37.544, which refer to studies performed for 3G UEs. |
| 9 | Uncertainty related to the use of phantoms | B.2.11 | c). The definition and value could be updated to cover new hand phantoms (i.e. wide grip hand) |
| 10 | Coarse sampling grid | B.2.12 | c). The definition and value were not updated from TS 37.544, which refer to studies performed for 3G UEs. |
| 11 | Random uncertainty | B.2.13 | c). The definition and value were not updated from TS 37.544, which refer to old studies. |
| 12 | Frequency Response | B.2.14 | a). This term was new compared to TS 37.544 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |
| 13 | Uncertainty of network analyzer | B.2.15 | c). The value could be updated to reflect the performance of state-of-the-art equipment |
| 14 | Mismatch of transmitter chain | B.2.1 | - |
| 15 | Insertion loss of transmitter chain | B.2.2 | - |
| 16 | Mismatch in the connection of calibration antenna | B.2.1 | - |
| 17 | Influence of the calibration antenna feed cable | B.2.3 | c). The value could be updated to reflect realistic performance from labs. |
| 18 | Influence of the measurement antenna cable | B.2.3 | - |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | B.2.16 | c). The value could be updated to reflect the performance of state-of-the-art equipment |
| 20 | Measurement distance | B.2.7 | a) and b) to reflect the impact of the change in minimum range length definition |
| 21 | Quality of quiet zone | B.2.8 | a). c). The value could be updated to reflect realistic performance from labs with the revised definition. |

For those marked with c), further review and justification of the MU value used in the tables is recommended in order to have an accurate and fair derivation of the MTSU for TS 38.561.

Proposal 1: MU values shall be reviewed for contributors marked as c), and updated if needed. Analysis and revision of other MU values is not precluded.

# MU work plan

Looking at the work plan information presented [8], the target to finalize the clauses related to MU in TS 38.561 is RAN5#100 (August 2023). That means that there are 4 meetings (including RAN5#97) to complete the MU assessment. In order to achieve that, the following split is proposed:

- RAN5#97 (November 2022)

  - Agree on the MU values that require further review and justification.

  - Approve this workplan.

- RAN5#98 (February 2023)

  - Initial analysis of MU values.

  - Initial draft for TR 38.561 Annex A.

- RAN5#99 (May 2023)

  - Further progress of MU values.

- RAN5#100 (August 2023)

  - Conclude MU values and MTSU.

  - Finalize any pending text proposal for TR 38.870 Annex B.

Proposal 2: Approve the proposed workplan to develop the MU assessment for TS 38.561.

# Conclusion

In this contribution, the following observations and proposals were made:

Proposal 1: MU values shall be reviewed for contributors marked as c), and updated if needed. Analysis and revision of other MU values is not precluded.

Proposal 2: Approve the proposed workplan to develop the MU assessment for TS 38.561.

# References

- R5-225924, “LS on 3GPP NR TRP TRS OTA requirements”, TSG RAN WG4, 3GPP TSG RAN5 Meeting #97, November 2022

- RP-222269, “Revised WID: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)”, vivo, 3GPP TSG RAN Meeting #97-e, September 2022

- R5-221623, “TP to TR 38.834 for Annex B MU tables”, MVG Industries, Rohde & Schwarz, 3GPP TSG RAN5 Meeting #94-e, February 2022

- R5-221624, “TP to TR 38.834 on MU General section and descriptions”, Rohde & Schwarz, MVG Industries, 3GPP TSG RAN5 Meeting #94-e, February 2022

- RP-221690, “New WID on UE Conformance - Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)”, Apple, Rohde & Schwarz, vivo, 3GPP TSG RAN Meeting #96, June 2022

- 3GPP TR 38.834 v17.2.0 (September 2022)

- 3GPP TS 37.544 v16.2.0 (September 2021)

- R5-225652, “Discussion on initial work plan for NR FR1 TRP TRS”, Apple Portugal, 3GPP TSG RAN5 Meeting #96-e, August 2022