3GPP TSG-RAN5 Meeting #99	R5-232109

# Incheon, South Korea, 22 – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2190 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Output power requirements for CA_n28A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing Output power requirements for CA_n28A-n78A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Output power requirements for CA_n28A-n78A in Table 6.2A.1.0.3-1 and Table 6.2A.1.1.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Output power requirements for CA_n28A-n78A will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

##### 6.2A.1.0.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1.3-1 apply for power class 3 and other power classes if indicated in clause 5.5A.3.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in clause 6.2A.1.0.4 apply.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmitter power requirements specified in clause 6.2A.1.0.5 apply.

For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.0.3-1.

Table 6.2A.1.0.3-1: UE Power Class for uplink inter-band CA (two bands)

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n71A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: Void. NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2A.1.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.1.5-1 for Inter-band 2 UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.1.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n71A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: Void. NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table 6.2A.1.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-1a: Maximum Output Power test requirement for Power Class except class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n3A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n3A-n78A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n79A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n41A-n79A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| NOTE 1: Void. NOTE 2: It is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refer to Table6.2A.1.1.5-2. NOTE 7: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-2: Test Tolerance (UE maximum output power for CA)

| TT for overall output power (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |
|  |  |  | BW ≤ 40MHz |  | 40MHz < BW ≤ 100MHz |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
|  | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 |
| SCell |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |

-------- End of changes --------